3GPP TSG-SA WG1 Meeting #75	S1-162541

San Francisco, US, 22-26 August 2016

(revision of S1-162077, S1-162363, S1-162391, S1-162455, S1-162510)

Title: 	Use case to support Emergency Trajectory Alignment

Agenda Item:	9.2

Source:	Huawei

Contact:	Holger Rosier, holger.rosier@huawei.com

Laurence Meriau, laurence.meriau@huawei.com

Abstract: The use case introduces new KPIs for low latency but high reliable message exchange for safety Emergency Trajectory Alignment (EtrA) perfomed in case of abnormal condition of cooperated automated driving.

# References

[1] NGMN Alliance White Paper, “Perspectives on Vertical Industries and Implications for 5G”, [online], Available: https://www.ngmn.org/uploads/media/160610_NGMN_Perspectives_on_Vertical_Industries_and_Implications_for_5G_v1_0.pdf , [Accessed July, 2016]

[2] R. Alieiev, A. Kwoczek and T. Hehn, "Automotive requirements for future mobile networks," Microwaves for Intelligent Mobility (ICMIM), 2015 IEEE MTT-S International Conference on, Heidelberg, 2015, pp. 1-4.

[3] M. Düring, K. Franke, et al., "Adaptive cooperative maneuver planning algorithm for conflict resolution in diverse traffic situations," 2014 International Conference on Connected Vehicles and Expo (ICCVE), Vienna, 2014, pp. 242-249.

# Proposed additions to the eV2X TR

The following changes are proposed for eV2X TR.

Note: this is a fully new use case but no revision marks have been used, with the goal to get a better visibility of changes done during SA1 week.

************************************* Start of change *****************************

## 5.x  Emergency Trajectory Alignment

### 5.x.1 Description

Emergency Trajectory Alignment (EtrA) messages complement cooperative automated driving [3]. Maneuver cooperation through EtrA has been invented to assist the driver in hazardous and challenging driving situations to further increase traffic safety [3].

EtrA messages cover sensor data and status information with specific information for cooperative evasive maneuver coordination to bring more security in case of unexpected road conditions:

- When a vehicle obtains from on-board sensors the information about obstacles on the road (e.g. pedestrians on the road, loss of goods, deer crossing), it calculates the maneuver to avoid an accident.

- This vehicle then informs other vehicles (via 3GPP V2X communication service) about the safety-critical situation immediately. An important reliability of message transfer is expected to support the safety aspect of this information.

- Vehicles in proximity start to align trajectories to perform the emergency reaction cooperatively.

The following Key Performance Indicators (KPIs) are expected:

- Less than 3ms guaranteed radio latency for cooperative maneuver planning within the application time limit [2] [1].

- Throughput of 30Mbps to exchange messages with 90 kb between vehicles with sensor and trajectory data (0.3m resolution [2], 100 way-points per trajectory, 50 trajectories per message plus sensor data).

- 99.999% reliability [3] to avoid trajectory miscalculation on the application layer in safety-critical situation within 500m communication range [1].

### 5.x.1.1 Pre-conditions

- Vehicle A, B, C and D supports 3GPP V2X communication.

- Vehicle A, B, C and D supports EtrA on the application.

- Vehicle A detects an obstacle on the road with on-board sensors.

- Vehicle A calculates the driving maneuver to avoid crashing without posing a risk to other road users.

### 5.x.1.2 Service Flows

- Vehicle A informs B, C and D about the need of immediate trajectory re-calculation via 3GPP V2X communication service.

- After detecting and informing involved vehicles about trajectory re-calculation, vehicles A, B, C, D calculate possible trajectories and transmit them to all involved vehicles via 3GPP V2X communication service.

- Vehicle A, B, C, D decodes, verifies and evaluates trajectories via known algorithm on application layer and sends message with selected trajectories via 3GPP V2X communication service.

- Vehicle A, B, C and D sends messages via 3GPP V2X communication service to confirm on commonly selected trajectories for each vehicle.

- Vehicle A, B, C, D sends messages with re-calculated and selected trajectories regularly via 3GPP V2X communication service.

### 5.x.1.3	Post-conditions

- Temporary evasive maneuver group performs driving maneuver.

- Temporary evasive maneuver group finishes sending EtrA messages via 3GPP V2X communication service after the maneuver is performed completely and terminated.

### 5.x.2 Potential Requirements

### [PR-5.x.2-001] The 3GPP network shall enable communication between UEs with data rate [30Mbps], less than [3ms] radio latency and [99.999%] reliability within communication range of [500m].

### NOTE: The purpose of the requirement is to be able to provide very fast feedback.

************************************* End of change *****************************