3GPP TSG-SA WG1 Meeting #76	S1-163083

Tenerife Spain, 7-11 November 2016	(revision of S1-16xxxx)

Title:	FS_PLeaSe – access to local service for roaming

Agenda Item:	5

Source:				Nokia, Alcatel-Lucent Shanghai Bell, Verizon, Qualcomm, Sprint

Contact:			Betsy Covell betsy.covell@nokia.com

Abstract: This document proposes a use case for access to local services.  In this use case, the calling party roams into a region where her home network operator does not have a roaming agreement with the serving operator. She finds the contact information for a service that allows her to contact the serving operator in order to establish roaming. She is able to establish a temporary service with the serving operator. The call to the serving operator for roaming must be disconnected before any other service attempt can be made.

First Change

# 5	Use Cases

## 5.x	Access to operator local service

### 5.x.1	Description

An operator provides a service to establish temporary roaming service. The information on how to access the services (e.g., phone number, secure portal) is posted on the operator website.  A user whose home network operator does not have a roaming agreement with this operator roams into the area.  After a failed call attempt, she finds the contact information for the roaming service and uses her UE to connect to the service. Once the roaming capability is activated, the call must be fully disconnected, ensuring any other service access attempt follows normal access procedures.

### 5.1.2	Pre-conditions

The operator provides information on their website on how to establish roaming service, including a local phone number or URL for a secure portal.

A user who has roamed into the serving system looks up the information on the website.

### 5.1.3	Service Flows

The serving network broadcasts an indication that local services are supported.

The user uses her UE to contact the roaming service center. Even though the UE has failed on prior service attempts due to the lack of a roaming agreement, the access attempt is allowed to complete.

The user communicates with the roaming service center staff, and establishes a temporary service for the duration of her stay in the area.

### 5.1.4	Post-conditions

The UE is disconnected from the network.

The next origination attempt from the customer's UE goes through the normal session set up process.

### 5.1.5	Potential Requirements

A 3GPP system that supports local services will broadcast a notification that such services are supported.

Based on operator policy and regional regulations, the 3GPP system shall support a mechanism to identify a set of addresses (e.g., phone number, secure portal) for access to local services.

Based on operator policy and regional regulations, the 3GPP system shall support mechanisms to recognize a session origination to a local service and to allow all such originations to succeed.