3GPP TSG-SA WG1 Meeting #76	S1-163086

Tenerife Spain, 7-11 November 2016	(revision of S1-16xxxx)

Title:	FS_PLeaSe – access to local service for service establishment

Agenda Item:	5

Source:				Nokia, Alcatel-Lucent Shanghai Bell, Verizon, Qualcomm, Sprint

Contact:			Betsy Covell betsy.covell@nokia.com

Abstract: This document proposes a use case for access to local services.  In this use case, the calling party purchases an unlocked phone and is able to contact the customer subscription center of the operator they want to establish service with. Once the subscription is established, they are able to use their UE for other services.

First Change

# 5	Use Cases

## 5.x	Access to operator local service for service establishment

### 5.x.1	Description

An operator provides a local service to allow end users to establish a subscription. The information on how to access the services (e.g., phone number, secure portal) is available from the operator.  A user purchases an unlocked UE and determines the customer subscription center access information for his operator of choice.  He contacts the customer subscription center, establishes a subscription and activates his UE for use with that subscription. The call must be fully disconnected once the subscription is established, ensuring any other service access attempt follows normal access procedures.

### 5.1.2	Pre-conditions

The operator provides information on how to contact the customer subscription center, including a local phone number or URL for a secure portal.

A user with an unlocked UE finds the customer subscription center access information.

### 5.1.3	Service Flows

The serving network broadcasts an indication that local services are supported.

The user uses his UE to contact the customer subscription center.

The customer subscription center is able to securely establish a subscription for the user and securely activate the UE for service under that subscription.

### 5.1.4	Post-conditions

The UE is disconnected from the network.

The next origination attempt from the customer's UE goes through the normal session set up process, including authentication.

### 5.1.5	Potential Requirements

A 3GPP system that supports local services will broadcast a notification that such services are supported.

Based on operator policy and regional regulations, the 3GPP system shall support a mechanism to identify a set of addresses (e.g., phone number, secure portal) for access to local services.

Based on operator policy and regional regulations, the 3GPP system shall support mechanisms to recognize a session origination to a local service and to allow all such originations to succeed.