3GPP TSG-SA WG1 Meeting #76	S1-163090

Tenerife Spain, 7-11 November 2016	(revision of S1-16xxxx)

Title:	FS_PLeaSe – Potential requirements

Agenda Item:	5

Source:				Nokia, Alcatel-Lucent Shanghai Bell, Verizon, Qualcomm, Sprint

Contact:			Betsy Covell betsy.covell@nokia.com

Abstract: This document proposes potential requirements for provision of local operator services.

First Change

# 7	Potential Requirements

A 3GPP system that supports local services shall broadcast a notification that such services are supported.

Based on operator policy and regional regulations, the 3GPP system shall support a mechanism to identify a set of addresses (e.g., phone number, secure portal) for access to local services

Based on operator policy and regional regulations, the 3GPP system shall support a mechanism to identify a call attempt to be redirected to a local service.

Based on operator policy and regional regulations, the 3GPP system shall support mechanisms to recognize a session origination to a local service and to allow all such originations to succeed.

Based on operator policy and regional regulations, the 3GPP system shall support a mechanism to indicate a local service call attempt in a call origination.