3GPP TSG-SA WG1 Meeting #76	S1-163247

Tenerife Spain, 7-11 November 2016	(revision of S1-163083)

Title:	FS_PLeaSe – access to local service for roaming

Agenda Item:	5

Source:				Nokia, Alcatel-Lucent Shanghai Bell, Verizon, Qualcomm, Sprint

Contact:			Betsy Covell betsy.covell@nokia.com

Abstract: This document proposes a use case for access to local services.  In this use case, the calling party roams into a region where her home network operator does not have a roaming agreement with the serving operator. She finds the contact information for a service that allows her to contact the serving operator in order to establish roaming. She is able to establish a temporary service with the serving operator. The call to the serving operator for roaming must be disconnected before any other service attempt can be made.

First Change

# 5	Use Cases

## 5.x	Access to restricted local operator service for manual roaming

### 5.x.1	Description

An operator provides a service to establish temporary manual roaming service. The information on how to access the services (e.g., phone number) is posted on the operator website.  A user whose home network operator does not have a roaming agreement with this operator roams into the area.  After a failed call attempt, she finds the contact information for the temporary manual roaming service and uses her UE to place a VoLTE call to the temporary manual roaming service. Once the temporary manual roaming capability is activated, the call must be fully disconnected, ensuring any other service access attempt follows normal access procedures.

### 5.1.2	Pre-conditions

The operator provides information on their website on how to establish temporary manual roaming service, including a local phone number or URL for a secure portal.

A user who has roamed into the serving system looks up the information on the website.

### 5.1.3	Service Flows

The serving network provides an indication that restricted local operator services are supported.

The user uses her UE to place a VoLTE call to the temporary manual roaming service center. Even though the UE has failed on prior service attempts due to the lack of a roaming agreement, the access attempt is allowed to complete.

The user communicates with the temporary manual roaming service center staff, and establishes a temporary service for the duration of her stay in the area.

### 5.1.4	Post-conditions

The UE is disconnected from the network.

The next origination attempt from the customer's UE goes through the normal session set up process.

### 5.1.5	Potential Requirements

A 3GPP system shall support a mechanism to indicate to UEs that restricted local operator services are available.

Editor’s note: definition of restricted local operator service is needed.

Based on operator policy and regional regulations, the 3GPP system shall support a mechanism to configure a set of addresses (e.g., phone number, secure portal) for access to restricted local operator services.

Based on operator policy and regional regulations, the 3GPP system shall support mechanisms to allow access to restricted local operator services by unauthenticated UEs.