3GPP TSG-SA WG1 Meeting #76	S1-163386

Tenerife Spain, 7-11 November 2016	(revision of S1-163088)

Title:	FS_PLeaSe – failed access to local service

Agenda Item:	5

Source:				Nokia, Alcatel-Lucent Shanghai Bell, Verizon, Qualcomm, Sprint

Contact:			Betsy Covell betsy.covell@nokia.com

Abstract: This document proposes a use case for failed access to local services.  In this use case, a roaming user attempts to reach a local service that is not supported by the serving system.  The call attempt fails if the UE cannot be authenticated.

First Change

# 5	Use Cases

## 5.x	Failed access to restricted local operator service

### 5.x.1	Description

An operator has chosen not to provide restricted local operator services to their customers.  When a user who is accustomed to restricted local operator services in their home network roams into a network where restricted local operator services are not available, they will not have access to those services.

### 5.x.2	Pre-conditions

A user with a UE containing a list of pre-provisioned restricted local operator services roams into a network where restricted local operator services are not supported.

The user selects a restricted local operator service from the list provided on the UE.

### 5.x.3	Service Flows

The service network is not providing an indication of support for restricted local operator service.

The UE does not initiate the call attempt to that network.

### 5.x.4	Post-conditions

The UE remains disconnected from the network.

### 5.x.5	Potential Requirements

No additional requirements are needed in systems that do not support local services.