3GPP TSG-SA WG1 Meeting #76	S1-163388

Tenerife Spain, 7-11 November 2016	(revision of S1-163090)

Title:	FS_PLeaSe – Potential requirements

Agenda Item:	5

Source:				Nokia, Alcatel-Lucent Shanghai Bell, Verizon, Qualcomm, Sprint

Contact:			Betsy Covell betsy.covell@nokia.com

Abstract: This document proposes potential requirements for provision of local operator services.

First Change

# 7	Potential Requirements

A 3GPP system shall support a mechanism to indicate to UEs that restricted local operator services are available.

Based on operator policy and regional regulations, the 3GPP system shall support a mechanism to configure a set of addresses (e.g., phone number, secure portal) for access to restricted local operator services.

Based on operator policy and regional regulations, the 3GPP system shall support mechanisms to allow access to restricted local operator services by unauthenticated UEs.

Based on operator policy and regional regulations, the 3GPP system shall support a mechanism to indicate a restricted local operator service origination attempt.