3GPP TSG-SA WG1 Meeting #76	S1-163416

Tenerife Spain, 7-11 November 2016	(revision of S1-163295)

(revision of S1-163261)

(revision of S1-163072)

Title:	SMARTER – mobility management

Agenda Item:	7.1

Source:				Nokia, Alcatel-Lucent Shanghai Bell

Contact:			Betsy Covell betsy.covell@nokia.com

Abstract: This document proposes to clarify the requirements related to diverse mobility to avoid potential confusion in downstream groups.

First Change

### 6.2.2	General requirements

The 5G network shall allow operators to optimize network behaviour (e.g., mobility management support) based on the mobility patterns (e.g., stationary, nomadic, spatially restricted mobility, full mobility) of a device or group of devices.

The 3GPP system shall enable operators to specify and modify the types of mobility support provided for a device.

The 3GPP system shall optimize mobility management support for devices that use only mobile originated communications.

### 6.2.3	Service continuity requirements

The 3GPP system shall enable packet loss to be minimized during inter- and/or intra-RAT cell changes for some or all connections associated with a device.

For applications that require a constant IP address, the 3GPP system shall enable maintaining the IP address assigned to a device when moving across different cells and RATs for connections associated with a device.

The 3GPP system shall enable minimizing impact to the user experience (e.g., minimization of interruption time) when changing the IP address and IP anchoring point for some or all connections associated with a device.

The 3GPP system shall support service continuity for a remote UE, when the remote UE changes from one relay UE to another and both relay UEs use 3GPP access to the 3GPP core network.