3GPP TSG-SA WG1 Meeting #76	S1-163441

Tenerife, Spain, 7-11 Nov 2016	Revision of 317 Merger of S1-163316/040/041/042/043/044

Title:	FS_eV2X Consolidation of Requirements

Agenda Item:	8.1

Source:	SA1 eV2X Drafting Session

Contact:	Rapporteur (sungduck.chun@lge.com)

Laurence Meriau (Laurence.meriau@huawei.com)

- Abstract

This document proposes consolidated potential requirements for eV2X TR.

- Proposal

Below is the result of the consolidation, proposed to be added in Section 7 of FS_eV2X TR.

--------------------------------------------------------------------------------------------------

------------------------------- START of text proposal ------------------------------------

--------------------------------------------------------------------------------------------------

7	Potential Requirements

7.1	General

Different V2X scenarios require the transport of V2X messages with different performance requirements for the 3GPP system.

Vehicles Platoonning enables the vehicles to dynamically form a platoon travelling together. All the UEs in the platoon obtain information from the leading vehicle to manage this platoon. These information allow the vehicles to drive closer (short time or distance inter-vehicle gap) than normal in a coordinated manner, going to the same direction and travelling together. These are expected to be a set of sophisticated application.

Extended Sensors enables the exchange of raw or processed data gathered through local sensors or live video images among  vehicles, road site units, devices’ of pedestrian and V2X application servers. The vehicles can increase the perception of their environemnt beyond of what their own sensors can detect and have have a more broad and holistic view of the local situation. High data rate is one of the key characteristics.

Advanced driving enables semi-automated or full-automated driving. Longer inter-vehicle distance is assumed. Each vehicle and/or RSU shares its own perception data obtained from its local sensors with vehicles in proximity and that allows vehicles to synchronize and coordinate their trajectories or manoeuvres. Each vehicle shares its driving intention with vehicles in proximity, too. The benefits of this use case group are safer traveling, collision avoidance, and improved traffic efficiency.

Remote Driving enables a remote driver or a V2X application to operate a remote vehicle for those passengers who cannot drive by themselves or remote vehicles located in dangerous environments. For a case where variation is limited and routes are predictable, such as public transportation, driving based on cloud computing can be used. Also, access to cloud-based back-end service platform can be considered for this use case group. High reliability and short low latency are the main requirements.

7.2	Consolidated Requirements

Editor's Note: Potential requirements out of new use cases agreed in SA1#76 will be consolidated at SA1#76bis.

7.2.1	General Requirements

Some requirements in this section are applicable to all V2X scenarios.

[CPR.G-001]	The 3GPP system shall be able to control the communication range for a message based on the characteristic of the messages transmitted by a UE supporting V2X application.

[CPR.G-002]	The 3GPP system shall be able to optimize the communication between UEs supporting V2X application belonging to the same group and in proximity.

[CPR.G-003]	The 3GPP system shall be able to support the message transfer for group management operations as requested by the application layer.

[CPR.G-004]	The 3GPP system shall be able to support message transfer among a group of UEs supporting V2X application.

[CPR.G-005]	The 3GPP system shall be able to support message transfer between two UEs belonging to the same group of UEs supporting V2X application.

[CPR.G-006]	The 3GPP system shall be able to support confidentiality and integrity of message transfer among a group of UEs supporting V2X application.

[CPR.G-007]	The 3GPP system shall support relative lateral position accuracy of 0.1 m between UEs supporting V2X application.

[CPR.G-008]	The 3GPP system shall support mechanisms to ensure sufficient reliability metrics are reached.

[CPR.G-009]	The 3GPP system shall support high connection density for congested traffic.

NOTE:	Example estimate is for worst case US Freeway scenario that does not include arterial roads (i.e. onramps): 5 lanes in each direction or 10 lanes total per highway, for up to 3 highways intersecting = 15,840 cars per mile.

[CPR.G-010]	The 3GPP system shall support efficient coordination of radio resources used for transport of V2X communications to maximize the utilisation of the available spectrum and to ensure the required reliability.

[CPR.G-011]	The 3GPP system shall be able to control the UL and DL reliability of transport of V2X communications, depending on the requirement of V2X application

[CPR.G-012]	Impact to E-UTRA(N) by UE supporting only 5G RAT(s) based V2X communication shall be minimized.

[CPR.G-013]	Impact to 5G RA(N) by UE supporting only E-UTRA based V2X communication shall be minimized.

[CPR.G-014]	The 3GPP system shall be able to support message transfer between UEs or between a UE and a UE-type RSU, regardless of whether or not they are subscribers of the same PLMN supporting V2X communications. In case they are subscribers to different PLMNs, there shall be no service degradation of the message transfer.

[CPR.G-015]	The 3GPP system shall be capable of supporting high reliability without requiring application-layer message retransmissions.

[CPR.G-016]	The 3GPP system shall enable discovery and communication between UEs supporting the same V2X application.

[CPR.G-018]	The 3GPP system shall be able to support for the operator to select which 3GPP RAT to use for a V2X application.

[CPR.G-019]	The 3GPP system shall enable a UE supporting a V2X application to obtain network access via another UE supporting V2X application.

[CPR.G-020]	The 3GPP system shall enable a UE supporting a V2X application to discover another UE supporting V2X application that can offer access to the network.

[CPR.G-021]	The 3GPP system shall enable a UE supporting a V2X application to switch the network access from direct 3GPP connection to an indirect 3GPP connection via another UE supporting V2X application that is connected via 3GPP access to the 3GPP core network, and vice versa.

[CPR.G-022]	The 3GPP system should provide integrity and confidentiality protection (end to end) for the network access traffic of a V2X UE via another such UE.

[CPR.G-023]	The 3GPP system shall allow UEs supporting V2X application to use 5G RAT for direct communication when the UEs are not being serviced by a 5G cell.

[CPR.G-024]	An RSU shall be able to communicate with up to 200 UEs supporting a V2X application.

[CPR.G-025]	The 3GPP system shall support less than 5 ms communication latency for transport of V2V messages between two UEs supporting V2V applications, that are part of a group of UEs supporting V2Vapplications.

NOTE:	The determination of group membership may be done at the upper layers and/or lower layers (application and/or Layer 2).

7.2.2	Requirements for Platooning

[CPR.P-001]	The 3GPP system shall be able to support up to 5 UEs for a group of UEs supporting V2X application..

[CPR.P-002]	For Vehicle Platooning, the 3GPP system shall be able to support reliable V2V communications between a specific UE supporting V2X applications and up to 19 other UEs supporting V2X applications.

[CPR.P-003]	The 3GPP system shall support relative longitudinal position accuracy of less than 0.5 m for UEs supporting V2X application for platooning in proximity.

Table 7.2.2-1 Performance requirements for Platooning

| Communication Scenario (Related use case section Number) |  | Payload (Bytes) | Tx Rate (Message/ Sec) | Max Latency (ms) | Reliabi-lity (%) | Data Rate (Mbps) | Commu-nication Range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Among a group of UEs (or two UEs) supporting V2X application (Section 5.1) | [CPR.7.2.2-004] | 50-1200 (NOTE 1) | 30 | 10 (NOTE 2) |  |  |  |
|  | [CPR.7.2.2-005] | 300-400 | 30 | 25 | 90 |  |  |
| Between UE supporting V2X application and RSU via another UE supporting V2X application (Section 5.2) | [CPR.7.2.2-006] | [50-1200] | 2 | 500 |  |  |  |
| Between UEs supporting V2X application (Section 5.5) | [CPR.7.2.2-007] Driver control | 300-400 (NOTE 3) |  | 25 | 90 |  |  |
|  | [CPR.7.2.2-008] [CPR.7.2.2-009] Fully automated driving | 1200 |  | 10 | 99.99 |  | 80 |
| Between UEs supporting V2X application (Section 5.12 & 5.13) | [CPR.7.2.2-016] [CPR.7.2.2-019] [CPR.7.2.2-018] Driver control/ Limited automated driving | [6500] | 50 | [20ms] |  |  | [10] sec * (max. relative speed) [m/s] |
|  | [CPR.7.2.2-021] [CPR.7.2.2-023] [CPR.7.2.2-025] Fully automated driving |  |  | [20ms] |  | [65] | [5] sec * (max. relative speed) [m/s] |
| Between UE supporting V2X application and RSU (Section 5.12 & 5.13) | [CPR.7.2.2-017] [CPR.7.2.2-019] [CPR.7.2.2-018] Driver control/ Limited automated driving | [6000] | 50 | [20ms] |  |  | [10] sec * (max. relative speed) [m/s] |
|  | [CPR.7.2.2-022] [CPR.7.2.2-025] Fully automated driving |  |  | [20ms] |  | [50] | [5] sec * (max. relative speed) [m/s] |

NOTE 1:	This value does not including security related messages component.

NOTE 2:	This value is end-to-end latency.

NOTE 3:	This value is applicable for both triggered and periodic transmission of data packets 
OTE1: e use case.ts for platonning can be found below and specific  can be used to serve specific to each scenario;requirements

7.2.3	 Requirements for Advanced Driving

Table 7.2.3-1 Performance requirements for Advanced Driving

| Communication Scenario (Related use case section Number) |  | Payload (Bytes) | Tx Rate (Message/Sec) | Max Latency (ms) | Reliabi-lity (%) | Data Rate (Mbps) | Commu-nication Range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Between UEs supporting V2X applications (Section 5.9) | [CPR.7.2.3-001] Fully automated driving | [2000] |  | [10] | [99.99] | [10] |  |
| Between UEs supporting V2X applications (directly or via an RSU) (Section 5.10) | [CPR.7.2.5-001] [CPR.7.2.5-001a] Driver control/ Limited automated driving | [6500] | 10 | [100] |  |  | [10] sec * (maximum relative speed) [m/s] |
| Between UEs supporting V2X applications (Section 5.11) | [CPR.7.2.5-004] Fully automated driving |  |  | [100] |  | [53] | [5] sec * (max. relative speed) [m/s] |
| Between the UE supporting V2X application and the RSU (Section 5.10) | [CPR.7.2.5-002] [CPR.7.2.5-002a] Driver control/ Limited automated driving | [6000] | 10 | [100] |  |  | [10] sec * (max. relative speed) [m/s] |
| Between the UE supporting V2X application and the RSU (Section 5.11) | [CPR.7.2.5-005] Fully automated driving |  |  | [100] |  | [50] | [5] sec * (max. relative speed) [m/s] |
| Between UEs supporting V2X application (Section 5.20) | [CPR.7.2.4-001] Fully automated driving |  |  | [3] | [99.999] | [30] | [500m] |
| Between RSU and UE supporting V2X application (Section 5.22) | [CPR.7.2.9-001] [CPR.7.2.9-002] | 450 | 50 |  |  | DL: [0.5] UL: [50] |  |

7.2.4	 Requirements for Extended Sensors

Table 7.2.4-1 Performance Requirements for Extended Sensors

| Communication Scenario (Related use case section Number) |  | Payload (Bytes) | Max Latency (ms) | Reliabi-lity (%) | Data Rate (Mbps) | Communication Range (meters) |
| --- | --- | --- | --- | --- | --- | --- |
| Between UEs supporting V2X application (Section 5.3) | [CPR.7.2.6-001] Fully automated driving |  | 10 | 95 | Peak data rate [25] |  |
| Between UEs supporting V2X application (Section 5.6) | [CPR.7.2.7-001] Driver control | [1600] | 100 | 99 |  | 1000 |
|  | [CPR.7.2.7-002] [CPR.7.2.7-003] [CPR.7.2.7-004] [CPR.7.2.2-021] Fully automated driving |  | 3 | 99.999 |  | 200 |
|  |  |  | 10 | 99.99 |  | 500 |
|  |  |  | 50 | 99 |  | 1000 |
|  |  |  |  |  | 1000 | 50 |
| Between UEs supporting V2X application (Section 5.16) | [CPR.7.2.8-001] Driver control/ Automated driving |  | [50] | 90 | [10] | [100] |
|  | [CPR.7.2.8-002] Fully automated driving |  | [10] | 99.99 | [700] | [500] |

7.2.5 Requirements for Remote Driving

[CPR.R-001]	The 3GPP system shall support user experienced data rate up to 1 Mbps at DL and 20 Mbps at UL for UE supporting V2X application between V2X application server and UE for an absolute speed of up to 250 km/h.

[CPR.R-002]	The 3GPP system shall support ultra-high UL and DL reliability [99.999 % or higher] for UE supporting safety-related V2X application.

[CPR.R-003]	The 3GPP system shall support end-to-end latency 5 ms between V2X application server and UE supporting safety-related V2X application for an absolute speed of up to 250 km/h.

Table 7.2.5-1 Performance requirements for Remote Driving

| Communication Scenario (Related use case section Number) |  | Payload (Bytes) | Max Latency (ms) | Reliabi-lity (%) | Data Rate (Mbps) | Communication Range (meters) |
| --- | --- | --- | --- | --- | --- | --- |
| Between a UE supporting V2X application & V2X Application Server (Section 5.21) | [CPR.7.2.10-002] Driver control |  | [20] | [99.999] | UL: 25 DL: 1 |  |

--------------------------------------------------------------------------------------------------

------------------------------- END of text proposal ------------------------------------

--------------------------------------------------------------------------------------------------

- Annex Consolidation details

In this section, the information on how each the potential requirement of use cases resulted in the proposed consolidated requirements in the previous section is provided.

The mapping information between potential requirements and consolidated requirement will be further refined and will be put into Annex of TR at SA1#76bis.

The following table considers:

- Column 1 is the use case title

- Column 2 contains the Existing Potential Requirements (PR)

- Column 3 discusses reasons for text changes

- Column 4 proposes the text for the consolidated requirement (CPR)

| Use case Title | Current Potential Requirement | Discussion | Proposed new text and requirements grouping |
| --- | --- | --- | --- |
| 5.1 eV2X support for Vehicle Platooning | Set 1&2 [PR.5.1-001] The 3GPP system shall be able to control the communication range for a message based on the characteristic of the messages transmitted by a UE supporting V2X application. |  | Group: General [CPR.G-001] The 3GPP system shall be able to control the communication range for a message based on the characteristic of the messages transmitted by a UE supporting V2X application. |
|  | Set 1&2 [PR.5.1-002] The 3GPP system shall be able to support in a periodic manner at least 30 message per second broadcast by a UE supporting V2X application. | No change. | [CPR.7.2.2-001] Group: platooning Scenario: eV2X support for Vehicle Platooning The 3GPP system shall be able to support in a periodic manner at least 30 message per second broadcast by a UE supporting V2X application. |
|  | Set 1&2 [PR.5.1-003] The 3GPP system shall be able to create/destroy a group for UEs supporting V2X application. | The need is to be able to allow the 3GPP system to optimise the communication between UEs belonging to the same group, when they are close / in same area) (NOTE: The determination of group membership may be done at the upper layers and/or lower layers (application and/or Layer 2) | Group: General [CPR.G-002] The 3GPP system shall be able to optimize the communication between UEs belonging to the same group, when they are close / in same area |
|  | Set 1&2 [PR.5.1-004] The 3GPP system shall be able to support up to 5 UEs for a group of UEs supporting V2X application. | No Change | [CPR.P-001] [CPR.7.2.2-002] Group: platooning Scenario: eV2X support for Vehicle Platooning The 3GPP system shall be able to support up to 5 UEs for a group of UEs supporting V2X application. |
|  | Set 1&2 [PR.5.1-005] The 3GPP system shall be able to add/remove a UE supporting V2X application into a group of UEs supporting V2X application. | Wording change | Group: General [CPR.G-003] The 3GPP system shall be able to support the message transfer for group management operations as requested by the application layer. |
|  | Set 1&2 [PR.5.1-006] The 3GPP system shall be able to support at least in a periodic manner [28] message transfer per second among a group of UEs supporting V2X application. | No change | [CPR.7.2.2-003] Group: platooning Scenario: eV2X support for Vehicle Platooning The 3GPP system shall be able to support at least [28] message transfer per second among a group of UEs supporting V2X application. |
|  | Set 1&2 [PR.5.1-007] The 3GPP system shall be able to support message transfer among a group of UEs supporting V2X application. | This is a generic requirement valid for all V2X use cases that need communication between groups of UEs. No text change | Group: Generic [CPR.G-004] The 3GPP system shall be able to support message transfer among a group of UEs supporting V2X application. |
|  | Set 1&2 [PR.5.1-008] The 3GPP system shall be able to support message transfer between two UEs belonging to the same group of UEs supporting V2X application. | This is a generic requirement valid for all V2X use cases. No text change | Group: Generic [CPR.G-005] The 3GPP system shall be able to support message transfer between two UEs belonging to the same group of UEs supporting V2X application. |
|  | Set 1&2 [PR.5.1-009] The 3GPP system shall be able to support confidentiality and integrity of message transfer among a group of UEs supporting V2X application. | This is a generic requirement valid for all V2X use cases. No text change | Group: Generic [CPR.G-006] [CPR.7.2.1-003] The 3GPP system shall be able to support confidentiality and integrity of message transfer among a group of UEs supporting V2X application. |
|  | Set 2 [PR.5.1-010] The 3GPP system shall be able to support 10 ms end-to-end latency for message transfer among a group of UEs supporting V2X application. | Includes KPI from [PR.5.1-011] as for same use case. The use case describes a fully automated platooning allowing very short distance between vehicles allowing fully automatic level of platooning . | [CPR.7.2.2-004] Group: platooning Scenario: eV2X support for Vehicle Platooning, fully automated The 3GPP system shall be able to support 10 ms end-to-end latency and with variable message data packets of 50-1200 Bytes, not including security-related message component for message transfer among a group of UEs (or two UEs; MV) supporting V2X application (). |
|  | Set 2 [PR.5.1-011] The 3GPP system shall be capable of transferring messages among a group of UEs supporting V2X application with variable message payloads of 50-1200 bytes, not including security-related message component. | Merged with [PR-5.1-010] | Merged |
|  | Set 1 [PR-5.1-012] The 3GPP system shall support communication latency no larger than [25 ms]. | According to use case description, this is not short distance vehicle platooning. Included also KPIs from requirements [PR-5.1-013] and [PR-5.1-014] as in the same use case. | [CPR.7.2.2-005] Group: platooning Scenario: eV2X support for Vehicle Platooning The 3GPP system shall support periodic transmission of small data packets (e.g. 300-400 bytes) exchange between V2X applications with less than 25ms latency and reliability of 90%. |
|  | Set 1 [PR-5.1-013] The 3GPP system shall support [90%] reliability. | Merged with [PR-5.1-012] as complement for the same use case. | Merged |
|  | Set 1 [PR-5.1-014] The 3GPP system shall support triggered and periodic transmission of small data packets (e.g. 300-400 bytes). | Merged with [PR-5.1-012] as complement for the same use. | Merged |
| 5.2 Information exchange within platoon | [PR.5.2-001] The 3GPP system shall be able to support a maximum latency of [500] ms for the transport of V2X messages between a UE supporting V2X application and a RSU via another UE supporting V2X application. | Remove “via another UE” as the -----Requirement does not need to consider relay case. -Included also KPIs from requirements [PR-5.2-002] and [PR-5.2-003] as linked to the same use case. -Add a clarification of the use case served by this requirement: vehicle platooning. The use case does not indicate whether it is set 1 or 2. | [CPR.7.2.2-006] Group: platooning Scenario: eV2X support for Vehicle Platooning The 3GPP system shall be able to support a maximum latency of [500] ms and a maximum transport of [2] V2X messages per second for the transport of V2X messages with variable payload of [50-1200] bytes between a UE supporting V2X application and a RSU supporting V2X application for vehicles platooning via another UE supporting V2X application. |
|  | [PR.5.2-002] The 3GPP system shall be able to support the maximum transport of [2] V2X messages per second between a UE supporting V2X application and a RSU via another UE supporting V2X application. | Merged with [PR.5.2-001] as this complements the same use case. | Merged in [CPR.7.2.2-006] |
|  | [PR.5.2-003] The 3GPP system shall be able to support the transport of V2X messages with variable payload of [50-1200] bytes between a UE supporting V2X application and a RSU via another UE supporting V2X application. | Merged with [PR.5.2-001] as this complements the same use case No change. | Merged in [CPR.7.2.2-006] |
|  | [PR.5.2-004] The 3GPP system shall be able to support a maximum latency of [10] ms for V2V communications. | This KPI already considered in [PR.5.1-010] and this is the same use case. No change. | Already into [CPR.7.2.2-004] |
| 5.5 Automated Cooperative Driving for Short distance Grouping | Set 1 [PR-5.5-001] The 3GPP system shall support communication latency for data packets 300-400 bytes no larger than 25 ms. | No change | [CPR-7.2.2-007] Group: platooning Scenario Automated Cooperative Driving for Short distance Grouping, with driver control(LM w/t/keep) The 3GPP system shall support communication latency for data packets 300-400 bytes no larger than 25 ms and over 90% target packet delivery reliability rate. Note – 005 (w/ or w/o “periodic transmission”) |
|  | Set 1 [PR-5.5-002] The 3GPP system shall support over 90% target packet delivery reliability rate. | Merged with [PR-5.5-001] | Merged |
|  | Set 1 [PR-5.5-003] The 3GPP system shall support triggered and periodic transmission of data packets 300-400 bytes. | NOTE for [PR-5.5-001] | NOTE: requirement applicable to both event-triggered and periodic transmission of data packets |
|  | [PR-5.5-003a] The 3GPP system shall support less than 5 ms communication latency for transport of V2V messages between two UEs supporting V2V applications, that are part of a group of UEs supporting V2Vapplications. NOTE: The determination of group membership may be done at the upper layers and/or lower layers (application and/or Layer 2). | Keep | [CPR.G-025] The 3GPP system shall support less than 5 ms communication latency for transport of V2V messages between two UEs supporting V2V applications, that are part of a group of UEs supporting V2Vapplications. NOTE: The determination of group membership may be done at the upper layers and/or lower layers (application and/or Layer 2). |
|  | Set 2 [PR-5.5-004] The 3GPP system shall support less than 10 ms communication latency for transport of V2X messages between two UEs supporting V2X application in proximity. | No change. From the use case description, it is platooning with full automated driving. Replace “system” by “network” as the message exchange is between UE, so in the network. Requirement [PR-5.5-005] below from the same use case is merged. | [CPR.7.2.2-008] Group: platooning Scenario Automated Cooperative Driving for Short distance Grouping, for full automated driving. The 3GPP system (EH: b/w UE to UE) shall support message exchange between two UEs with less than 10 ms latency guaranteed in highly loaded network, with [99.99%] reliability and message sizes within 80 m range |
|  | Set 2 [PR-5.5-005] The 3GPP system shall support over [99.99%] target packet delivery reliability rate within 80 m range. | Merged with [PR-5.5-004] as complement in the same use. | Merged |
|  | Set 2 [PR-5.5-006] The 3GPP system shall support transmission of one data packet up to 1200 bytes every 25 ms within 80 m range. | No change, for fully automated driving | [CPR.7.2.2-009] Group: platooning Scenario Automated Cooperative Driving for Short distance Grouping, for full automated driving The 3GPP system shall support transmission of one data packet up to 1200 bytes every 25 ms within 80 m range between UEs. |
|  | Set 2 [PR-5.5-007] The 3GPP system shall support relative lateral position accuracy of 0.1 m. | Add a clarification that the use case is for platooning | [CPR.G-007] Group: general (outside Ptoon) [CPR.G-007] The 3GPP system shall support relative lateral position accuracy of 0.1 m between UEs supporting V2X application. |
|  | Set 2 [PR-5.5-008] The 3GPP system shall support relative longitudinal position accuracy of less than 0.5 m for UEs supporting V2X application in proximity. | Add a clarification that the use case is for platooning | [CPR.7.2.2-013] [CPR.P-003] Group: platooning Scenario Automated Cooperative Driving for Short distance Grouping, for full automated driving The 3GPP system shall support relative longitudinal position accuracy of less than 0.5 m for UEs supporting V2X application for platooning in proximity. (if concrete idea, bring it in) |
|  | Set 2 [PR-5.5-009] The 3GPP system shall support to ensure sufficient reliability metrics are reached. | No Change, generic requirement | [CPR.G-008] Group: Generic The 3GPP system shall support mechanisms to ensure sufficient reliability metrics are reached. |
|  | Set 2 [PR-5.5-010] The 3GPP system shall support high connection density for congested traffic. | No Change, generic requirement | [CPR.G-009] Group: Generic The 3GPP system shall support high connection density for congested traffic. NOTE: Example estimate is for worst case US Freeway scenario that does not include arterial roads (i.e. onramps): 5 lanes in each direction or 10 lanes total per highway, for up to 3 highways intersecting = 15,840 cars per mile. (SD) |
| 5.12 Information sharing for limited automated platooning | [PR-5.12-001] The 3GPP system shall be capable of supporting user experienced data rate of [2.75] Mbps between UEs supporting V2X application. | No Change Add a clarification of the driver control. | 5.12-004,005 cover this |
|  | [PR-5.12-002] The 3GPP system shall be capable of supporting user experienced data rate of [2.5] Mbps between a UE supporting V2X application and an RSU. | No Change Add a clarification from the use case that this is for limited vehicle platooning/driver control | 5.12-004,005 cover this |
|  | [PR-5.12-003] The 3GPP system shall be capable of transferring periodic broadcast/multicast messages between two UEs supporting V2X application with message payloads of [6500] bytes, not including security-related message component. | This requirements of [PR-5.12-003] and [PR-12.1-006] of the same use case can be combined. Same change of “system” into “network” Add a clarification of driver control | [CPR.7.2.2-016] Group: platooning The 3GPP system shall be capable of transferring periodic broadcast/multicast messages between two UEs supporting V2X applications, directly or via an RSU, with a maximum application-layer end-to-end latency of [20] ms. and with message payloads of [6500] bytes , not including security-related message component to enable the automated platooning with driver control. |
|  | [PR-5.12-004] The 3GPP system shall be capable of transferring periodic broadcast/multicast messages between a UE supporting V2X application and an RSU with message payloads of [6000] bytes, not including security-related message component. | This requirements of [PR-5.12-004] and [PR-12.1-007] of the same use case can be combined. Change of “system” into “network” | [CPR.7.2.2-017] Group: platooning The 3GPP system shall be capable of transferring periodic broadcast/multicast messages between a UE supporting V2X application and an RSU with a maximum application-layer end-to-end latency of [20] ms and with message payloads of [6000] bytes not including security-related message component to enable the automated platooning with driver control. |
|  | [PR-5.12-005] The 3GPP system shall be capable of supporting a maximum frequency of [50] messages per second per transmitting UE. | No Change Add a clarification of the use case served by this requirement: limited vehicle platooning with driver control | [CPR.7.2.2-018] Group: platooning The 3GPP system shall be capable of supporting a maximum frequency of [50] messages per second per transmitting UE to enable the automated platooning with driver control. |
|  | [PR-5.12-006] The 3GPP system shall be capable of transferring messages between two UEs supporting V2X application, directly or via an RSU, with a maximum application-layer end-to-end latency of [19] ms. | This requirements of [PR-5.12-003] and [PR-12.1-006] of the same use case can be combined. | Merged |
|  | [PR-5.12-007] The 3GPP system shall be capable of transferring messages between a UE supporting V2X application and an RSU with a maximum application-layer end-to-end latency of [19] ms. | This requirements of [PR-5.12-004] and[PR-12.1-007] of the same use case can be combined | Merged |
|  | [PR-5.12-008] The 3GPP system shall be capable of supporting high reliability without requiring application-layer message retransmissions. | This is a generic requirement valid for all V2X use cases. This requirement is the same as [PR-5.10-008]. No new requirement | Already in [CPR.7.2.1-012] |
|  | [PR-5.12-009] The 3GPP system shall be capable of supporting a communication range sufficient to give the predictive information to the vehicles in proximity (e.g. [10] sec * (maximum relative speed) [m/s]). | No Change Add a clarification of the use case served by this requirement: limited vehicle platooning with driver control. | [CPR.7.2.2-019] Group: platooning The 3GPP system shall be capable of supporting a communication range sufficient to give the predictive information to the vehicles in proximity (e.g. [10] sec * (maximum relative speed) [m/s]) to enable the automated platooning with driver control. |
|  | [PR-5.12-010] The 3GPP system shall be capable of supporting a high connection density. | No Change | [CPR.7.2.2-020] Group: platooning The 3GPP system shall be capable of supporting a high connection density. |
| 5.13 Information sharing for full automated platooning | [PR-5.13-001] The 3GPP system shall be capable of supporting user experienced data rate of [65] Mbps between UEs supporting V2X application. | No Change Add a clarification of the use case served by this requirement: full automated vehicle platooning | Group: platooning [CPR.7.2.2-021] The 3GPP system shall be capable of supporting user experienced data rate of [65] Mbps between UEs supporting V2X application, to enable fully automated platooning. |
|  | [PR-5.13-002] The 3GPP system shall be capable of supporting user experienced data rate of [50] Mbps between a UE supporting V2X application and an RSU. | No Change Add a clarification of the use case served by this requirement: full automated vehicle platooning | Group: platooning [CPR.7.2.2-022] The 3GPP system shall be capable of supporting user experienced data rate of [50] Mbps between a UE supporting V2X application and an RSU, to enable fully automated platooning. |
|  | [PR-5.13-003] The 3GPP system shall be capable of transferring messages between two UEs supporting V2X application, directly or via an RSU, with a maximum application-layer end-to-end latency of [19] ms. | No Change Add a clarification of the use case served by this requirement: full automated vehicle platooning | Group: platooning [CPR.7.2.2-023] The 3GPP system shall be capable of transferring messages between two UEs supporting V2X application, directly or via an RSU, with a maximum application-layer end-to-end latency of [20] ms, to enable fully automated platooning. |
|  | [PR-5.13-004] The 3GPP system shall be capable of transferring messages between a UE supporting V2X application and an RSU with a maximum application-layer end-to-end latency of [19] ms. | No Change Add a clarification of the use case served by this requirement: full automated vehicle platooning | Group: platooning [CPR.7.2.2-024] The 3GPP system shall be capable of transferring messages between a UE supporting V2X application and an RSU with a maximum application-layer end-to-end latency of [20] ms to enable fully automated platooning. |
|  | [PR-5.13-005] The 3GPP system shall be capable of supporting high reliability without requiring application-layer message retransmissions. | This is a generic requirement valid for all V2X use cases. | N/A |
|  | [PR-5.13-006] The 3GPP system shall be capable of supporting a communication range sufficient to give the predictive information to the vehicles in proximity (e.g. [5] sec * (maximum relative speed) [m/s]). | No Change Add a clarification of the use case served by this requirement is added | Group: platooning [CPR.7.2.2-025] The 3GPP system shall be capable of supporting a communication range sufficient to give the predictive information to the vehicles in proximity (e.g. [5] sec * (maximum relative speed) [m/s]) to enable full automated platooning. |
|  | [PR-5.13-007] The 3GPP system shall be capable of supporting a high connection density. | Requirements of [PR-5.12-010] and [PR-5.13-007] can be merged as the need is the same whatever the level of automation and density of platooning. | Merged; [CPR.7.2.2-020] |

| 5.17 Changing Driving-Mode | [PR.5.17-001] The 3GPP system shall be able to support reliable V2V communications between a specific UE supporting V2X applications and up to 19 other UEs supporting V2X applications. | No Change Add a clarification that the use case is vehicle platooning | Group: Platoonning 3306 (LG) should reflect this change in mapping; [CPR.P-002] For Vehicle Platooning, the 3GPP system shall be able to support reliable V2V communications between a specific UE supporting V2X applications and up to 19 other UEs supporting V2X applications. |
| --- | --- | --- | --- |

| 5.3 Automotive: Sensor and State Map Sharing | [PR-5.3-001] The 3GPP system shall support less than 10ms latency between V2X applications. | This requirement is combined with [PR-5.3-003] and [PR-5.3-002] of the same use case. Considering the message is exchanged between UE, replace “system” by “network” | Group: Cooperative Awareness/Sensor exchange [CPR.7.2.6-001] The 3GPP system shall support message exchange between UEs supporting V2X applications with less than 10ms latency, reliability of 95% and support peak data rate up to [25 Mbps] per UE for occasional transmissions. (agreed 3305) |
| --- | --- | --- | --- |
|  | [PR-5.3-002] The 3GPP system shall support message reliability of 95% between V2X applications. | This requirement is complement to [PR-5.3-001] of the same use case. | ## Merged; |
|  | [PR-5.3-003] The 3GPP system shall support burst transmission of large quantities of data [TBD Mbs]. | This requirement is complement to [PR-5.3-001] of the same use case. | ## Merged; |
|  | [PR-5.3-004] The 3GPP system shall support efficient coordination of radio resources used for V2X communication to maximize the utilisation of the available spectrum and to ensure the required reliability. | This generic, valid for all V2X use cases. Editorial clarification | [CPR.G-010] Group: Generic The 3GPP system shall support efficient coordination of radio resources used for transport of V2X communications to maximize the utilisation of the available spectrum and to ensure the required reliability. |
|  | [PR-5.3-005] The 3GPP system shall support high connection density for congested traffic, estimate below is for worst case US Freeway scenario that does not include arterial roads (i.e. onramps): 5 lanes in each direction or 10 lanes total per highway, for up to 3 highways intersecting = 15,840 cars per mile. | [PR-5.5-010] | Merged; |

| 5.4 eV2X support for Remote Driving | [PR.5.4-001] The 3GPP system shall support user experienced data rate up to 1 Mbps at DL and 20 Mbps at UL for UE supporting V2X application for an absolute speed of up to 250 km/h. | No change in the requirement. Add a clarification that the use case eV2X support for Remote Driving is for: Remote Driving. Clarify that this is without driver control Merged with requirement [PR.5.4-003] of the same use case. | Group: Remote Driving Scenario: eV2X support for Remote Driving, operated by a machine Server without driver control, [CPR.R-001] [CPR.7.2.10-001] The 3GPP system shall support user experienced data rate up to 1 Mbps at DL and 20 Mbps at UL for UE supporting V2X application between V2X application server and UE for an absolute speed of up to 250 km/h. Ericsson: “safety-related” and other were proposed to be merged (001 vs. (003, 004)); QC; LG: all are related to “safety” in the context of “remote driving”; tentative agreement: leave three separate; |
| --- | --- | --- | --- |
|  | [PR.5.4-002] The 3GPP system shall be able to control the UL and DL reliability of V2X communication, depending on the requirement of V2X application. | This is a generic requirement. Editorial change | [CPR.G-011] Group: Generic The 3GPP system shall be able to control the UL and DL reliability of transport of V2X communications, depending on the requirement of V2X application |
|  | [PR.5.4-003] The 3GPP system shall support ultra-high UL and DL reliability (99.999 % or higher) for UE supporting safety-related V2X application. | No change Merge with [PR.5.4-001] as complement in the same use. | Group: Remote Driving [CPR.R-002] The 3GPP system shall support ultra-high UL and DL reliability (99.999 % or higher) for UE supporting safety-related V2X application. |
|  | [PR.5.4-004] The 3GPP system shall support end-to-end latency 5 ms between V2X application server and UE supporting safety-related V2X application for an absolute speed of up to 250 km/h. | No Change Add also a clarification of the use case served by this requirement: to support teleoperated driving. Note: there is no need to clarify again that V2X is for safety application | [CPR.R-003] [PR.5.4-004] The 3GPP system shall support end-to-end latency 5 ms between V2X application server and UE supporting safety-related V2X application for an absolute speed of up to 250 km/h. |

| 5.6 Collective Perception of Environment | Set 1 [PR.5.6-001] The 3GPP system shall be able to support [100ms] end-to-end latency for message transfer among a group of UEs supporting V2X application. | No change in the requirement. Add a clarification that the use case is also collective perception of environment with driver control. Merged with requirement [PR-5.6-002] and [PR.5.6-003] of the same use case. | Group: Extended sensors [CPR.7.2.7-001] The 3GPP system shall be able to support [100ms] end-to-end latency and [99%] reliability for support periodic transmission of data packets [e.g. 1600 bytes] among a group of UEs supporting V2X application within [1000m] communication range. |
| --- | --- | --- | --- |
|  | Set 1 [PR.5.6-002] The 3GPP system shall be able to support over [99%] target packet delivery reliability rate in [1000meters]. | No change Merge with [PR.5.6-001] as complement in the same use. | Merged |
|  | Set 1 [PR.5.6-003] The 3GPP system shall be able to support periodic transmission of data packets [e.g. 1600 bytes]. | No change Merge with [PR.5.6-001] as complement in the same use | Merged |
|  | Set 2 [PR.5.6-004] The 3GPP system shall be able to support [3ms] end-to-end latency and [99.999%] reliability for message transfer among a group of UEs supporting V2X application within [200m] communication range. | No Change This is the same use case as [PR.5.6-001] but for fully automated driving and a specific degree for its KPIs. | Group: CoExtended Sensors [CPR.7.2.7-002] The 3GPP system shall be able to support [3ms] end-to-end latency and [99.999%] reliability for message transfer among a group of UEs supporting V2X application within [200m] communication range. |
|  | Set 2 [PR.5.6-005] The 3GPP system shall be able to support [10ms] end-to-end latency and [99.99%] reliability for message transfer among a group of UEs supporting V2X application within [500m] communication range. | No Change This is the same use case as [PR5.6-004] but for a different degree of KPIs | Group: Extended Sensors [CPR.7.2.7-003] The 3GPP system shall be able to support [10ms] end-to-end latency and [99.99%] reliability for message transfer among a group of UEs supporting V2X application within [500m] communication range. |
|  | Set 2 [PR.5.6-006] The 3GPP system shall be able to support [50ms] end-to-end latency and [99%] reliability for message transfer among a group of UEs supporting V2X application within [1000m] communication range. | No Change This is the same use case as [PR5.6-004] but for a different degree of KPIs | Group: Extended Sensors[CPR.7.2.7-004]The 3GPP system shall be able to support [50ms] end-to-end latency and [99%] reliability for message transfer among a group of UEs supporting V2X application within [1000m] communication range. |
|  | Set 2 [PR.5.6-007] The 3GPP system shall be able to support peak data rate of [1Gbps] for a single UE for a short period of time in range of [50meters], in case of imminent collision. | No Change This is the same use case as [PR5.6-004] but for a different degree of KPIs | Group: Extended Sensors [CPR.7.2.2-021] The 3GPP system shall be able to support peak data rate of [1Gbps] for a single UE for a short period of time in range of [50meters], in case of imminent collision. |

| 5.7 Communication between vehicles of different 3GPP RATs | [PR.5.7-001] UE using 5G RA(N) based V2X communication shall be able to communicate with UE using E-UTRA(N) based V2X communication. | No Change, generic requirement | [CPR.G-012] Group: Generic UE using 5G RA(N) based V2X communication shall be able to communicate with UE using E-UTRA(N) based V2X communication. |
| --- | --- | --- | --- |
|  | [PR.5.7-002] Impact to E-UTRA(N) by UE supporting only 5G RAT(s) based V2X communication shall be minimized. | No Change, generic requirement | [CPR.G-012] Group: Generic Impact to E-UTRA(N) by UE supporting only 5G RAT(s) based V2X communication shall be minimized. |
|  | [PR.5.7-003] Impact to 5G RA(N) by UE supporting only E-UTRA based V2X communication shall be minimized. | No Change, generic requirement | [CPR.G-013] Group: Generic Impact to 5G RA(N) by UE supporting only E-UTRA based V2X communication shall be minimized. |

| 5.8 Multi-PLMN environment | [PR-5.8-001] The 3GPP system shall be able to support message transfer between UEs or between a UE and a UE-type RSU, regardless of whether or not they are subscribers of the same PLMN supporting V2X communications. In case they are subscribers to different PLMNs, there shall be no service degradation of the message transfer. | Generic requirement No change | [CPR.G-014] Group: Generic The 3GPP system shall be able to support message transfer between UEs or between a UE and a UE-type RSU, regardless of whether or not they are subscribers of the same PLMN supporting V2X communications. In case they are subscribers to different PLMNs, there shall be no service degradation of the message transfer. |
| --- | --- | --- | --- |

| 5.9 Cooperative Collision Avoidance (CoCA) of connected automated vehicles | [PR-5.9-001] The 3GPP network shall enable communication between UEs to support message exchange with [10 Mbps] data throughput with less than [10 ms] latency. | No change. Add a clarification that the use case is for fully automated driving. Modify 2MByte into 2 Kbytes as the message sent includes only the trajectory of a vehicle and some sensor objects. Merge [PR-5.9-002] as for the same use case. | Group: Advanced driving [CPR.7.2.3-001] The 3GPP network shall enable communication between UEs to support message exchange with [10 Mbps] data throughput with less than [10 ms] latency and with [99.99%] reliability and message sizes in range up to [2 KByte] |
| --- | --- | --- | --- |
|  | [PR-5.9-002] The 3GPP network shall enable UE’s message exchange with [99.99%] reliability and message sizes in range up to [2 MByte]. | Merge with [PR-5.9-001] as they are complement in the same use case. | Merged |

| 5.10 Information sharing for limited automated driving | [PR-5.10-001] The 3GPP system shall be capable of supporting user experienced data rate of [0.55] Mbps between UEs supporting V2X application. |  | No change. Add a clarification that the use case information sharing is with driver control. Clarify that this is with driver control. Requirement of [PR-5.10-001], [PR-5.10-003], [PR-5.10-006] and [PR-5.10-009] of the same use case for communication between UEs are merged. |  | Group: Advanced driving [CPR.7.2.5-001] DCM; |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | [PR-5.10-002] The 3GPP system shall be capable of supporting user experienced data rate of [0.5] Mbps between a UE supporting V2X application and an RSU. |  | No change Add a clarification that the use case information sharing is for: Automated Driving. Clarify that this is with driver control. The requirements of [PR-5.10-002] and [PR-5.2-004], [PR-5.10-007] and [PR-5.10-009] of the same use case can be combined as complementary. |  | Group: Advanced driving Scenario: Information sharing for limited automated driving, exchanging periodically V2X messages between UEs and an RSU with driver control. DCM; |  |
|  | [PR-5.10-003] The 3GPP system shall be capable of transferring periodic broadcast/multicast messages between two UEs supporting V2X application with message payloads of [6500] bytes, not including security-related message component. |  | No change “Broadcast /multicast” is already considered in LTEV2X specification so does not need to be replicated in Rel-15 LTE/NR V2X. Requirement of [PR-5.10-001], [PR-5.10-003], [PR-5.10-006] and [PR-5.10-009] of the same use case for communication between UEs are merged. |  | Combined Group: advanced driving [CPR.7.2.5-001] The 3GPP system shall be capable of transferring periodic broadcast/multicast messages between two UEs supporting V2X applications, directly or via an RSU, with a maximum application-layer end-to-end latency of [100] ms. and with message payloads of [6500] bytes , not including security-related message component to enable the advanced driving with driver control |  |
|  | [PR-5.10-004] The 3GPP system shall be capable of transferring periodic broadcast/multicast messages between a UE supporting V2X application and an RSU with message payloads of [6000] bytes, not including security-related message component. |  | No change The requirements of [PR-5.10-002] and [PR-5.2-004], [PR-5.10-007] and [PR-5.10-009] of the same use case can be combined as complementary. |  | Group: adv. driving [CPR.7.2.5-002a] The 3GPP system shall be capable of transferring periodic broadcast/multicast messages between a UE supporting V2X application and an RSU with a maximum application-layer end-to-end latency of [100] ms and with message payloads of [6000] bytes not including security-related message component to enable the advanced driving with driver control. |  |
|  | [PR-5.10-005] The 3GPP system shall be capable of supporting a maximum frequency of [10] messages per second per transmitting UE. |  | No Change Add a clarification that the use case is Automated Driving with driver control for information sharing. |  | Group: Advanced driving [CPR.7.2.5-003] The 3GPP system shall be capable of supporting a maximum frequency of [10] messages per second per transmitting UE, to enable advanced driving with driver control. |  |
|  | [PR-5.10-006] The 3GPP system shall be capable of transferring messages between two UEs supporting V2X application, directly or via an RSU, with a maximum application-layer end-to-end latency of [100] ms. |  | No change Requirement of [PR-5.10-001], [PR-5.10-003],[PR-5.10-006] and [PR-5.10-009] of the same use case for communication between UEs are merged. |  | Combined |  |
|  | [PR-5.10-007] The 3GPP system shall be capable of transferring messages between a UE supporting V2X application and an RSU with a maximum application-layer end-to-end latency of [100] ms. |  | No change The requirements of [PR-5.10-002] and [PR-5.2-004], [PR-5.10-007] and [PR-5.10-009] of the same use case can be combined as complementary. |  | Combined with [PR-5.10-002] |  |
|  | [PR-5.10-008] The 3GPP system shall be capable of supporting high reliability without requiring application-layer message retransmissions. |  | No Change, this is a generic requirement, which is valid for all V2X use cases. |  | Group: Generic [CPR.G-015] The 3GPP system shall be capable of supporting high reliability without requiring application-layer message retransmissions. |  |
|  | [PR-5.10-009] The 3GPP system shall be capable of supporting a communication range sufficient to give the predictive information to the vehicles in proximity (e.g. [10] sec * (maximum relative speed) [m/s]). |  | This requirement is need for messages’ exchanged between UEs supporting V2X application, as well as between a UE supporting V2X application and an RSU. Requirement of [PR-5.10-001], [PR-5.10-003], [PR-5.10-006] and [PR-5.10-009] of the same use case for communication between UEs are merged. The requirements of [PR-5.10-002] and [PR-5.2-004], [PR-5.10-007] and [PR-5.10-009] of the same use case can be combined as complementary. |  | Group: adv. Driving The 3GPP system shall be capable of supporting a communication range sufficient to give the predictive information to the vehicles in proximity (e.g. [10] sec * (maximum relative speed) [m/s]). |  |
|  | [PR-5.10-010] The 3GPP system shall be capable of supporting a high connection density |  | This requirement is the same as [PR-5.5-010] No new requirement |  | N/A |  |
| 5.11 Information sharing for full automated driving |  | [PR-5.11-001] The 3GPP system shall be capable of supporting user experienced data rate of [53] Mbps between UEs supporting V2X application. | No Change. Add a clarification that the use case information sharing is for: Automated Driving. Clarify that this is fully automated. Change “system” into “network” as data is transfered between UEs Requirements [PR-5.11-001], [PR-5.11-003] and [PR-5.11-006] can be combined as supporting the same use case. | Group: Advanced driving Scenario: Information sharing for fully automated driving, exchanging periodically V2X messages between UEs supporting V2X applications, fully automated. [PR-5.11-001] The 3GPP system shall be capable of supporting user experienced data rate of [53] Mbps between UEs supporting V2X application. b/w UEs,; (001, 003, 006) |  |
|  |  | [PR-5.11-002] The 3GPP system shall be capable of supporting user experienced data rate of [50] Mbps between a UE supporting V2X application and an RSU. | No change. Add a clarification that the use case information sharing is for: Automated Driving. Change “system” into “network” as between UEs Clarify that this is fully automated. Requirements [PR-5.11-002], [PR-5.11-004] and [PR-5.11-006] can be combined as supporting the same use case. | Group: Advanced driving Scenario: Information sharing for fully automated driving, exchanging periodically V2X messages between UEs supporting V2X applications and an RSU, fully automated. [PR-5.11-002] The 3GPP system shall be capable of supporting user experienced data rate of [50] Mbps between a UE supporting V2X application and an RSU. b/w UE and RSU (002,004,006) |  |
|  |  | [PR-5.11-003] The 3GPP system shall be capable of transferring messages between two UEs supporting V2X application, directly or via an RSU, with a maximum application-layer end-to-end latency of [100] ms. | Requirements [PR-5.11-001], [PR-5.11-003] and [PR-5.11-006] can be combined as supporting the same use case. | Group: Advanced driving [PR-5.11-003] The 3GPP system shall be capable of transferring messages between two UEs supporting V2X application, directly or via an RSU, with a maximum application-layer end-to-end latency of [100] ms. |  |
|  |  | [PR-5.11-004] The 3GPP system shall be capable of transferring messages between a UE supporting V2X application and an RSU with a maximum application-layer end-to-end latency of [100] ms. | Requirements PR-5.11-002], [PR-5.11-004] and [PR-5.11-006] can be combined as supporting the same use case. | Group: Advanced driving [PR-5.11-004] The 3GPP system shall be capable of transferring messages between a UE supporting V2X application and an RSU with a maximum application-layer end-to-end latency of [100] ms. b/w UE and RSU (002,004,006) |  |
|  |  | [PR-5.11-005] The 3GPP system shall be capable of supporting high reliability without requiring application-layer message retransmissions. | The requirement is the same as [PR-5.10-008] in “Information sharing for limited automated driving”. No new requirement. | N/A |  |
|  |  | [PR-5.11-006] The 3GPP system shall be capable of supporting a communication range sufficient to give the predictive information to the vehicles in proximity (e.g. [5] sec * (maximum relative speed) [m/s]). | This requirement is need for messages’ exchanged between UEs supporting V2X application, as well as between a UE supporting V2X application and an RSU. Requirements [PR-5.11-002], [PR-5.11-004] and [PR-5.11-006] can be combined as supporting the same use case. Also, Requirements [PR-5.11-001], [PR-5.11-003] and [PR-5.11-006] can be combined as supporting the same use case. | Group: Advanced driving [PR-5.11-006] The 3GPP system shall be capable of supporting a communication range sufficient to give the predictive information to the vehicles in proximity (e.g. [5] sec * (maximum relative speed) b/w UEs (001,003, 005) + b/w UE and RSU (002,004,006); |  |
|  |  | [PR-5.11-007] The 3GPP system shall be capable of supporting a high connection density. | This requirement is the same as [PR-5.5-010] No new requirement | N/A |  |

| 5.14 Dynamic Ride Sharing | [PR.5.14-001] A UE supporting a V2X application shall be able to discover other such UEs in proximity supporting the same application. | wording changed | [CPR.G-016] [CPR.7.2.1-013] Group: Generic [CPR.G-016] The 3GPP system shall enable discovery and communication between UEs supporting the same V2X application |
| --- | --- | --- | --- |
|  | [PR.5.14-002] A UE supporting a V2X application shall be able to establish direct unicast communication with another such UE based on discovery results. | Merged into [CPR.G-016] | [CPR.G-017] N/A |
| 5.15 Use case on Multi-RAT | [PR.5.15-001] The 3GPP network shall allow the operator to associate a V2X application to one or more 3GPP RATs in a UE supporting V2X application. | wording changed | [CPR.G-018] Group: Generic The 3GPP system shall be able to support for the operator to select which 3GPP RAT to use for a V2X application. |

| 5.16 Video data sharing for assisted and improved automated Driving (VaD) | Set 1 [PR-5.16-001] The 3GPP network shall enable communication between UEs with [10Mbps] data rate, less than [50ms] latency and [90%] reliability within a communication range of [100m]. | No Change The TR states “KPIs for set 1 enables service to be considered by a human visual system (driver is still in the control loop, but it does not exclude being a machine). We propose to add this clarification. | Group: Extended Sensors? Group: Extended Sensor 3306 (LG, mapping/grouping) [CPR.7.2.8-001] For video sharing for semi-automated driving, the 3GPP system shall enable communication between UEs supporting V2X application with video sharing with [10Mbps] data rate, less than [50ms] latency and [90%] reliability within a communication range of [100m]. |
| --- | --- | --- | --- |
|  | Set 2 [PR-5.16-002] The 3GPP network shall enable communication with up to [700Mbps] data rate, less than [10ms] latency and [99.99%] reliability within communication range of [500m]. | No Change This it is the same scenario as [PR.5.6-001] but the KPIs are for a fully automated driving (driver control is not possible). | [CPR.7.2.8-002]: For cooperative awareness with video sharing with fully automated driving, the 3GPP system shall enable communication between UEs supporting V2X application with video sharing with up to [700Mbps] data rate, less than [10ms] latency and [99.99%] reliability within communication range of [500m]. |

| 5.18 Tethering via Vehicle | [PR.5.18-001] The 3GPP system shall enable a UE supporting a V2X application to obtain network access via another UE supporting V2X application. | No Change, this is a generic requirement | [CPR.G-019] Group: Generic The 3GPP system shall enable a UE supporting a V2X application to obtain network access via another UE supporting V2X application. |
| --- | --- | --- | --- |
|  | [PR.5.18-002] The 3GPP system shall enable a UE supporting a V2X application to discover another UE supporting V2X application that can offer access to the network. | No Change, this is a generic requirement | [CPR.G-020] Group: Generic The 3GPP system shall enable a UE supporting a V2X application to discover another UE supporting V2X application that can offer access to the network. |
|  | [PR.5.18-003] The 3GPP system shall enable a UE supporting a V2X application to switch the network access from direct 3GPP connection to an indirect 3GPP connection via another UE supporting V2X application that is connected via 3GPP access to the 3GPP core network, and vice versa. | No Change, this is a generic requirement | [CPR.G-021] [CPR.7.2.1-018] Group: Generic The 3GPP system shall enable a UE supporting a V2X application to switch the network access from direct 3GPP connection to an indirect 3GPP connection via another UE supporting V2X application that is connected via 3GPP access to the 3GPP core network, and vice versa. |
|  | [PR.5.18-004] The 3GPP system should provide integrity and confidentiality protection (end to end) for the network access traffic of a V2X UE via another such UE. | No Change, this is a generic requirement | [CPR.G-022] Group: Generic The 3GPP system should provide integrity and confidentiality protection (end to end) for the network access traffic of a V2X UE via another such UE. |

| 5.19 Use case out of 5G coverage | [PR.5.19-001] The 3GPP system shall allow UEs supporting V2X application to use 5G RAT for direct communication when the UEs are not being serviced by a 5G cell. | No Change, this is a generic requirement | [CPR.G-023] Group: Generic The 3GPP system shall allow UEs supporting V2X application to use 5G RAT for direct communication when the UEs are not being serviced by a 5G cell. “5G” still “5G” (it was intended to be New Radio only; to consider this as per PCG’s input) |
| --- | --- | --- | --- |

| 5.20 Emergency Trajectory Alignment | [PR-5.20-001] The 3GPP network shall enable communication between UEs with data rate [30Mbps], less than [3ms] radio latency and [99.999%] reliability within communication range of [500m]. | No Change | Group: Advanced Driving [CPR.7.2.4-001] The 3GPP network shall enable communication between UEs supporting V2X application for cooperative manoeuvre with data rate [30Mbps], less than [3ms] radio latency and [99.999%] reliability within communication range of [500m]. |
| --- | --- | --- | --- |

| 5.21 Teleoperated Support (TeSo) | [PR-5.21-001] The 3GPP network shall enable communication between a UE and a server (e.g. far end server or a server hosted by another UE), to exchange messages to support [15Mbps] in uplink and downlink direction with less than [20ms] end-to-end latency and [99.999%] reliability. | Add a clarification that the use case Teleoperated Support (TeSo) is for: Remote Driving. Clarify that this is with driver control. | Group: Remote Driving, Use case: Teleoperated Support (TeSo) with driver control . [CPR.7.2.10-002] The 3GPP network shall enable communication between a UE and a server (e.g. far end server or a server hosted by another UE), to exchange messages to support [25Mbps] in uplink and [1Mpbs] in downlink direction with less than [20ms] end-to-end latency and [99.999%] reliability. 3303; |
| --- | --- | --- | --- |

| 5.22 Intersection Safety Information Provisioning for Urban Driving | [PR-5.22-001] The 3GPP system shall be able to support an average [0.5Mbps] in downlink and [50Mbps] in uplink. | No Change. | Group: Adv. Driving [CPR.7.2.9-001] The 3GPP system shall be able to support an average [0.5Mbps] in downlink and [50Mbps] in uplink. |
| --- | --- | --- | --- |
|  | [PR-5.22-002] An RSU shall be able to communicate with up to 200 UEs supporting a V2X application. | No Change, this is a generic requirement | [CPR.G-024] Group: Generic An RSU shall be able to communicate with up to 200 UEs supporting a V2X application. |
|  | [PR-5.22-003] RSU shall be able to support [50] packet transmission per a second with an average message size [450] bytes. | No Change Add a clarification that the use case is provision of intersection safety information. | [CPR.7.2.9-002] The RSU shall be able to support [50] packet transmission per a second with an average message size [450] bytes. |