## 3GPP TSG-SA WG1 Meeting #76bis				S1-170023

Spokane, US, 16-20, Jan 2017	(revision of S1-17xxxx)

Title: 	new use cases for transmission of real-time data

Agenda Item:	6.2

Source: 	Huawei, Hisilicon

Contact: 	Carry Liu (liujianning.liu@huawei.com )

Laurence Meriau (laurence.meriau@huawei.com)

Abstract: This document is aim to propose use cases for transmission of real-time data based on requirements from the China railway industry, which also align with the URS from UIC.

Reference

[1] Future Railway Mobile Communication System User Requirements Specification, UIC

Discussion

In the railway industry, real time data communication is indispensable and shall be supported in FRMCS system. For example, some key information, e.g., health and status of train, vital parameter condition, and onset of fault condition data, shall be supported to transfer from intelligent on-train systems to train maintenance [1].

Proposal

To add the proposed text into TR 22.989

------------------------------- Start of Content------------------------------

## 7.X	Use case：Real time data transmission

#### 7.X.1	Description

This use case allows FRMCS users to exchange real time data from each other, for example, health and status of train are transferred between intelligent on-train systems to train maintenance.

#### 7.X.2	Pre-conditions

The user A, B and C are entitled FRMCS user.

User A is a FRMCS administrator.

User B is an equipment for monitoring the train.

User C is a normal FRMCS user

#### 7.X.3	Service flows

User A requests User B to transmit the real time data (e.g., health, status of train, etc.).

User B receives the request from User A, and accepts the request.

User B starts to transmit the real time data to User A.

User C requests User B to transmit the real time data (e.g., health, status of train, etc.).

User B receives the request from C, and rejects the request (e.g., without authorization to the data).

User C receives a notification that the request was rejected by User B.

#### 7.X.4	Post-conditions

The real time data of User B is sent to User A.

The real time data of User B is not sent to User C.

#### 7.X.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-7.X.5-001] | The FRMCS system shall enable an entitled FRMCS-user to request another FRMCS user to transmit data in real time. | A | Not covered |  |
| [R-7.X.5-002] | The FRMCS user shall be able to accept a real time data transmission request. | A | Not covered |  |
| [R-7.X.5-003] | The FRMCS user shall be able to reject a real time data transmission request. | A | Not covered |  |
| [R-7.X.5-004] | The FRMCS user shall be able to ignore a real time data transmission request. | A | Not covered |  |
| [R-7.X.5-005] | The requesting FRMCS user shall be notified if the request is rejected or ignored. | A | Not covered |  |

------------------------------- End of Content------------------------------