3GPP TSG-SA WG1 Meeting #76-bis	S1-170217

Spokane, US, 16-20 January 2017	(revision of S1-170208 )

Title:	Dynamic Policy Control

Agenda Item:	5.1 Smarter: New Services and Markets Technology Enablers [SP-160486]

Source:	U.S. Department of Commerce

Contact:	Ihab.Guirguis@FirstNet.gov

Andrew@its.bldrdoc.gov

Abstract: This document proposes to add a requirement in sub-clause 6.8.2 for Dynamic Policy Control in the SMARTER TS 22.261 V1.0.0 (2016-12)

# Proposal

This document proposes to add a requirement in sub-clause 6.8.2 for Dynamic Policy Control in the SMARTER TS 22.261 V1.0.0 (2016-12). This additional requirement addressed the need to maintain the stability of existing identified high priority session in case of real-time change in priority.

--- Beginning of Change ---

## 6.8	Dynamic Policy Control

### 6.8.1	Description

Editor's Note: Text in this section is to be added.

### 6.8.2	Requirements

The 5G system shall support the creation and enforcement of prioritisation policy for users and traffic, during connection setup and when connected.

NOTE:	Relative prioritization, pre-emption, and precedence of critical traffic associated with certain priority services (e.g., MPS and Emergency) are subject to regional/national regulatory and operator policies.

The 5G system shall support optimised signalling for prioritised users and traffic where such signalling is prioritized over other signalling traffic.

Based on operator policy, the 5G system shall allow flexible means to create and enforce relative priority among the different service flows by authorized users.

Based on operator policy, the 5G system shall allow real-time, dynamic, secure, limited interaction with the QoS and policy framework for modification of the QoS and policy framework by authorized users.

Based on operator policy, the 5G system shall maintain a session when prioritization of that session increases in real time.

Based on operator policy, the 5G system shall allow modifications by authorized users and context aware network functionality to the QoS and policy framework to have a variable duration.

## 6.9	Connectivity models

--- End of Change ---