3GPP TSG-SA WG1 Meeting #76-Bis	S1-170244

Spokane, WA, 16-20 January 2017	(revision of S1-170135)

Title:	SMARTER - Authentication

Agenda Item:	5.1

Source:				Qualcomm Incorporated, Intel, NEC

Contact:			Jack Nasielski   jackn@qti.qualcomm.com

Abstract: This document provides text for the SMARTER TS Security clause. The requirements are drawn from the SMARTER building block TRs.

## --- 1st change ---

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Editor's Note: The definitions of active communication and service continuity propose changes to 21.905 and need to be propagated once agreed in SA1. To add to 21.905 we need to consider another note for service continuity, e.g.,

NOTE x: 	Examples of access changes include the following. From pre-5G: CS/PS domain change. From 5G: switching between a direct 3GPP connection and an indirect 3GPP connection.

3GPP connection: The connection between a device and a 3GPP network.

Active communication: a UE is in active communication when it has one or more connections established. A UE may have any combination of PS connections (e.g., PDP contexts, active PDN connections).

Availability (%): Percentage value of the amount of time the 3GPP system is delivering services divided by the amount of time it is expected to deliver services in a specific area.

Direct 3GPP connection: One mode of 3GPP connection, where there is no relay UE between the device and the 3GPP network.

Direct device connection: The connection between two devices without any network entity in the middle.

Hosted Service: A service containing the operator’s own application(s) and/or trusted 3rd party application(s) in the Hosting Service Environment, which can be accessed by the user.

Indirect 3GPP connection: One mode of 3GPP connection, where there is a relay UE between the device and the 3GPP network.

Network slice: A set of network functions and corresponding resources necessary to provide the required telecommunication services and network capabilities.

Private network slice: a network slice dedicated for isolated deployment for trusted 3rd parties. A private network slice creates a private network for the 3rd party where neither the slice nor its users interact with any other slice (private or public) or users in the operator’s network.

## --- End 1st change ---

## --- 2nd change ---

## 8.3	Authentication

The 5G network shall support a resource efficient mechanism for authenticating groups of devices.

The 5G system shall support an efficient means to authenticate a user to a device (e.g., biometrics).

The 5G system shall be able to support authentication over a non-3GPP access technology using 3GPP credentials.

The 5G system shall support operator controlled alternative authentication methods (i.e., alternative to AKA) with different types of credentials for network access for IoT devices to allow new isolated deployment scenarios (e.g., for industrial automation).

Based on operator policy, an operator may provide isolated private network slices that are dedicated for use by trusted 3rd parties. For such isolated slices, the 5G system shall support network access using identities, credentials, and authentication methods provided and managed by the 3rd party and supported by 3GPP.

## --- End 2nd change ---