3GPP TSG-SA WG1 Meeting #77	S1-171129

Jeju, South Korea, 13-17 February 2017	(revision of S1-17xxxx)

Title:	FS_PARLOS – Definition

Agenda Item:	8.5

Source:	Nokia, Alcatel-Lucent Shanghai Bell, Verizon, Alibaba, Qualcomm, LG  Electronics, Sony, Samsung, Sprint

Contact:			Betsy Covell betsy.covell@nokia.com

Abstract: This document proposes a definition for restricted local operator services and expands acronyms used in the TR.

First Change

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Restricted local operator services: communication services provided by an operator that involve either automated or human assistance (e.g., credit card billing, directory assistance, customer care) for which successful authentication is not necessary.

Second Change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CFR	Code of Federal Regulations

URN	Uniform Resource Name

Third Change

### 5.2.5	Potential Requirements

A 3GPP system shall support a mechanism to indicate to UEs that restricted local operator services are available.

Based on operator policy and regional regulations, the 3GPP system shall support a mechanism to configure a set of addresses (e.g., phone number, secure portal) for access to restricted local operator services.

Based on operator policy and regional regulations, the 3GPP system shall support mechanisms to allow access to restricted local operator services by unauthenticated UEs.