3GPP TSG-SA WG1 Meeting #77	S1-171306

Jeju, KOREA, 13-17 FEB 2017	Revision of S1-171022

Title:	eV2X Normative Requirements for Extended Sensors

Agenda Item:	8.1

Source:	LG Electronics

Contact:	SungDuck Chun (sungduck.chun@lge.com)

- Background

At SA1 #76bis, it was agreed that normative requirements for eV2X be incorporated into new TS.

During Study phase of eV2X, potential requirements of 25 use cases were consolidated into 5 categories.

- General Aspect

- Platooning

- Extended sensor

- Advanced driving

- Remote driving

This contribution proposed to adopt requirements related to Extended Sensors into new TS.

Following is latest CPRs considering 22.886V15.0.0 and S1-170319, which were agreed at SA1 #76bis

Table 7.2.4-1 Performance requirements for extended sensors Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Communication range (meters). Section #; Description; CPR #. 5.3; Between UEs supporting V2X application Fully automated driving; [CPR.E-001]; 10; 95; Peak data rate [25]. 5.6; Between UEs supporting V2X application; Driver control; [CPR.E-002]; [1600]; 100; 99; 1000. Fully automated driving; [CPR.E-003]; 3; 99.999; 200. [CPR.E-004]; 10; 99.99; 500. [CPR.E-005]; 50; 99; 1000. [CPR.E-006]; 1000; 50. 5.16; Between UEs supporting V2X application; Driver control/ Automated driving; [CPR.E-007]; [50]; 90; [10]; [100]. Fully automated driving; [CPR.E-008]; [10]; 99.99; [700]; [500].

- Discussion

Overall Comments:

- Because this is contents for TS, the direct link between use case and requirement cannot be pursued.

- The description of each requiremen should be general to avoid the impression that the KPIs is for specifics scenario.

- Avoid using use case name in the description as much as possible.

- ‘fully autonomated driving’ and ‘driver control, limited automated driving’ is not clearly defined.

- According to NHTSA/SAE, there are 6 level of automation for driving. Level 0, 1 and 2 is for ‘Human Driver” and level 3, 4 and 5 is for “automated driving system”.

- Use ‘Lower level of automation’ and “Higher level of automation’ to describe scenario.

- However, background information for each requirement will benefit readers for both in 3GPP and outside of 3GPP.

- Annex can be used to include iformation that allows tracing back to the use case.

- (Payload size * TX rate) = Data rate. If this value does not match, there will be a confusion.

- Either fill “Payload and Tx Rate” or “Data rate”. Not both.

CPR specific comments

Following table summarized discussion/proposed changes for each CPR.

| CPR Of TR 22. 886 | Discussion (empty cell means no change) | Req # for new TS | Suggested changes for new TS (empty cell means CPR is mirror into Req without any change) |
| --- | --- | --- | --- |
| [CPR.E-001] Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Comm Range (meters). Section #; Description; CPR #. 5.3; Between UEs supporting V2X application Fully automated driving; [CPR.E-001]; 10; 95; Peak data rate [25]. | [CPR.E-001] [CPR.E-003] [CPR.E-004] [CPR.E-005] [CPR.E-006] [CPR.E-008] can be grouped together. | [R.5.4-002] |  |
| [CPR.E-002] Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Comm Range (meters). Section #; Description; CPR #. 5.6; Between UEs supporting V2X application; Driver control; [CPR.E-002]; [1600]; 100; 99; 1000. | In section 5.6, repetition rate is mentioned as 5-10HZ | [R.5.4-001] | Clarify that max tx rate is 10hz |
| [CPR.E-003] Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Comm Range (meters). Section #; Description; CPR #. 5.6; Between UEs supporting V2X application; Fully automated driving; [CPR.E-003]; 3; 99.999; 200. | [CPR.E-001] [CPR.E-003] [CPR.E-004] [CPR.E-005] [CPR.E-006] [CPR.E-008] can be grouped together. Size of payload is missing in [003~005]. Section 5.11 mentions 50Mbps for perception up to 278m for suburban. | [R.5.4-003] | Data rate is 50 mbps up to 200 m |
| [CPR.E-004] Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Comm Range (meters). Section #; Description; CPR #. 5.6; Between UEs supporting V2X application; Fully automated driving; [CPR.E-004]; 10; 99.99; 500. | [CPR.E-001] [CPR.E-003] [CPR.E-004] [CPR.E-005] [CPR.E-006] [CPR.E-008] can be grouped together. Size of payload is missing in [003~005]. Section 5.11 mentions 50Mbps for perception up to 278m for suburban. Then, for 1000 m distance, data rate should be smaller than that. | [R.5.4-004] | Data rate is 25 mbps up to 500 m |
| [CPR.E-005] Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Comm Range (meters). Section #; Description; CPR #. 5.6; Between UEs supporting V2X application; Fully automated driving; [CPR.E-005]; 50; 99; 1000. | [CPR.E-001] [CPR.E-003] [CPR.E-004] [CPR.E-005] [CPR.E-006] [CPR.E-008] can be grouped together. Size of payload is missing in [003~005]. Section 5.11 mentions 50Mbps for perception up to 278m for suburban. Then, for 1000 m distance, data rate should be smaller than that. | [R.5.4-005] | Data rate is 10 mbps up to 1000 m |
| [CPR.E-006] Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Comm Range (meters). Section #; Description; CPR #. 5.6; Between UEs supporting V2X application; Fully automated driving; [CPR.E-006]; 1000; 50. | [CPR.E-001] [CPR.E-003] [CPR.E-004] [CPR.E-005] [CPR.E-006] [CPR.E-008] can be grouped together. | [R.5.4-006] |  |
| [CPR.E-007] Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Comm Range (meters). Section #; Description; CPR #. 5.16; Between UEs supporting V2X application; Driver control/ Automated driving; [CPR.E-007]; [50]; 90; [10]; [100]. |  | [R.5.4-007] |  |
| [CPR.E-008] Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Comm Range (meters). Section #; Description; CPR #. 5.16; Between UEs supporting V2X application; Fully automated driving; [CPR.E-008]; [10]; 99.99; [700]; [500]. | [CPR.E-001] [CPR.E-003] [CPR.E-004] [CPR.E-005] [CPR.E-006] [CPR.E-008] can be grouped together. Section 5.16 mentions [27] as reference for ‘700 Mbps’. But, there is no reason why communication range is [500]. CPR.E.006 which requires 1Gbps only states 50m. Merge 008 to 006 and distance is set to 50 m In the reference, it is mentioned that 10 times compression is applicable. Thus, other requirements can cover this requirement with reasonable KPIs. During eV2X session, there was no consensus on the validity of this combination of KPIs. This should be revisited at the next meeting. | R.5.4-008] | Mark that further clarification/ validation is needed. |

- Proposal

It is proposed to agree on the text proposal below.

------------------------------------------------------------------------------------------------

------------------------------ START of first text Proposal----------------------------

------------------------------------------------------------------------------------------------

5.4	Requirements to support Extended Sensors

Table 5.4-1 Performance requirements for extended sensors

| Communication scenario |  | Req # | Payload (Bytes) | Tx rate (Message/ Sec) | Max end-to-end latency (ms) | Reliability (%) | Data rate (Mbps) | Minimum required communication range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |  |  |  |
| Sensor information sharing between UEs supporting V2X application | Lower degree of automation | [R.5.4-001] | [1600] | 10 | 100 | 99 |  | 1000 |
|  | Higher degree of automation | [R.5.4-002] |  |  | 10 | 95 | [25] (NOTE 1) |  |
|  |  | [R.5.4-003] |  |  | 3 | 99.999 | [50] | 200 |
|  |  | [R.5.4-004] |  |  | 10 | 99.99 | [25] | 500 |
|  |  | [R.5.4-005] |  |  | 50 | 99 | [10] | 1000 |
|  |  | [R.5.4-006] (NOTE 2) |  |  | 10 | 99.99 | 1000 | 50 |
| Video sharing between UEs supporting V2X application | Lower degree of automation | [R.5.4-007] |  |  | [50] | 90 | [10] | [100] |
|  | Higher degree of automation | [R.5.4-008] (NOTE 3) |  |  | [10] | 99.99 | [700] | [TBD] |
| NOTE 1: This is peak data rate. NOTE 2: This is for imminent collision scenario. NOTE 3: The KPI values (e.g. data rate, minimum required communication range) in this requirement needs update based on further study. |  |  |  |  |  |  |  |  |

------------------------------------------------------------------------------------------------

------------------------------ START of 2nd text Proposal----------------------------

------------------------------------------------------------------------------------------------

Annex <X> (informative):
<Background information on service Requirements>

For each requirement in this specification, Table X-1 indicates reference to the consolidated potential requirements in TR 22.886:

Table X-1 Reference information for requirements

| Requirement # in this specification | CPR # in TR 22.886 |
| --- | --- |
| [R.5.4-001] | [CPR.E-002] |
| [R.5.4-002] | [CPR.E-001] |
| [R.5.4-003] | [CPR.E-003] |
| [R.5.4-004] | [CPR.E-004] |
| [R.5.4-005] | [CPR.E-005] |
| [R.5.4-006] | [CPR.E-006] |
| [R.5.4-007] | [CPR.E-007] |