3GPP TSG-SA WG1 Meeting #77	S1-171313

Jeju, KOREA, 13-17 FEB 2017	Revision of S1-171307/021

Title:	eV2X Normative Requirements for Remote Driving

Agenda Item:	8.1

Source:	LG Electronics

Contact:	SungDuck Chun (sungduck.chun@lge.com)

- Background

At SA1 #76bis, it was agreed that normative requirements for eV2X be incorporated into new TS.

During Study phase of eV2X, potential requirements of 25 use cases were consolidated into 5 categories.

- General Aspect

- Platooning

- Extended sensor

- Advanced driving

- Remote driving

This contribution proposes to adopt requirements related to Remote Driving into new TS.

Following is latest CPRs considering 22.886V15.0.0 and S1-170319, which were agreed at SA1 #76bis

[CPR.R-001] The 3GPP system shall support user experienced data rate up to 1 Mbps at DL and 20 Mbps at UL for UE supporting V2X application between V2X application server and UE for an absolute speed of up to 250 km/h. [CPR.R-002] The 3GPP system shall support ultra-high UL and DL reliability [99.999 or higher] % for UE supporting safety-related V2X application. [CPR.R-003] The 3GPP system shall support end-to-end latency 5 ms between V2X application server and UE supporting safety-related V2X application for an absolute speed of up to 250 km/h. Table 7.2.5-1 Performance requirements for remote driving Communication scenario; Payload (Bytes); Max latency (ms); Reliabi-lity (%); Data rate (Mbps); Communication range (meters). Section #; Description; CPR #. 5.21; Between a UE supporting V2X application & V2X Application Server. Driver Control; [CPR.R-004]; [20]; [99.999]; UL: 25 DL: 1.

- Discussion

Overall Comments:

- Because this is contents for TS, the direct link between use case and requirement cannot be pursued.

- The description of each requiremen should be general to avoid the impression that the KPIs is for specifics cenario.

- Avoid using use case name in the description as much as possible.

- ‘fully autonomated driving’ and ‘driver control, limited automated driving’ is not clearly defined.

- According to NHTSA/SAE, there are 6 level of automation for driving. Level 0, 1 and 2 is for ‘Human Driver” and level 3, 4 and 5 is for “automated driving system”.

- Use ‘Human Driver’ and “Automated Driving System’ to describe scenario.

- However, background information for each requirement will benefit readers for both in 3GPP and outside of 3GPP.

- Annex can be used to include iformation that allows tracing back to the use case.

- (Payload size * TX rate) = Data rate. If this value does not match, there will be a confusion.

- Either fill “Payload and Tx Rate” or “Data rate”. Not both.

CPR specific comments

Following tables summarized discussion/proposed changes for each CPR.

| CPR # for TR 22. 886 | Discussion (empty cell means no change) | Req # for new TS | Suggested changes for new TS (empty cell means CPR is mirror into Req without any change) |
| --- | --- | --- | --- |
| [CPR.R-001] The 3GPP system shall support user experienced data rate up to 1 Mbps at DL and 20 Mbps at UL for UE supporting V2X application between V2X application server and UE for an absolute speed of up to 250 km/h. | [CPR.R-001] [CPR.R-002] [CPR.R-003] are from same use case and can be expressed in a similar fashion as CPR.R-004. | [R.5.5-001] [R.5.5-002] | Merge [CPR.R-002] [CPR.R-002] Keep the requirement for 250km/h |
| [CPR.R-002] The 3GPP system shall support ultra-high UL and DL reliability [99.999 or higher] % for UE supporting safety-related V2X application. | Merge this requirement into [CPR.R-001] | [R.5.5-002] |  |
| [CPR.R-003] The 3GPP system shall support end-to-end latency 5 ms between V2X application server and UE supporting safety-related V2X application for an absolute speed of up to 250 km/h. | Merge this requirement into [CPR.R-001] | [R.5.5-002] | During eV2X session, it was agreed to merge CPR.R-001, 002, 003 with CPR.R-004. |
|  | Because this is V2N scenario within network coverage, communication range between vehicles is not applicable. | [R.5.5-002] |  |

- Proposal

It is proposed to agree on the text proposal below.

------------------------------------------------------------------------------------------------

------------------------------ START of first text Proposal----------------------------

------------------------------------------------------------------------------------------------

5.5	Requirements to support Remote Driving

[R.5.5-001]	The 3GPP system shall support message exchange between a UE supporting V2X application and V2X application server for an absolute speed of up to 250 km/h.

Table 5.5-1 Performance requirements for remote driving

| Communication scenario |  | Max end-to-end latency (ms) | Reliability (%) | Data rate (Mbps) |
| --- | --- | --- | --- | --- |
| Description | Requirement # |  |  |  |
| Information exchange between a UE supporting V2X application and a V2X Application Server. | [R.5.5-002] | 5 | [99.999] | UL: 25 DL: 1 |

------------------------------------------------------------------------------------------------

------------------------------ START of 2nd text Proposal----------------------------

------------------------------------------------------------------------------------------------

Annex <X> (informative):
<Background information on service Requirements>

For each requirement in this specification, Table X-1 indicates reference to the consolidated potential requirements in TR 22.886:

Table X-1 Reference information for requirements

| Requirement # in this specification | CPR # in TR 22.886 |
| --- | --- |
| [R.5.5-001] | [CPR.R-001] |
| [R.5.5-002] | [CPR.R-001] [CPR.R-002] [CPR.R-003] [CPR.R-004] |