3GPP TSG-SA WG1 Meeting #77	S1-171335

Jeju Island, South Korea, 13-17 February 2017	(revision of S1-17xxxx)

Title:	Deleting ignoring related requirements in TR 22.889

Agenda Item:	8.2

Source:	Huawei

Contact:	Carry Liu (Liujianning.liu@huawei.com)

Abstract: This document is propose to delete all the ignoring related requirements in TR 22.889, as they are not in scope of 3GPP.

Propose

It is proposed to update all the deleting as following in TR 22.889

************************************Start of 1st Change********************************************

7	Performance communication applications related use cases

7.1	Use cases related to transmission of real time video

Editors Note: 
UIC FWG to check applicability of placement of use case to section 7.3
Latency requirement missing to specify real-time treatment by the 3GPP transport system.
Clarification on multiple access towards the train to stream the video.

7.1.1	Introduction

In this subclause, the use cases related to the function of transmitting video in real time are defined.

-	An FRMCS user requesting another FRMCS user to transmit real time video

-	An FRMCS user receiving a request to transmit real time video

-	An FRMCS user accepting a request to transmit real time video

-	An FRMCS user rejecting a request to transmit real time video

************************************End of 1st Change********************************************

*****************************Start of 2nd Change********************************************

*****************************End of 2nd Change********************************************

*****************************Start of 3rd Change********************************************

7.4	Use case：Data transmission in real time

7.4.1	Description

This use case allows the authorized FRMCS users to exchange data in real time from each other, for example, health and status of train are transferred between intelligent on-train systems and train maintenance.

7.4.2	Pre-conditions

The user A is authorized FRMCS user (e.g., administrator who has authorization to the data).

User B is an equipment for monitoring the train.

User C is a normal FRMCS user (e.g., user without authorization to the data)

7.4.3	Service flows

User A requests User B to transmit the data (e.g., health, status of train, etc.). in real time.

User B receives the request from User A, and accepts the request.

User B starts to transmit the requesteddata in real time to User A.

User C requests User B to transmit the data (e.g., health, status of train, etc.) in real time.

User B receives the request from C, and rejects the request (e.g., without authorization to the data).

User C receives a notification that the request was rejected by User B.

7.4.4	Post-conditions

The data of User B is sent to User A in real time.

The data of User B is not sent to User C in real time.

7.4.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-7.4.5-001] | The FRMCS system shall enable an authorized FRMCS-user to request another FRMCS user to establish real time data transmission. | A |  | [To be assessed] |
| [R-7.4.5-002] | The FRMCS system shall provide a list of identities for the requesting authorized FRMCS user to select from. The list may contain functional identities, FRMCS user identities, or FRMCS equipment identities | A |  | [To be assessed] |
| [R-7.4.5-003] | The real time data request shall either be based on the functional identity, FRMCS user identity, or FRMCS equipment identity. | A |  | [To be assessed] |
| [R-7.4.5-004] | The FRMCS user shall be able to accept a real time data transmission request. | A |  | [To be assessed] |
| [R-7.4.5-005] | The FRMCS user shall be able to reject a data in real time transmission request. | A |  | [To be assessed] |
| [R-7.4.5-007] | The requesting FRMCS user shall be notified if the request is rejected. | A |  | [To be assessed] |

Editor’s Note: the term of “real time” is to be clarified by UIC so as to align with the understanding of railway industry.

*****************************End of 3rd Change********************************************

************************************Start of 4th Change********************************************

9.5	FRMCS-user communication handling related use cases

Editor's Note: Consideration to be spend on floor control use vs conference call mode of operation

9.5.1	Introduction

In this chapter the use cases related to the function of inviting a FRMCS-user to a communication are defined.

- Invite-a-FRMCS-user to a FRMCS communication

- FRMCS-user receiving an invitation to a FRMCS communication

- FRMCS-user accepting an invitation to a FRMCS communication

- FRMCS-user rejecting an invitation to a FRMCS communication

************************************End of 4th Change********************************************

************************************Start of 5th Change********************************************

************************************End of 5th Change********************************************