3GPP TSG-SA WG1 Meeting #78	S1-172040

# Porto, Portugal, 8-12 May 2017														(revision of S1-17xxxx)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.186 | CR | 0007 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial modifications for TS 22.186 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2X |  |  |  |  |  |  | Date: |  | 2017-04-28 |
| Category: | D |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Editorial changes are needed to make the terminologies consistent in the specification: - Wording in Overview description of Platoon is not in line with terminology used in Platoon section - TS 22.186 curently uses 3 similar terms for the same idea: “UE supporting V2X application”, “UE supporting V2X applications”, and “UE supporting a V2X application”, but a single common term should be used instead. - In TS 22.186 Annex A mapping table, the CPRs [CPR.P-008] is in fact associated to [R.5.2-006] and not to [R.5.2-005] (based on the discussion during the consolidation of requirements (in agreed S1-171310)). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a “s” into “Vehicle Platooning” “Vehicles Platooning” to align with Platoon section title - Change “Road Side Units” to “RSUs” as RSU has a definition - Change “UE supporting a V2X application” and “UE supporting V2X application” into “UE supporting V2X application” in line with V2XLTE TS terminololy. - link [CPR.P-008] reference with the [R.5.2-006] in the Annex<A> mapping table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reader will be confused if the TS contains different terminologies at different places. The reference to the consolidated potential requirements from theTR 22.886 in the TS will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 5.1, 5.2, 5.3, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***************************** Start of 1st Change  ***************************************

# Overview

## 4.1	Categories of requirements to support eV2X scenarios

Different V2X scenarios require the transport of V2X messages with different performance requirements for the 3GPP system. This Technical Specification specifies service requirements to enhance 3GPP support for V2X scenarios in the following five areas:

-	General Aspects: interworking, communication-related requirements valid for all V2X scenarios

-	Vehicles Platooning

-	Advanced Driving

-	Extended Sensors

-	Remote Driving

Vehicles Platooning enables the vehicles to dynamically form a group travelling together. All the vehicles in the platoon receive periodic data from the leading vehicle, in order to carry on platoon operations. This information allows the distance between vehicles to become extremely small, i.e., the gap distance translated to time can be very low (sub second). Platooning applications may allow the vehicles following to be autonomously driven.

Advanced Driving enables semi-automated or fully-automated driving. Longer inter-vehicle distance is assumed. Each vehicle and/or RSU shares data obtained from its local sensors with vehicles in proximity, thus allowing vehicles to coordinate their trajectories or maneuvers. In addition, each vehicle shares its driving intention with vehicles in proximity. The benefits of this use case group are safer traveling, collision avoidance, and improved traffic efficiency.

Extended Sensors enables the exchange of raw or processed data gathered through local sensors or live video data among vehicles, RSUs, devices of pedestrians and V2X application servers. The vehicles can enhance the perception of their environment beyond what their own sensors can detect and have a more holistic view of the local situation.

Remote Driving enables a remote driver or a V2X application to operate a remote vehicle for those passengers who cannot drive themselves or a remote vehicle located in dangerous environments. For a case where variation is limited and routes are predictable, such as public transportation, driving based on cloud computing can be used. In addition, access to cloud-based back-end service platform can be considered for this use case group.

***************************** Next Change  ***************************************

# Requirements

## 5.1	General Requirements

[R.5.1-001]	The 3GPP system shall be able to control the communication range for a message based on the characteristic of the messages transmitted by a UE supporting V2X application.

[R.5.1-002]	The 3GPP system shall be able to optimize the communication between UEs supporting V2X application belonging to the same group and in proximity.

[R.5.1-003]	The 3GPP system shall be able to support the message transfer for group management operations as requested by the application layer.

NOTE 1:	The determination of group membership may be done at the upper layers and/or lower layers (application and/or Layer 2).

[R.5.1-004]	The 3GPP system shall be able to support message transfer among a group of UEs supporting V2X application.

[R.5.1-005]	The 3GPP system shall be able to support message transfer between two UEs belonging to the same group of UEs supporting V2X application.

[R.5.1-006]	The 3GPP system shall be able to support confidentiality and integrity of message transfer among a group of UEs supporting V2X application.

[R.5.1-007]	The 3GPP system shall support relative lateral position accuracy of 0.1 m between UEs supporting V2X application.

[R.5.1-008]	The 3GPP system shall support high connection density for congested traffic.

NOTE 2:	Example estimate is for worst case US Freeway scenario that does not include arterial roads (i.e. onramps): 5 lanes in each direction or 10 lanes total per highway, for up to 3 highways intersecting = 3,100 to 4,300 cars per square kilometer

[R.5.1-009]	The 3GPP system shall support efficient coordination of radio resources used for transport of messages to maximize the utilization of the available spectrum and to ensure the required reliability.

[R.5.1-010]	The 3GPP system shall be able to control the UL and DL reliability of transport of V2X communications, depending on the requirement of V2X application

[R.5.1-011]	Impact to E-UTRA(N) by UE supporting only NR based V2X communication shall be minimized.

[R.5.1-012]	Impact to NR by UE supporting only E-UTRA based V2X communication shall be minimized.

[R.5.1-013]	The 3GPP system shall be able to support message transfer between UEs or between a UE and a UE-type RSU, regardless of whether or not they are subscribers of the same PLMN supporting V2X communications. In case they are subscribers to different PLMNs, there shall be no service degradation of the message transfer.

[R.5.1-014]	The 3GPP system shall enable discovery and communication between UEs supporting the same V2X application.

[R.5.1-015]	The 3GPP system shall be able to support the operators to select which 3GPP RAT to use for a V2X application.

Editor’s note:	The meaning of V2X application needs further clarification

[R.5.1-016]	The 3GPP system shall enable a UE supporting V2X application to obtain network access via another UE supporting V2X application.

[R.5.1-017]	The 3GPP system shall enable a UE supporting V2X application to discover another UE supporting V2X application that can offer access to the network.

[R.5.1-018]	The 3GPP system shall support switching between direct 3GPP connection and indirect 3GPP connection via a UE supporting V2X application, for a UE supporting V2X application.

[R.5.1-019]	The 3GPP system should be able to support confidentiality and integrity of message transfer between a UE supporting V2X application and network, when the UE is using an indirect 3GPP connection.

[R.5.1-020]	The 3GPP system shall allow UEs supporting V2X application to use NR for direct communication when the UEs are not served by a RAN using NR.

[R.5.1-021]	The 3GPP system shall allow UEs supporting V2X application to use E-UTRA for direct communication when the UEs are not served by a RAN using E-UTRA.

[R.5.1-022]	An RSU shall be able to communicate with up to 200 UEs supporting V2X application.

NOTE 3:	This requirement specifies the number of UEs that RSU should be capable of handling. The requirement is not applied to the number of UE in a platoon.

NOTE 4:	This requirement does not applied to UE type RSU.

[R.5.1-023]	The 3GPP system shall be able to support confidentiality and integrity of message transfer between a UE supporting V2X application and a V2X application server.

## 5.2	Requirements to support Vehicles Platooning

[R.5.2-001] The 3GPP system shall be able to support up to 5 UEs for a group of UEs supporting V2X application.

[R.5.2-002] For Vehicle Platooning, the 3GPP system shall be able to support reliable V2V communications between a specific UE supporting V2X application and up to 19 other UEs supporting V2X application.

NOTE 1: 	For group of heavy goods vehicle platooning, the number of UEs in a platoon can be smaller, due to communication range, the length of truck, inter-truck distance, etc.

[R.5.2-003] The 3GPP system shall support relative longitudinal position accuracy of less than 0.5 m for UEs supporting V2X application for platooning in proximity.

Table 5.2-1 Performance requirements for Vehicles Platooning

| Communication scenario description |  | Req # | Payload (Bytes) | Tx rate (Message/ Sec) | Max end-to-end latency (ms) | Reliability (%) (NOTE 4) | Data rate (Mbps) | Min required communication range (meters) (NOTE 6) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Degree |  |  |  |  |  |  |  |
| Cooperative driving for vehicle platooning Information exchange between a group of UEs supporting V2X application. | Lowest degree of automation | [R.5.2-004] | 300-400 (NOTE 1) | 30 | 25 | 90 |  |  |
|  | Low degree of automation | [R.5.2-005] | 6500 (NOTE 2) | 50 | 20 |  |  | 350m |
|  | Highest degree of automation | [R.5.2-006] | 50-1200 (NOTE 3) | 30 | 10 | 99.99 |  | 80 |
|  | High degree of automation | [R.5.2-007] |  |  | 20 |  | [65] (NOTE 2) | 180m |
| Reporting needed for platooning between UEs supporting V2X application and between a UE supporting V2X application and RSU. | N/A | [R.5.2-008] | 50-1200 | 2 | 500 |  |  |  |
| Information sharing for platooning between UE supporting V2X application and RSU. | Lower degree of automation | [R.5.2-009] | 6000 (NOTE 2) | 50 | 20 |  |  | 350m |
|  | Higher degree of automation | [R.5.2-0010] |  |  | 20 |  | [50] (NOTE 2) | 180m |
| NOTE 1: This value is applicable for both triggered and periodic transmission of data packets. NOTE 2: The data that is considered in this V2X scenario includes both cooperative manoeuvres and cooperative perception data that could be exchanged using two separate messages within the same period of time (e.g., required latency 20ms). NOTE 3: This value does not including security related messages component. NOTE 4: Sufficient reliability should be provided even for cells having no value in this table NOTE 5: The naming used in column degree is specific to the scenario. NOTE 6: This is obtained considering UE speed of 130km/h |  |  |  |  |  |  |  |  |

## 5.3	Requirements to support Advanced Driving

Table 5.3-1 Performance requirements for advanced driving

| Communication scenario description |  | Req # | Payload (Bytes) | Tx rate (Message/Sec) | Max end-to-end latency (ms) | Reliability (%) (NOTE3) | Data rate (Mbps) | Min required Communication range (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Degree |  |  |  |  |  |  |  |
| Cooperative collision avoidance between UEs supporting V2X application. |  | [R.5.3-001] | 2000 |  | 10 | 99.99 | 10 |  |
| Information sharing for automated driving between UEs supporting V2X application. | Lower degree of automation | [R.5.3-002] | 6500 (NOTE 1) | 10 | 100 |  |  | [700] |
|  | Higher degree of automation | [R.5.3-003] |  |  | 100 |  | 53 (NOTE 1) | [360] |
| Information sharing for automated driving between the UE supporting V2X application and the RSU | Lower degree of automation | [R.5.3-004] | 6000 (NOTE 1) | 10 | 100 |  |  | [700] |
|  | Higher degree of automation | [R.5.3-005] |  |  | 100 |  | 50 (NOTE 1) | [360] |
| Emergency trajectory alignment between UEs supporting V2X application. |  | [R.5.3-006] | 2000 |  | 3 | 99.999 | 30 | 500 |
| Intersection safety information between RSU and UE supporting V2X application. |  | [R.5.3-007] | 450 | 50 |  |  | DL: 0.5 UL: 50 (NOTE 2) |  |
| Cooperative lane change between UEs supporting V2X application. | Lower degree of automation | [R.5.3-008] | 300-400 |  | 25 | 90 |  |  |
|  | Higher degree of automation | [R.5.3-009] | 12000 |  | 10 | 99.99 |  |  |
| Video sharing between a UE supporting V2X application and a V2X application server. |  | [R.5.3-010] |  |  |  |  | UL: 10 |  |
| NOTE 1: This includes both cooperative manoeuvers and cooperative perception data that could be exchanged using two separate messages within the same period of time (e.g., required latency 100ms). NOTE 2: This value is referring to a maximum number of 200 UEs. The data rate per UE is DL: 250Kbps and UL: 2.5Kbps. NOTE 3: Sufficient reliability should be provided even for cells having no values in this table |  |  |  |  |  |  |  |  |

Editor’s note:	The KPI values of minimum required communication range in requirement [R.5.3-002] [R.5.3-003] [R.5.3-004] [R.5.3-005] need update based on further study.

***************************** Next Change  ***************************************

# Annex <A> (informative):
Background information on service Requirements

For each requirement in this specification, Table A-1 indicates reference to the consolidated potential requirements in TR 22.886:

Table A-1 Reference information for requirements

| Requirement # in this specification | CPR # in TR 22.886 |
| --- | --- |
| [R.5.1-001] | [CPR.G-001] |
| [R.5.1-002] | [CPR.G-002] |
| [R.5.1-003] | [CPR.G-003] |
| [R.5.1-004] | [CPR.G-004] |
| [R.5.1-005] | [CPR.G-005] |
| [R.5.1-006] | [CPR.G-006] |
| [R.5.1-007] | [CPR.G-007] |
| [R.5.1-008] | [CPR.G-009] |
| [R.5.1-009] | [CPR.G-010] |
| [R.5.1-010] | [CPR.G-011] |
| [R.5.1-011] | [CPR.G-012] |
| [R.5.1-012] | [CPR.G-013] |
| [R.5.1-013] | [CPR.G-014] |
| [R.5.1-014] | [CPR.G-016] |
| [R.5.1-015] | [CPR.G-017] |
| [R.5.1-016] | [CPR.G-018] |
| [R.5.1-017] | [CPR.G-019] |
| [R.5.1-018] | [CPR.G-020] |
| [R.5.1-019] | [CPR.G-021] |
| [R.5.1-020] | [CPR.G-022] |
| [R.5.1-021] | [CPR.G-022] |
| [R.5.1-022] | [CPR.G-023] |
| [R.5.1-023] | [CPR.G-025] |
| [R.5.2-001] | [CPR.P-001] |
| [R.5.2-002] | [CPR.P-002] |
| [R.5.2-003] | [CPR.P-003] |
| [R.5.2-004] | [CPR.P-005] |
| [R.5.2-004] | [CPR.P-007] |
| [R.5.2-005] | [CPR.P-009] |
| [R.5.2-006] | [CPR.P-008] |
| [R.5.2-006] | [CPR.P-004] |
| [R.5.2-007] | [CPR.P-010] |
| [R.5.2-008] | [CPR.P-006] |
| [R.5.2-009] | [CPR.P-011] |
| [R.5.2-010] | [CPR.P-012] |
| [R.5.3-001] | [CPR.A-001] |
| [R.5.3-002] | [CPR.A-002] |
| [R.5.3-003] | [CPR.A-003] |
| [R.5.3-004] | [CPR.A-004] |
| [R.5.3-005] | [CPR.A-005] |
| [R.5.3-006] | [CPR.A-006] |
| [R.5.3-007] | [CPR.A-007] |
| [R.5.3-008] | [CPR.A-008] |
| [R.5.3-009] | [CPR.A-009] |
| [R.5.3-010] | [CPR.A-010] |
| [R.5.4-001] | [CPR.E-002] |
| [R.5.4-002] | [CPR.E-001] |
| [R.5.4-003] | [CPR.E-003] |
| [R.5.4-004] | [CPR.E-004] |
| [R.5.4-005] | [CPR.E-005] |
| [R.5.4-006] | [CPR.E-006] |
| [R.5.4-007] | [CPR.E-007] |
| [R.5.4-008] | [CPR.E-008] |
| [R.5.5-001] | [CPR.R-001] |
| [R.5.5-002] | [CPR.R-001] [CPR.R-002] [CPR.R-003] [CPR.R-004] |

***************************** End of Change  ***************************************