3GPP TSG-SA WG1 Meeting #78	S1-172302

# Porto, Purtugal, 8-12 May 2017										(revision of S1-172243, S1-172041)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.186 | CR | 0008 | rev | 2 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Consolidate communication range in 5.3 Advanced Driving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2X |  |  |  |  |  |  | Date: |  | 2017-04-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is still an Editor’s note in 5.3 Advanced Driving, indicating the need to double check the values for the communciation range columns which are still with brackets. For in Advanced Driving, we consider that vehicles have the same speed as the one considered for the Platoon case, i.e. 130 km/h. But, differently from the Platoon, it is possible that vehicles drive in opposite directions. The communication range is then different from the one of the Platoon: - For lower degree of automation: with communication range equal to 10 sec * relative speed at absolute speed around 130km/h for one vehicle, and then the relative speed between two vehicles around 260km per hour (i.e. 72m per second) , we obtain 10s * 72m/s = 720 m for communication range, this is in line with the double of platoon value. - For higher degree of automation: 5 sec * relative speed, we obtain 5s * 72m/s=360m for communication range. The current values with brackets are correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A Note is added for Advance Driving to clarify that the vehicule are at 130km/h (in line with existing Note for Platoon table) and to indicate that these vehicle can drive in opposite direction. - The brackets for communcation range of Advanced Driving and associated Editor’s Note are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TS specification will not be completed with remaining Editor’s note and values with brackets. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***************************** START OF CHANGE  ***************************************

5.3	Requirements to support Advanced Driving

Table 5.3-1 Performance requirements for advanced driving

| Communication scenario description |  | Req # | Payload (Bytes) | Tx rate (Message/Sec) | Max end-to-end latency (ms) | Reliability (%) (NOTE3) | Data rate (Mbps) | Min required Communication range (meters) (NOTE 4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Degree |  |  |  |  |  |  |  |
| Cooperative collision avoidance between UEs supporting V2X applications. |  | [R.5.3-001] | 2000 |  | 10 | 99.99 | 10 |  |
| Information sharing for automated driving between UEs supporting V2X application. | Lower degree of automation | [R.5.3-002] | 6500 (NOTE 1) | 10 | 100 |  |  | 700 |
|  | Higher degree of automation | [R.5.3-003] |  |  | 100 |  | 53 (NOTE 1) | 360 |
| Information sharing for automated driving between the UE supporting V2X application and the RSU | Lower degree of automation | [R.5.3-004] | 6000 (NOTE 1) | 10 | 100 |  |  | 700 |
|  | Higher degree of automation | [R.5.3-005] |  |  | 100 |  | 50 (NOTE 1) | 360 |
| Emergency trajectory alignment between UEs supporting V2X application. |  | [R.5.3-006] | 2000 |  | 3 | 99.999 | 30 | 500 |
| Intersection safety information between RSU and UE supporting V2X application. |  | [R.5.3-007] | 450 | 50 |  |  | DL: 0.5 UL: 50 (NOTE 2) |  |
| Cooperative lane change between UEs supporting V2X applications. | Lower degree of automation | [R.5.3-008] | 300-400 |  | 25 | 90 |  |  |
|  | Higher degree of automation | [R.5.3-009] | 12000 |  | 10 | 99.99 |  |  |
| Video sharing between a UE supporting V2X application and a V2X application server. |  | [R.5.3-010] |  |  |  |  | UL: 10 |  |
| NOTE 1: This includes both cooperative manoeuvers and cooperative perception data that could be exchanged using two separate messages within the same period of time (e.g., required latency 100ms). NOTE 2: This value is referring to a maximum number of 200 UEs. The data rate per UE is DL: 250Kbps and UL: 2.5Kbps. NOTE 3: Sufficient reliability should be provided even for cells having no values in this table NOTE 4: This is obtained considering UE speed of 130km/h. vehicles are possible to drive in different derections. |  |  |  |  |  |  |  |  |

***************************** END OF CHANGE  ***************************************