3GPP TSG-SA WG1 Meeting #78	S1-172330

# Porto, Portugal, 08-12 May 2017														(revision of S1-172072)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.179 | CR | 0052 | rev | 1 | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multi-talker management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, UIC, Ericsson, KCC, ETRI, HanSung University. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | Monastery |  |  |  |  |  |  | Date: |  | 2017-04-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To update 22.179 to include railway requiremetns for MONASTERY |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new requirements to supprot multiple users to talk simultaneously |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not support FRMCS requirements in rel-15 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*****************************start of the change*********

6.2.3.7	Multi- talker management

6.2.3.7.1	Overview

The multi-talker management applies to specially designated MCPTT Groups and results in Floor control for that group allowing any participant talking simultaneously within the MCPTT Group. Multi-talker management is used by railway communication e.g. during shunting operation.

Other than the difference in floor control logic, the MCPTT Groups configured to support multi-talker management behave in the same way as other MCPTT Groups with group management, affiliation, selection of a group, requesting the floor, the notifications received related to Floor control etc working in the same way.

6.2.3.7.2		Requirements

[R-6.2.3.7.2-001] The MCPTT Group shall be configurable to allow multi-talker management.

NOTE 1:	MCPTT Groups configured for multi-talker management behave in the same way as MCPTT Groups not  configured for multi-talker management in all other respects other than the Floor control logic described in this sub-clause.

[R-6.2.3.7.2-002] The MCPTT Service shall provide a mechanism for multiple MCPTT Users to talk simultaneously in a group call.

[R-6.2.3.7.2-003] The initial maximum number of simultaneous talkers for multi-talker management shall be provided at call establishment.

[R-6.2.3.7.2-004] The the MCPTT Service shall be able to change the maximum number of simultaneous talkers at any time during a group call.

[R-6.2.3.7.2-005] If the number of MCPTT Users requesting the permission to talk exceeds the maximum number of simultaneous talkers, the MCPTT Service shall be able to revoke the permission of some MCPTT User(s) or reject the request based on certain criteria, e.g., priorities etc.

[R-6.2.3.7.2-006] If the number of MCPTT Users requesting the permission to talk exceeds the maximum number of simultaneous talkers, the multi-talker management shall be able to provide an authorized MCPTT User the list of MCPTT Users granted with the permisssion to talk and the list of pending requests to decide whose permission will be revoked or whose request will be rejected.

6.2.3.7.3	Requesting permission to transmit

[R-6.2.3.7.3-001] Any authorized Participant shall be able to request to transmit to an MCPTT Group configured to support multi-talker management.

[R-6.2.3.7.3-002] At call setup the MCPTT Service shall be able to provide a notification, for example audio and/or visual, to the MCPTT Group Member attempting to transmit that there are no other Group Members who have affiliated to the MCPTT Group configured to support multi-talker management.

[R-6.2.3.7.3-003] Following an MCPTT Request for permission to transmit on the Selected MCPTT Group configured to support multi-talker management, the Affiliated MCPTT Group Member that made and was granted the request shall be given an indication of being allowed to transmit.

6.2.3.7.4	Terminating permission to transmit

[R-6.2.3.7.4-001] The MCPTT Service shall be able to enable an authorized MCPTT User to terminate the permission to transmit of a transmitting Participant at any time on a group configured for multi-talker management.

[R-6.2.3.7.4-002] A transmitting Participant shall be able to indicate to the MCPTT Service that the Participant no longer wants to transmit on a group configured for multi-talker management.

6.2.3.7.5	Transmit time limit

[R-6.2.3.7.5-001] The MCPTT Service shall enable an MCPTT Administrator to configure the limit for the length of time that a Participant transmits from a single request to transmit on a group configured for multi-talker management.

[R-6.2.3.7.5-002] The Floor control functionality for a group configured for multi-talker management shall have a configurable limit for the length of time that a Participant transmits from a single request to transmit.

[R-6.2.3.7.5-003] The Floor control functionality for a group configured for multi-talker management shall provide an indication to the transmitting Participant that the Participant is within a configurable amount of time before his transmit time limit is reached.

[R-6.2.3.7.5-004] The Floor control functionality for a group configured for multi-talker management shall provide an indication to the transmitting Participant that the Participant's transmit time limit has been reached.

[R-6.2.3.7.5-005] The Floor control functionality for a group configured for multi-talker management shall remove the permission to transmit from the transmitting Participant when the Participant's transmit time limit has been reached.

*****************************The End of change*********