3GPP TSG-SA WG1 Meeting #78	S1-172402

# Porto, Portugal, 08-12 May 2017		(revision of S1-172334, S1-172332, S1-172330, S1-172072)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.179 | CR | 0052 | rev | 4 | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multi-talker control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, UIC, Ericsson, KCC, ETRI, Hansung University, KRRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | Monastery |  |  |  |  |  |  | Date: |  | 2017-05-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To update 22.179 to include railway requirements for MONASTERY |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new requirements to support multiple users to talk simultaneously |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of FRMCS requirements in Rel-15 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 6.2.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## *****************************start of the change*********

### 6.2.3.7	MCPTT Groups designated for multi-talker control

##### 6.2.3.7.1	Overview

The multi-talker control applies to designated MCPTT Groups and results in allowing several Participants talking simultaneously within the MCPTT Group. For example, Multi-talker control is used by railway communication e.g. during shunting operation.

When a MCPTT Group is designated for multi-talker control, the requirements listed below apply.

Except for Floor Control as specified in clauses 6.2.3.1, 6.2.3.2, 6.2.3.3, 6.2.3.4, 6.2.3.5, 6.2.3.6 in all other aspects MCPTT Groups designated to support multi-talker control behave in the same way as other MCPTT Groups with respect to group management, affiliation, selection of a group, and notifications.

#### 6.2.3.7.2	General aspects

[R-6.2.3.7.2-001] The MCPTT Service shall be configurable to allow MCPTT Groups to be designated for multi-talker control.

[R-6.2.3.7.2-002] The MCPTT Service shall provide a mechanism for multiple MCPTT Users to talk simultaneously in a MCPTT Groups designated for multi-talker control.

[R-6.2.3.7.2-003] The MCPTT Service shall determine at a point in time which Participant(s) are allowed to transmit to other Participant(s).

[R-6.2.3.7.2-004] The MCPTT Service shall support Participant(s) to receive audio from all other  Participant(s) that are transmitting in the group.

[R-6.2.3.7.2-005] The MCPTT Service shall provide a mechanism for the MCPTT Administrator to configure the number of simultaneous talkers in a MCPTT Group designated for multi-talker control.

[R-6.2.3.7.2-006] The MCPTT Service shall allow an authorized MCPTT User to change the maximum number of simultaneous talkers at any time during a group call.

#### 6.2.3.7.3	Requesting permission to transmit

[R-6.2.3.7.3-001] The MCPTT Service shall enable authorized Participants to request to transmit to an MCPTT Group designated for multi-talker control.

[R-6.2.3.7.3-002] At call setup the MCPTT Service shall provide a notification, for example audio and/or visual, to the MCPTT Group Member attempting to transmit that there are no other Group Members who have affiliated to the MCPTT Group.

[R-6.2.3.7.3-003] The MCPTT Service shall determine the transmitting Participant(s) when there are simultaneous requests for permission to transmit within the same call.

[R-6.2.3.7.3-004] Following an MCPTT Request for permission to transmit on the Selected MCPTT Group designated for multi-talker control and the number of MCPTT Users requesting the permission to talk is not exceeding the maximum number of simultaneous talkers the MCPTT Service shall provide the Affiliated MCPTT Group Member(s) that made and were granted the request an indication of being allowed to transmit.

##### 6.2.3.7.4	Override

##### 6.2.3.7.4.1	General aspects

[R-6.2.3.7.4.1-001]  If the number of MCPTT Users requesting the permission to talk exceeds the maximum number of simultaneous talkers in a MCPTT Group designated to multi-talker control, the MCPTT Service shall apply the override mechanism.

[R-6.2.3.7.4.1-002]  The MCPTT Service shall enable MCPTT Administrators to create a priority hierarchy for determining what Participants, Participant types, and urgent transmission types shall be granted a request to override an active MCPTT transmission.

[R-6.2.3.7.4.1-003]  The MCPTT Service shall enable an MCPTT Administrator to configure which MCPTT Group transmission a Participant(s) receives, overriding and/or overridden for cases where an authorized Participant overrides an MCPTT transmission.

[R-6.2.3.7.4.1-004] The MCPTT Service shall enable the MCPTT Administrator to configure the MCPTT Group to allow only the overriding Participant to transmit or to allow both the overriding and overridden Participant to transmit.

[R-6.2.3.7.4.1-005]  The priority hierarchy used for granting a request to override an active MCPTT transmission shall contain at least four (4) levels.

[R-6.2.3.7.4.1-006]  The transmitting Participant shall be determined by the relative priorities of the Participants and Call type based on priority (e.g MCPTT Emergency).

[R-6.2.3.7.4.1-007]  The MCPTT Service shall provide a mechanism for Participants, to override an active MCPTT transmission of a transmitting Participant when the priority level of the overriding Participant or Call type based on priority (e.g MCPTT Emergency) are ranked higher than the priority level of the transmitting Participant or Call type based on priority.

[R-6.2.3.7.4.1-008]  If an authorized Participant overrides an MCPTT transmission, the MCPTT Service shall provide a means of notifying the overridden Participant(s) that the transmission has been overridden.

[R-6.2.3.7.4.2-009]  The MCPTT Service shall revoke the transmit permission of the overridden transmitting Participant.

#### 6.2.3.7.5	Terminating permission to transmit

[R-6.2.3.7.5-001] The MCPTT Service shall enable authorized MCPTT Users to terminate the permission to transmit of a transmitting Participant at any time.

[R-6.2.3.7.5-002] The MCPTT Service shall enable a transmitting Participant to indicate to the MCPTT Service that the Participant no longer wants to transmit.

NOTE:	In this case the audio of the transmitting participant immediately stops being transmitted to the receiving Participant(s).

[R-6.2.3.7.5-003] The MCPTT Service shall be able to provide an indication to receiving Participants that the transmitting Participant has finished transmitting.

#### 6.2.3.7.6	Transmit time limit

[R-6.2.3.7.6-001] The MCPTT Service shall enable an MCPTT Administrator to configure the limit for the length of time that a Participant transmits from a single request to transmit.

[R-6.2.3.7.6-002] The MCPTT Service shall provide a means to support for a configurable limit of the length of time that a Participant transmits from a single request to transmit.

[R-6.2.3.7.6-003] The MCPTT Service shall provide an indication to the transmitting Participant that the Participant is within a configurable amount of time before his transmit time limit is reached.

[R-6.2.3.7.6-004] The MCPTT Service shall provide an indication to the transmitting Participant that the Participant's transmit time limit has been reached.

## [R-6.2.3.7.6-005] The MCPTT Service shall remove the permission to transmit from the transmitting Participant when the Participant's transmit time limit has been reached.

## *****************************The End of change*********