3GPP TSG-SA WG1 Meeting #79	S1-173363

# Guilin, CHINA，21-25 Aug，2017	                                                     (revision of S1-173086)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.280 | CR | 0043 | rev | 1 | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction requirement for Functional alias |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MONASTERY |  |  |  |  |  |  | Date: |  | 2017-08-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | [R-5.9a-008] is already defined how to deal with the case “one MCX User per functional alias”, But there is no requirement to define how to deal with the case “multiple MCX Users per functional alias ”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add a requirement, to enable the MCX Service to reject the MCX User’s request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The funcitonal alias features are not completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9a.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the Change  ***************

5.9a	Functional alias

### 5.9a.1 	Overview

A functional alias is a user selectable alias that is tied to the assignment or task of the user. A MCX User can activate one or multiple functional aliases at the same time. A functional alias can be used for communication and for addressing targets of an MCX communication as an alternative to the MCX User ID. The activation of the functional alias(es) will take place after the user has signed in to the MCX Service using the MCX User ID.

The same functional alias can be assigned for use to multiple users depending on MCX Service Administrator settings. A functional alias can be taken over by an authorized MCX User, depending on MCX Service Administrator settings. These two capabilities are mutually exclusive and can not be configured for the same functional alias.

A functional alias can be used to identify for example the driver(s) of a particular train, identified by train number and the role of the user on that train.

Each functional alias that is active on the MCX Service is unique for addressing purposes. For example, if there are two drivers of TRAIN29 (e.g. DRIVER1_TRAIN29, DRIVER2_TRAIN29), then each active functional alias should be uniquely addressable.

### 5.9a.2	Requirements

[R-5.9a-001] If the MCX Service System supports functional aliases, then the MCX Service shall provide a mechanism for the MCX User to activate one or more functional alias(es).

[R-5.9a-002] If the MCX Service System supports functional aliases, the MCX User shall be reachable by its functional alias(es).

[R-5.9a-003] If the MCX Service System supports functional aliases, then the MCX Service shall provide a mechanism for the MCX User to deactivate a functional alias.

[R-5.9a-004] If the MCX Service System supports functional aliases, then the MCX Service shall upon request provide the MCX User a list of functional aliases from which the user can select for activation.

NOTE: 	The list may contain functional aliases based on a certain context, like location of the MCX User, operational schedule, etc.

[R-5.9a-005] If an MCX Service System supports functional alias, the MCX Service shall provide a mechanism for an MCX Service Administrator to manage functional aliases, and for each functional alias indicate either if it can be simultaneously active for multiple MCX Users up to a per-alias configurable number, or if it is allowed to be taken over by an authorized MCX User, or none of these two options.

[R-5.9a-006] If a MCX User attempts to activate a functional alias that is already active for another MCX User,  the MCX Service shall inform the MCX User that the functional alias is already in use.

[R-5.9a-007] If a MCX User attempts to activate a functional alias that is already active for at least one other MCX User, and that functional alias is allowed to be simultaneously active for multiple MCX Users and the upper limit of number of simultaneous MCX Users is not reached, the MCX Service shall activate the functional alias for the MCX user and inform all other MCX User(s) with the same functional alias.

[R-5.9a-008] If an authorized MCX User attempts to activate a functional alias that is already used by another MCX User, and that functional alias is allowed to be taken over, and is not indicated for simultaneous activation to multiple MCX Users, the MCX Service shall offer the MCX User to take over the functional alias from the MCX User using the alias.

[R-5.9a-00x] If an authorized MCX User attempts to activate a functional alias that is already active for at least one other MCX User, and is allowed to be simultaneously active for multiple MCX Users and the upper limit of number of simultaneous MCX Users is reached, the MCX Service shall reject the MCX User’s request.

[R-5.9a-009] If an authorized MCX User takes over the functional alias that is already active for another MCX User, the MCX Service shall activate the functional alias to the MCX User and inform the previous MCX User that the alias has been deactivated.

[R-5.9a-010] If an MCX Service System supports functional alias, the MCX Service shall allow the MCX User to perform a registration of an unlisted functional alias. Once the newly listed functional alias is registered with the MCX Service, the MCX User may activate the new functional alias.

[R-5.9a-011] If an MCX Service System supports functional alias, then an authorized MCX User shall be able to interrogate the MCX Service System of the alias(es) active for a certain MCX User.

[R-5.9a-012] If an MCX Service System supports functional alias, the MCX Service shall provide a mechanism for an MCX Service Administrator to authorize a MCX User to take over a functional alias.

[R-5.9a-013] If an MCX Service System supports functional alias, the MCX Service shall provide a mechanism for an MCX Service Administrator to authorize a MCX User to interrogate the MCX Service System of the alias(es) active for a certain MCX User.

*************** End of the Change  ***************