3GPP TSG-SA WG1 Meeting #80	                                                      S1-174012

# Reno, US, Nov 27- 1 Dec 2017															(revision of S1-17)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.889 | CR | 0032 | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Railway Operation and Maintenance Communication related use cases for section 7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS 2 |  |  |  |  |  |  | Date: |  | 2017-11-15 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | New use cases related to voice and data communication for Performance Communication in section 7 to be added in TR22.889 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new use cases: - Trackside maintenance communication between FRMCS Users - Voice and data communication for crews on the trains and ground staffs - Tickets and passengers information communication between Train and Control center - Remote Software and configurations update for the devices on the train - Data communication for freight locomotives between FRMCS Equipments. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New railway related use cases are missing in FRMCS TR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 7.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*************** Start modification  ****************

## 7.x	Railway Operation and Maintenance Communication related use cases

### 7.x.1	Introduction

In this chapter the use cases related to Railway Operation and Maintenance communication are described. The following use cases are identified:

- Trackside maintenance communication between FRMCS Users

- Voice and data communication for crews on the trains and ground staffs

- Tickets and passengers information communication between Train and Control center

- Remote Software and configurations update for the devices on the train

- Data communication for freight locomotives between FRMCS Equipments.

### 7.x.2	Use case: Trackside maintenance communication between FRMCS Users

#### 7.x.2.1	Description

For train maintenance, the track side staffs and Dispatcher need voice, data and video communications to transmit maintenance orders, pictures and videos of the train equipments.

#### 7.x.2.2	Pre-conditions

User A, User B, User C and User D are authorized FRMCS Users.

User D is a dispatcher.

#### 7.x.2.3	Service flows

Dispatcher User D initiates a voice communication with trackside maintenance User A, User B and User C who explain what is the train problematic and sends data with videos of the damaged train’s equipment.

#### User D send messages to User A, User B and User C in order to distribute the train maintenance work.

#### 7.x.2.4	Post-conditions

User A, User B and User C receive the maintenance work tasks from dispatcher User D.

Dispatcher User D receives the latest progress info (e.g., picture, or video) from User A , B, and C.

#### 7.x.2.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
|  | The FRMCS system shall support voice, data and video communications between FRMCS users | T | covered |  |

### 7.x.3	Use case: Voice and data communication for crews on the trains

#### 7.x.3.1	Description

For e.g. train security, the driver and railways policemen (inside of the train) need voice, data and video communications, to transmit orders, pictures and videos.

The same scenario applies between users are in the same train station.

#### 7.x.3.2	Pre-conditions

User A, User B and User C are authorized FRMCS Users.

User A and B are railway policeman.

User B is the train conductor.

User B finds a suspicious person.

#### 7.x.3.3	Service flows

Train conductor User B initiates a data communication to send the suspicious person info to railway policeman User A.

User A initiates a voice communication to other railway policemen User C to ask to control that person.

#### 7.x.3.4	Post-conditions

All the other railway policemen can received the data from User A

#### 7.x.3.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
|  | The FRMCS system shall support voice and data communications between FRMCS users | T | covered |  |

### 7.x.5	Use case: data communication for exchanging Tickets and passengers information

#### 7.x.5.1	Description

The tickets management system will synchronize the train tickets information:  the train servers on the train receives the information including seats occupation state, seats occupation sections, passenger information, tickets lost information and Internet tickets information, and with these information, the train can provide better service, such as, selecting the appropriate seat for disabled people.

#### 7.x.5.2	Pre-conditions

Passenger A is a disabled people.  And his booked seat is located in the middle of carriage surrounded crowed passengers.

Train server B is an FRMCS User

#### 7.x.5.3	Service flows

The train server B receives the information of passenger A from tickets management system.

The train server B requests tickets management system to give a more appropriate seat for passenger A.

#### 7.x.5.4	Post-conditions

Passenger A has a much more convenient seat.

.7.x.5.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
|  | The FRMCS system shall support data communication between FRMCS users. | T | covered |  |

### 7.x.6	Use case: remote update software and configurations of the devices on the train

#### 7.x.6.1	Description

This use case introduce that devices on the train can remotely update the software and configurations

#### 7.x.6.2	Pre-conditions

Device A and Device B are FRMCS Equipments.

Server C is FRMCS server controlled by railway operator.

#### 7.x.6.3	Service flows

Device A sends request to Server C to update the configurations.

Server C sends latest version of software to Device A and Device B.

Device A and Device B receive the latest software.

#### 7.x.6.4	Post-conditions

Device A and Device B are running on the latest software.

Device A has new configuration.

#### 7.x.6.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
|  | The FRMCS system shall provide a mean to remotely update the configuration or software of FRMCS Equipments. | T |  | [Needs analysis] |

### 7.x.7	Use case: Data communication for freight locomotives

#### 7.x.7.1	Description

For the freight railway lines, the freight locomotive and freight management centre need to exchange data information, including the states of the freight locomotives, logistics information and large engineering equipment management information, etc.

#### 7.x.7.2	Pre-conditions

User A is a freight train, and is approaching to a station.

.Dispatcher B is authorized FRMCS User, or controller.

#### 7.x.7.3	Service flows

Train User A reports the basic train information to Dispatcher B or controller.

#### 7.x.7.4	Post-conditions

The controller or Dispatcher B know the latest state of this Train.

#### 7.x.7.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
|  | The FRMCS system shall support data communication between the FRMCS Users. | T | covered |  |

.

*************** End modification  ****************