3GPP TSG-SA WG1 Meeting #80	S1-174115

Reno, Nevada, USA, 27 November - 1 December 2017

Title:	CAV – smart grid | communication architecture

Agenda Item:	8.5

Source:	Siemens AG, Huawei

Contact:	Joachim W. Walewski (joachim.walewski@siemens.com)

Abstract: We introduce a description of the vertical "smart grid"

# Proposal

------------------------- START OF PROPOSED CHANGES ----------------------------

------------------------- Start of Change 1 ----------------------------

5.x Smart (electric power) grid

5.x.1 Description of this vertical’s communication architecture

The smart grid communication architecture, visualised in Fgure 5.X.1-1, is mapped to the power network it serves. The smart grid communication architecture covers multiple network domains,  where each domain relates to a specific grid voltage and geographic area. Every network domain instance clearly belongs to exactly one smart grid stakeholder.

Figure 5.X.1-1: Smart grid communication architecture [x1]. CC: Control centre; DER: distributed energy resource; DSO: distribution system operator, HV: High voltage; LV: low voltage; MV: medium voltage; NMS: network management system; TSO: transmission system operator

The following communication network domains can be distinguished:

- Backbone network: Communication network which connects the primary-substation LANs amongst each other and with regional control centers (often co-located) and central control centers.

- Primary-substation LAN: A primary substation LAN is quite complex and requires its own communication infrastructure that distinguishes between a process bus and a station bus. It is mainly based on a Gigabit Ethernet infrastructure.

- Backhaul network: Communication network which connects secondary-substation LANs with each other and with a control center. This network domain might also connect to the respective primary-substation LAN in case the DSO and TSO roles are linked.

- Secondary-substation LAN: Network inside the secondary substation (today this network is quite trivial and may consist of just one single Ethernet switch / IP router). The secondary-substation LAN is implemented in US-Style regions more in a distributed manner whereas in Europe the secondary-substation LAN is very often located in an encapsulated enclosure.

- Access network: Communication network which connects the customer premises or e.g. low-voltage sensors to a specific secondary substation.

- Customer premises LAN: In-building communication network whereas a customer is characterised by consumption and production of energy (prosumer) and the customer can be residential, public or industrial prosumer.

- Intra-DER Network: For medium-sized DERs like wind/solar parks a dedicated LAN is required for control, management and supervision purposes.

- Intra-Control-Center Network: LAN within a DSO’s or TSO’s control center.

- Public network: Fixed or mobile-network-operator owned communication network which offers different connectivity services either via dedicated services or via the open Internet.

Remarks:

- Each of the described communication network domains usually belongs to one Smart Grid stakeholder.

- Some stakeholders like aggregator or metering operator do not own private communication infrastructure. Usually they connect via a public network to their assets / customers. Thus, public networks are an important part of the overall Smart Grid architecture.

- Not all communication network domains are mandatory to exist: For example the secondary-substation LAN can alternatively connect to prosumers via a public network service. In this case no DSO-owned access network is required.

Mobile networks are used mainly for connecting secondary substation LANs and DERs to control centres and primary substations and for connecting devices like smart meters and sensors at customer premises to control and data centers. The requirements of this communication are relatively low regarding per device bit rate, latency and availability / reliability.

Mobile networks are not used in most cases for mission critical and real time communication (like tele-protection communication) due to the relatively high latency, missing reliability/ availability guarantees and Quality-of-Service capabilities of 2G, 3G and 4G networks. It is our expectation that URLLC-enabled 5G systems will change this, and that Smart Grid control functions might also be connected via mobile networks.

------------------------- End of Change 1 ----------------------------

------------------------- Start of Change 2 ----------------------------

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	IEC 61907, "Communication network dependability engineering", 2009.

[3]	3GPP TS 22.261: "Service requirements for the 5G system".

[4]	IEC 62290-1: "Railway applications - Urban guided transport management and command/control systems - Part 1: System principles and fundamental concepts".

[5]	TTA TTAK KO-06.0-369: "Functional Requirements for LTE-Based Communication System", Oct. 2014.

[6]	J. Kim, S. W. Choi, Y.-S. Song, and Y.-K. Kim, "Automatic train control over LTE: Design and performance evaluation", IEEE Comm. Mag., Oct. 2015.

[7]	IEEE 1474.1-2004: "IEEE Standard for Communications-Based Train Control (CBTC) Performance and Functional Requirements".

[8]	TS 22.261 (V15.0.0): " Technical Specification Group Services and System Aspects; Service requirements for the 5G system; Stage 1".

[9] 	Deterministic Networking, bas-usecase-detnet, IETF, October 2015

[10]	Richard C. Dorf and Robert H. Bishop, "Modern Control Systems", Pearson, 12th Edition , 2011.

[11]	Ernie Hayden, Michael Assante, and Tim Conway, "An Abbreviated History of Automation & Industrial Controls Systems and Cybersecurity", SANS Institute, https://ics.sans.org/media/An-Abbreviated-History-of-Automation-and-ICS-Cybersecurity.pdf {accessed: 2017-05-23}, 2014.

[12]	Gartner, "Gartner IT Glossary – Operational Technology", http://www.gartner.com/it-glossary/operational-technology-ot/ {accessed: 2017-08-01}.

[13]	ITU Study Group Q.22/1, "Report on Best Practices for a National Approach to Cybersecurity: A Management Framework for Organizing National Cybersecurity Efforts", 2008.

[14]	Gartner, "Gartner Says the Worlds of IT and Operational Technology Are Converging", http://www.gartner.com/newsroom/id/1590814 {accessed: 2017-08-01}, 2011.

[15]	ITU-T X.200, "REFERENCE MODEL OF OPEN SYSTEMS INTERCONNECTION FOR CCITT APPLICATIONS", 1988.

[16]	Lutz Rauchhaupt, Elke Hintze, André Gnad, "Über die Bewertung der Zuverlässigkeit industrieller Funklösungen - Teil 1 Theoretische Grundlagen", atp 49 Heft 3, S. 38, 2007.

[17]	BZKI, "Aspects of dependability assessment in RWCI" (German Version), June 2017.

[18]	BZKI, "Requirement Profiles in RWCI" (German Version), 2016 .

[19]	IEC 62657-1: "Industrial communication networks – Wireless communication networks – Part 1: Wireless communication requirements and spectrum considerations".

[20]	ISO 10795: "Space systems — Programme management and quality — Vocabulary", 2011.

[21]	20/53/EU: "Directive 2014/53/EU of the European Parliament and of the Council of 16 April 2014 on the harmonisation of the laws of the Member States relating to the making available on the market of radio equipment and repealing Directive 1999/5/EC", http://eur-lex.europa.eu/legal-content/EN/TXT/HTML/?uri=CELEX:32014L0053&from=DE, {accessed: 2017-08-03}.

[22]	ITU-T G.1000: "Communications quality of service: A framework and definitions", 2001.

[23]	NIST, "Framework for Cyber-Physical Systems", Release 1.0, https://s3.amazonaws.com/nist-sgcps/cpspwg/files/pwgglobal/CPS_PWG_Framework_for_Cyber_Physical_Systems_Release_1_0Final.pdf {accessed: 2017-08-04}, 2016.

[24]	IEC 62673: "Methodology for communication network dependability assessment and assurance".

[25]	IEC 61784-3: "Industrial communication networks – profiles – part 3: functional fieldbuses – general rules and profile definitions", 2016.

[26]	IEC 62439-1: "Industrial communication networks – High availiability automation networks  - Part 1: General concepts and calculation  methods".

[27]	H. Kagermann, W. Wahlster, and J. Helbig, "Recommendations for implementing the strategic initiative INDUSTRIE 4.0", Final report of the Industrie 4.0 working group, acatech – National Academy of Science and Engineering, Munich, April 2013

[28]	IEC 61158: "Industrial communication networks – fieldbus specification", 2014.

[29]	IEC 61784: "Industrial communication networks – profiles", 2014.

[30]	R. Zurawski, "Industrial communication technology handbook", second edition, CRC Press, September 2017.

[31]	IEC 61508: "Functional safety of electrical/electronic/programmable electronic safety-related systems", 2010.

[32]	IEC 62061: "Safety of machinery - Functional safety of safety-related electrical, electronic and programmable electronic control systems" (IEC 62061:2005 + A1:2012).

[33]	A. Manjeshwar and D.P. Agrawal, "TEEN: a routing protocol for enhanced efficiency in wireless sensor networks.", In Proceedings 15th International Parallel and Distributed Processing Symposium. IPDPS 2001. IEEE Comput. Soc. https://doi.org/10.1109/ipdps.2001.925197.

[34]	M. A. Mahmood, W. K. G. Seah, and I. Welch, I., "Reliability in wireless sensor networks: A survey and challenges ahead.", Computer Networks, 79, 166–187, 2015. https://doi.org/10.1016/j.comnet.2014.12.016

[35]	Bosch Connected Devices and Solutions GmbH, "Cross Domain Development Kit | XDK, Bosch XDK110 datasheet", April 2017. https://xdk.bosch-connectivity.com/hardware [Revised July 2017].

[36]	B. E. Keiser and E. Strange, "Pulse Code Modulation.", in Digital Telephony and Network Integration (pp. 19–34). Springer Netherlands. https://doi.org/10.1007/978-94-015-7177-7_3, 1985.

[37]	ISO/IED 13818-3: "Information technology -- Generic coding of moving pictures and associated audio information -- Part 3: Audio", 1998.

[38]	Zhao, G. (2011). Wireless sensor networks for industrial process monitoring and control: A survey. Network Protocols and Algorithms, 3(1), 46-63.

[39]	G. Ullrich, "Automated Guided Vehicle Systems - A Primer with Practical Applications.",  ISBN 978-3-662-44813-7, Springer Berlin Heidelberg, 2015.

[40]	IEC 62443: "Industrial communication networks - Network and system security", https://webstore.iec.ch/searchform&q=IEC%2062443.

[41]	IEC 62443-3-2: "Security for industrial automation and control systems - Part 3-2: Security risk assessment and system design", in progress.

[42]	IEC 62443-3-3: "Industrial communication networks - Network and system security - Part 3-3: System security requirements and security levels", 2013.

[43]	ZVEI, "Working ZVEI Whitepaper - Security Interessen für die 5G Standardisierung", (in German), https://www.zvei.org/fileadmin/user_upload/Themen/Cybersicherheit/5G/Working_ZVEI_Whitepaper_Security_Interessen_bei_5G_OEV.pdf  (accessed: 2017-07-25), 2017.

[44]	Avizienis, Algirdas, et al. "Basic concepts and taxonomy of dependable and secure computing." Dependable and Secure Computing, IEEE Transactions on 1.1, pp. 11-33, 2004.

[x1]	Feuchtinger, Ulrich, Frank, Reinhard, Riedl, Johannes, and Eger, Kolja, Smart Communications for Smart Grids, Siemens White Paper, 2012.

------------------------- End of Change 2 ----------------------------

------------------------- END OF PROPOSED CHANGES ----------------------------