3GPP TSG-SA WG1 Meeting #80 	S1-174143

Reno, Nevada, USA, 27 November - 1 December 2017	(revision of xx-17xxxx)

# Source:	China Telecom

# Title:	Study on Mutual Trust  of Different Authentications

# Document for:	Approval

Agenda Item:	xxx

Contact：           Rong Zhang  zhangr@gsta.com; Yuyong Jia  jiayuy@gsta.com

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

Title: Study on Mutual Trust Between Different Authentications

Acronym:  FS_MTBDA

Unique identifier:

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No |  |  | X |  |  |
| Don't know | X |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

3	Justification

In the 5G security authentication framework, there are still different authentication methods for Non-3GPP network access (EAP-AKA ') and 3GPP network access (EAP-AKA, 5G-AKA) .  With the development of ubiquitous convergence terminals with  multi-connection interfaces, a terminal will have multiple network access modes (such as FTTx, WiFi, 3/4/5G, etc.) and have the capability of simultaneously accessing the operator's network (For example: VoLTE handsets have both cellular and WiFi connections).  Currently, different networks need to be authenticated differently. If there is mutal trust between different authentications , it would save extra auth processes and promote users’ experience  when the terminal has passed one access authentication.

There are 2 basic classes and 1 senior class for mutual trust scenarios

(1)  mutual trust  among multi network access authentications

Use case 1.A     About 5G users 'application services across different carriers' networks.

- mutual trust  between a network access authentication and an application

- Use case 2.A

- based on the 2 above, a trust policy is added.

4	Objective

1) Solve the problem that a multi-connection terminal can access another network with the same authentication level or a lower authentication level without authentication after a certain network access authentication is passed.

2) Solve the problem that multiple connected terminals can access another network with the same authentication level or lower authentication level without authentication after passing the authentication of an application platform.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Series | Title | For info at TSG# | For approval at TSG# | Remarks |
| External TR | 22.9XX |  | TSG#80 |  |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |
| --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# |

6	Work item Rapporteur(s)

Rong Zhang; China Telecom; zhangr@gsta.com

7	Work item leadership

SA1

8	Aspects that involve other WGs

None

9	Supporting Individual Members

Supporting IM name

China Telecom