3GPP TSG-SA WG1 Meeting #80	S1-174338

Reno, Nevada, USA, 27 November - 1 December 2017	(revision of S1-174145)

Title:	Use Case: Low-latency audio streaming for live performance

Agenda Item:	8.5 - FS_CAV

Source:	Intel, Robert Bosch, Fraunhofer Heinrich-Hertz-Institut (HHI), IRT, EBU, BMWi,

Contact:	valerio.frascolla@intel.com

Abstract: This contribution describes the use case “Low-latency audio streaming for live performance” of the Programme Making and Special Events (PMSE) vertical. It contains the use case description and associated requirements for 5G systems.

### Acknowledgment

# The following entities contributed to this document:

### PMSE-xG Project (www.pmse-xg.de), co-funded by the German Ministry of Transportation and Digital Infrastructure.

# Discussion

The requirements from the PMSE vertical are more stringent than the ones commonly considered for the 5G use case Ultra-Reliable and Low Latency Communication (URLLC), i.e. end-to-end latencies below 1 ms and high reliability for isochronous packet transmission. In fact, even though the support of continuous streaming of packets is essential for any media application, the ability to have all devices tightly synchronized and the support of a reliable multicast connection are additional key requirements that PMSE applications need to fulfil for seamless operation.

Note: a high-level description of this use case (S1-172381) was presented at the 3GPP SA-1 #78 meeting in Porto.

# Proposal

------------------------- START OF PROPOSED CHANGES ----------------------------

------------------------- Start of Change 1 ----------------------------

### 5.X.2	Low-latency audio streaming for live performance

### 5.X.2.1	Description

The low latency audio streaming use case addresses key issues of wireless audio productions, focusing on low-latency and high-reliability aspects. Even if the number of active wireless audio links or data rates of the respective wireless audio streams may vary, the requirements regarding end-to-end latency, communication service availability and communication service reliability remain principally the same. Reliability is a strongly required feature for all kind of live audio streaming applications. In fact, due to their on-line transmission nature, such applications cannot accommodate the repetition of audio transmissions in case of error.

Latency must be much lower than the latency of other speech transmissions systems, like cellular services, because the audio source (e.g. a microphone) and audio sink (e.g. an in-ear monitor [IEM]) are co-located and the performing artist, or conference participant, can accept only a small time difference between the original and the processed audio signal.

Streaming applications with low latency require high synchronicity between all related devices. Therefore, a system clock, e. g. word clock, is provided by an external word clock generator. This generator can be a dedicated device or it can be integrated, e.g. into the audio mixing console. Timestamps are typically used to determine playback times of digital audio data. All wireless audio devices involved in the live performance production network must be enabled to capture and reproduce their audio samples at the exact same time, i.e. every audio sample has a precise and unique relation to the system time. Thereby, the system time and synchronization accuracy must be much higher than the audio sampling clock and respective clock jitters much smaller than the audio sampling period. In order to support today’s and future professional productions, a synchronicity of 1 µs (see Table 5.X.2-1) with respect to the system clock must be met at the application level of all involved devices (wireless microphones, in-ears monitors, audio mixing).

A typical live performance scenario is an artist on stage using a wireless microphone while hearing himself via the wireless IEM system. The artist may be wearing roller skaters. The audio signal coming from the microphone is streamed to a mixing console, where the different incoming audio streams are mixed.

After mixing, several audio streams can be distinguished, e.g. the Public Address (PA), the individual IEM, or recording mixes. Here, IEM mixes are transmitted wirelessly, whereas most of the other signals are streamed via wired connections.

Figure 5.X.2.1-1 shows a typical topology of current live performance use cases. The whole setup follows a link-based approach, meaning that every wireless audio link is based on one specific mobile terminal connected to one specific installed transmitter (IEM) or receiver (microphone). Similar setups can be found in live events such as TV shows, sports events, musicals, and press conferences.

Figure 5.X.2.1-1: Logical topology of the live performance use case

In Table 5.X.2.1-1, typical system characteristic parameters of the live performance use case such as end-to-end delay, system delay, data rates, reliability, and synchronicity are listed. Not all system parameters presented in the table are defined in this document; they are provided for guidance.

Table 5.X.2.1: System parameters of the live performance use case.

|  | Characteristic system parameter | Comments |
| --- | --- | --- |
| Application latency | < 4 ms | End-to-end maximum allowable latency of the use case in focus, which includes application and application interfacing, see Figure 5.X.2.1-3 |
| End-to-end latency | < 1 ms | Latency that is introduced per link of the wireless communication system, excluding application and application interfacing, see Figure 5.X.2.1-3 |
| User experienced data rate | 150 kb/s – 4,61 Mb/s | Different user data rates per audio link need to be supported for different audio demands |
| Control data rate | ≤ 50 kb/s | Data rate per control link |
| Communication service availability | 99,9999% |  |
| Reliability | 99,99% | The packet error ratio (PER) of the system shall be below 10-4 for a packet size corresponding to 1 ms audio data, with the following exemplary error distribution: - maximum continuous error duration = 30 ms, - consecutive minimum continuous error-free duration = 100 ms. To make packet errors inaudible, error concealment is used at application level. Every concealment is capable of handling one specific kind of error distribution. Therefore, the above numbers describe only one more or less typical concealment case |
| # of audio links | 50 - 300 | Simultaneous audio links |
| Service area | ≤ 10.000 m2 | Event area, indoor and outdoor. Typical heights of indoor stages: 5 – 10 m |
| Synchronicity | 0,25 µs – 1 µs | All wireless mobile devices of one local high quality network shall be synchronized at the application level within the specified accuracy |
| User speed | ≤ 50 km/h | For instance, in musical events like the “Starlight Express” artists are moving on with speeds up to 50 km/h |
| Security/ Integrity | The user data is encrypted |  |

The relation between application latency and end-to-end latency for the live performance use case is depicted in Figure 5.X.2.1-2. End-to-end latency is to be understood as the user plane latency between two terminal devices, communicating via potentially more than one hop. Without having the audio mix function integrated, additional wireless hops will be required for the complete round trip (end-to-end latency in Figure 5.X.2.1-3) from the microphone of the artist back into his/her IEM system. Furthermore, the application latency in the live performance use case must be guaranteed for every packet of every audio stream, independently of the system load, with the specified data rates, and for pre-defined durations.

Figure 5.X.2.1-2.: End-to-end delay shown for a live performance use case (system delay refers to an internal latency introduced per wireless hop)

### Pre-conditions

All wireless audio devices on-site (Figure 5.X.1-2) are switched on and connected to a PMSE production 5G network.

### Service Flows

A typical service flow in a live performance production network may look as follows:

- A number of wireless audio devices (microphones) capture and process audio signals, for instance from the artists or music instruments. Each audio signal is sampled and processed into an isochronous audio stream.

- The isochronous audio streams are transmitted wirelessly to the audio mixing console.

- The received audio streams are mixed according to the application requirements and several audio streams are produced (e.g., for the PA system and individual IEM mixings for the artists and recording mixings).

- The IEM mixes are transmitted wirelessly as isochronous audio streams to the corresponding artists.

### Post-conditions

The live performance devices run as required by the application and without any perceivable impairments.

### Challenges to the 5G System

Special challenges to the 5G system associated with this use case include the following aspects:

- Very stringent requirements on communication service availability, PER and end-to-end latency throughout the whole operation time. Here, end-to-end latency requirements are to be understood not as an average value for the data stream, but as a limit for every packet of the stream.

- Very stringent requirements on clock synchronicity between different nodes at application level. This implies that the 5G system must support deterministic packet transfer and ultra-precise time synchronization, even in the user equipment. Time-sensitive networking features may be revealed to the application or equivalent interfaces implemented.

- Transmission of rather small chunks of data, resulting in potentially significant overhead due to signalling, local routing, security, etc.

### Potential Requirements

| Reference Number | Requirement text | Application / Transport |
| --- | --- | --- |
| PMSE 1.1 | The 5G system shall support ultra-low latency communication. Maximum end-to-end latency shall be 1 ms for user experienced data rates between 150kb/s and 4,61Mb/s. NOTE: The end-to-end latency is the requirement for uplink (microphone to mixer) or downlink (mixer to ear piece). NOTE: In this use case the UE is either a transmit-only UE (microphone) or receive-only UE (ear piece). | A |
| PMSE 1.2 | The 5G system shall support a clock synchronicity of a communication group of 50 – 300 UEs of 1 us. | A |
| PMSE 1.3 | The 5G system shall support communication service availability of 99,9999%. | T |
| PMSE 1.4 | The 5G system shall support data integrity and confidentiality protection, even for communication services with ultra-low latency and ultra-high reliability requirements. | T |
| PMSE 1.5 | The 5G system shall support hot-plugging in the sense that new devices may be dynamically added to and removed from a live performance application, without any observable impact on the other devices. | T |
| PMSE 1.6 | The 5G system shall support UE speeds up to 14 m/s (50km/h), even for communication services with ultra-low latency and ultra-high reliability. | T |

Editor’s note: Service area for this use case is FFS.

------------------------- End of Change 1 ----------------------------

Start of Change 2

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

IEM	In-Ear Monitor

PA	Public Adress

PER	Packet Error Rate

PMSE	Programme Making and Special Events

UE	User Equipment

End of Change 2

--------------------------------------- END OF PROPOSED CHANGES ---------------------------------------