3GPP TSG-SA WG1 Meeting #80	S1-174339

Reno, Nevada, USA, 27 November - 1 December 2017	(revision of S1-174146)

Title:	Use case: Low-latency audio streaming for local conference systems

Agenda Item:	8.5 - FS_CAV

Source:	Intel, Robert Bosch, Fraunhofer Heinrich-Hertz-Institut (HHI), IRT, EBU, BMWi

Contact:	valerio.frascolla@intel.com

Abstract: This contribution describes the use case “Low-latency audio streaming for local conference systems” of the Programme Making and Special Events (PMSE) vertical. It contains the use case description and associated requirements for 5G systems.

### Acknowledgment

# The following entities contributed to this document:

### PMSE-xG Project (www.pmse-xg.de), co-funded by the German Ministry of Transportation and Digital Infrastructure.

# Discussion

Note: a high-level description of this use case (S1-172381) was presented at the 3GPP SA-1 #78 meeting in Porto.

# Proposal

------------------------- START OF PROPOSED CHANGES ----------------------------

------------------------- Start of Change 1 ----------------------------

### 5.X.3	Low-latency audio streaming for local conference systems

### 5.X.3.1	Description

In a conference system the voice of several speakers is captured by a conference unit, transmitted to the base station of the Local High Quality Network (see Figure 5.X.3.1-1), where the mixing of the different audio streams is done, and distributed to all other conference units connected to the network, which replay the received audio stream. Low latency is an issue because the speakers always hear themselves speaking.

The main task of a conference system is to distribute voice from one participant to other participants either within the same location, e.g. a room or the conference venue, or to a remote location. The conference units can be centrally configured and controlled, and can start a poll among the participants. As a concrete example, the local conference system is described in more details and its typical system parameters are listed in Table 5.X.3.1.

Figure 5.X.3.1-1describes the logical topology of a local conference system. Each conference unit can act as a transmitter or receiver of an audio stream. The speakers’ voices are transmitted to a conference service application via a central base station (red arrow) from where the processed audio streams are distributed again to the other conference units (blue arrows). The receiving conference units can select between different channels, e.g., each one containing a different mixture of the speakers’ voices or their respective language translations. The conference system can be deployed either in a fixed installation setup or in a moving/portable setup. In the fixed installation case the surrounding conditions, e.g. the presence of cellular network coverage, stay constant over a longer period in time. In the portable setup case, the conference system is deployed in an ad-hoc manner. The location and time of deployment are only known at short-term and indeed can change even across country-borders.

Figure 5.X.3.1-1: Logical topology of the local conference system use case

Table 5.X.3.1: System parameters of the local conference system use case

|  | Local Conference System | Comments |
| --- | --- | --- |
| Application latency | < 20 ms | End-to-end maximum allowable latency of the use case in focus (i.e. between the microphone and the speaker/headphone), includes application and application interfacing. |
| End-to-end latency | <4 ms | Latency that is introduced per link of the 5G wireless communication system excluding application and application interfacing. |
| User experienced data rate | 150kb/s -768 kb/s | 4x in UL, 42x in DL |
| Control data rate | ≤ 50 kb/s. | Data rate per control link |
| Communication service availability | 99,9999% |  |
| Reliability | 99,99% | The audio frames (with a packet duration of 4 ms) provided to the application layer shall have a packet error ratio (PER) below 10-4. |
| # of audio links | 500 | The system shall support the control of 500 user terminals per cell |
| Service area | ≤ 2500 m2 | Conference area, typically indoor. Typical heights: 3 – 5 m |
| Synchronicity | ≤ 20 µs | All wireless mobile devices shall be synchronized inside one local high quality network, for instance with respect to a reference system clock provided by one master device, e.g. the 5G base station. |
| System setup time | ≤ 10 ms |  |
| Internet access | optional | The system shall be able to work without access to the Internet |

### 5.X.3.2	Pre-conditions

All wireless audio devices on-site (Figure 5.X.3.1-1) are switched on and connected to a PMSE production network through a local 5G base station.

### 5.X.3.3	Service Flows

A typical service flow in a local conference network may look as follows:

- The voice of several speakers is captured by a conference unit and wirelessly transmitted to a conference system where the mixing of the different audio streams is done and transferred into the floor channel.

- The floor channel is wirelessly transmitted as a multicast stream to all other conference units connected to the network.

- Additional audio mixes (e.g. interpretation channels) are distributed wirelessly to dedicated conference units (unicast transmission) connected to the network.

### 5.X.3.4 Post-conditions

The local conferences devices run as required by the application.

### 5.X.3.5	Challenges to the 5G System

Special challenges to the 5G system associated with this use case include the following aspects:

- Very stringent requirements on end-to-end latency, PER and communication service availability.

- Very stringent requirements on clock synchronicity between different nodes at application level. This implies that the 5G system must support deterministic packet transfer and ultra-precise time synchronization, even in the user equipment. Time-sensitive networking features may be revealed to the application or equivalent interfaces implemented.

- Transmission of rather small chunks of data, resulting in potentially significant overhead due to signalling, routing, security, etc.

- Multichannel operation and multicast support.

### 5.X.3.6	Potential Requirements

| Reference Number | Requirement text | Application / Transport | Comments |
| --- | --- | --- | --- |
| PMSE 1.1 | The 5G system shall support a clock synchronicity at application level between a communication group of 50 – 5001 UEs in the order of 20 µs or below. | T |  |
| PMSE 1.2 | The 5G system shall support data integrity and confidentiality protection , even for communication services with ultra-low latency and ultra-high reliability requirements | T |  |
| PMSE 1.3 | The 5G system shall support communication service availability exceeding at least 99,9999% | T |  |
| PMSE 1.4 | The 5G system shall support hot-plugging in the sense that new devices may be dynamically added to and removed from a local conference application, without any observable impact on the other devices. | T |  |
| PMSE 1.6 | The 5G system shall support industry standards for precision clock synchronization (e.g.,IEEE 1588) for IP based A/V systems in a way that the synchronization requirements outlined in Table 5.X.3.1-1 can be met. | T |  |
| Note 1: In multicast operation |  |  |  |

------------------------- End of Change 1 ----------------------------

Start of Change 2

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

A/V	Audio/Video

IEEE	Institute of Electrical and Electronics Engineers

IP	Internet Protocol

PER	Packet Error Rate

PMSE	Programme Making and Special Events

PTP	Precision Time Protocol

UE	User Equipment

End of Change 2

--------------------------------------- END OF PROPOSED CHANGES ---------------------------------------