3GPP TSG-SA WG1 Meeting #81 	S1-180422

Fukuoka, Japan, 5 - 9 February 2018	(revision of S1-180405 was S1-180139)

Title:	FS-5GSAT -  Proposed definitions in TR 22.822

Source:	THALES, TNO, KPN, Fraunhofer IIS, Nomor Research Gmbh, Hughes Network System Ltd, CNES, CTTC

Type:	pCR

For:	Approval

Agenda Item:	7.7 FS_5GSAT

Specification:	TR 22.822

Contact:	Didier Delmas, didier.delmas@thalesaleniaspace.com

Abstract: A number Editor’s notes are identified in TR 22.822 v.1.0 calling for some clarifications or definitions of the terminology associated with the existing use cases. The purpose of this document is to answer to the need for clarification and to propose the some definitions .

# 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in [1].

------------------------------- START OF PROPOSED MODIFICATION TO TR 22.822 -------------------------------

The definitions given in 3GPP TS 22.261 V16.2.0 (2017-12)[2] are also applicable in this report.

The definitions given in 3GPP TS 23.501 V15.0.0 (2017-12) [3] are also applicable in this report

Satellite : a space-borne vehicle embarking a bent pipe payload or a regenerative payload telecommunication transmitter, placed into Low-Earth Orbit (LEO) typically at an altitude between 500 km to 2000 km, Medium-Earth Orbit (MEO) typically at an altitude between 8000 to 20000 lm, or Geostationary satellite Earth Orbit (GEO) at 35 786 km altitude.

Satellite network: Network, or segments of network, using a space-borne vehicle to embark a transmission equipment relay node or base station.

Satellite RAT: a RAT defined to support at least one satellite.

5G Satellite RAT: a Satellite RAT defined as part of the New Radio.

5G satellite access network: 5G access network using at least one satellite.

Terrestrial: located at the surface of Earth

Terrestrial network: Network, or segments of a network located at the surface of the Earth

-------------------------------- END OF PROPOSED MODIFICATION TO TR 22.822 -------------------------------