3GPP TSG-SA WG1 Meeting #81	                                                      S1-180435

# Fukuoka, Japan, 5-9 Feb, 2018														(revision of S1-180079)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.282 | CR | 0018 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS_22.282 CR_LOCOTROL information transmission between engines |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MONASTERY 2 |  |  |  |  |  |  | Date: |  | 2018-02-07 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To add requirement related to use case “LOCOTROL information transmission between engines” from TR 22.889 clause 6.16.3 . The potential requirements is partially covered by TS 22.282 as following: The FRMCS system shall provide a mean to exchange messages between different FRMCS Equipments with a QoS, priority and setup time as defined in clause 12.10. The QoS, priority related requirements are specified in TS 22.282, i.e., [R-6.1.2.1.2-003], [R-6.1.2.1.2-007] ~ [R-6.1.2.1.2-010]. But currently no requirement is specified for communication between robots. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a new requirement to support communication between Robots. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This capability will be missing from Rel-16 MCData |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*************** Start modification  ****************

6.1.2	Robots

6.1.2.1	Robots remote control

6.1.2.1.1	Service description

Robots as defined in 3GPP TS 22.281 [6] will be used more and more to provide unique services to mission critical organizations. Critical communications users need, as a consequence, a common communication framework for robots which can take advantage of different transport technologies such as a 3GPP system. The MCData Service, working in conjunction with existing robot control capabilities, will provide mechanisms to do that.

The following sub-clause aims at defining requirements to ensure robot control communication can be provided through a 3GPP system.

We expect different manufacturers for robots. As a consequence a well-known transport framework is needed in order to ensure easy integration of new robots.

6.1.2.1.2	Requirements

[R-6.1.2.1.2-001] The MCData Service shall enable the control of robots.

[R-6.1.2.1.2-002] The MCData Service shall provide a common transmission framework to use and control robots.

[R-6.1.2.1.2-003] The MCData Service shall provide a default control latency depending on the robots type under

- 50 ms for an aerial unmanned vehicle;

- 200 ms for an aquatic or submarine unmanned vehicle; and

- 400 ms for a terrestrial unmanned vehicle .

NOTE: 	At this stage of the work, the latency is an end to end latency. The split between network latency and robot latency is left for stage 2. The latency is measured between the action of the pilot and the movement of the robot (not only MCData Service).

[R-6.1.2.1.2-004] The MCData Service shall be able to simultaneously manage multiple robots and robot types.

[R-6.1.2.1.2-005] Void.

[R-6.1.2.1.2-006] The MCData Service shall support management of an aerial unmanned vehicle at an altitude of up to 150 m above the floor.

[R-6.1.2.1.2-007] The MCData Service shall have a default priority scheme for each kind of robot (aerial, aquatic, submarine or terrestrial unmanned vehicles).

[R-6.1.2.1.2-008] The MCData Service shall be able to provide relevant priorities to different MCData communications according to the default priority scheme without additional configuration.

[R-6.1.2.1.2-009] The MCData Service default priority scheme shall ensure that data exchanged for controlling a robot has relevant high priority amongst user data and cannot be pre-empted.

[R-6.1.2.1.2-010] The MCData Service default priority scheme shall ensure that essential robots telemetry data (such as position when out of sight) has also a high priority and cannot be pre-empted.

[R-6.1.2.1.2-011] The essential telemetry data shall be identified and minimized in order not to forbid critical operational data to be transmitted.

6.1.2.x	Robots communication

[R-6.1.2.1.2-012] The MCData Service shall support to initiate a MCData communication between robots automatically or by controller.

[R-6.1.2.1.2-013] The MCData Service shall support to terminate a MCData communication between robots automatically or by controller.

*************** End modification  ****************