3GPP TSG-SA WG1 Meeting #81	S1-180536

Fukuoka, Japan,  5-9 February 2018	(revision of S1-180348)

Title:	Consensus view of definition drafting session

Agenda Item:	7.4

Source:	Nokia, Nokia Shanghai Bell

Contact:	Betsy Covell  betsy.covell@nokia.com

Abstract: A drafting session was held Wed afternoon to discuss the need for a new term/definition for Rel 16 to capture the essence of a non-public network that can interact with a public network. The consensus view of that drafting session is captured below.

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

aggregator: Service provider managing a system of electric generation units, storage systems, and load (consumers), with independent control and customer support in its own coverage area

automation: the automatic operation or control of a process, device, or system

NOTE 1: This definition is based on [10].

communication service availability: percentage value of the amount of time the end-to-end communication service is delivered according to an agreed QoS, divided by the amount of time the system is expected to deliver the end-to-end service according to the specification in a specific area.

NOTE 2: The end point in "end-to-end" is assumed to be the communication service interface.

NOTE 3: The communication service is considered unavailable if it does not meet the pertinent QoS requirements. If availability is one of these requirements, the following rule applies: the system is considered unavailable in case an expected message is not received within a specified time, which, at minimum, is the sum of end-to-end latency, jitter, and survival time.

NOTE 4: This definition was taken from section 3.1 in [3].

communication service reliability: ability of the communication service to perform as required for a given time interval, under given conditions

NOTE 5: Given conditions would include aspects that affect reliability, such as: mode of operation, stress levels, and environmental conditions.

NOTE 6: Reliability may be quantified using appropriate measures such as meantime to failure, or the probability of no failure within a specified period of time.

NOTE 7: This definition is based on [2].

end-to-end latency: the time that takes to transfer a given piece of information from a source to a destination, measured at the communication interface, from the moment it is transmitted by the source to the moment it is successfully received at the destination

NOTE 8: This definition was taken from section 3.1 in [3].

exclusive network: a 3GPP network deployment that is not for public use and may interact with a public network. This network uses only 3GPP authentication methods, identities, and credentials for network access.

IoT device: a type of UE which is dedicated for a set of specific use cases or services and which is allowed to make use of certain features restricted to this type of UEs.

NOTE 9: An IoT device may be optimised for the specific needs of services and application being executed (e.g., smart home/city, smart utilities, e-Health and smart wearables). Some IoT devices are not intended for human type communications.

NOTE 10: This definition was taken from section 3.1 in [3].

jitter: the variation of a time parameter relative to a reference or target value

NOTE 11:  In this document, jitter is used for describing the variation of end-to-end latency, and update time.

microgrid: Local grid, with own energy generation and power consumption; limited geographical area, typical example: power network for a university campus

private network: an isolated network deployment that does not interact with a public network.

NOTE 12: This definition was taken from section 3.1 in [3].

renewable generators: photovoltaic panels or wind turbines; energy generation unit

transmission time: the interval from a start event at the reference interface of a source until a stop event of the same transmission at the reference interface of a target

NOTE 13: Depending on the type of reference interface, the start event can be the transfer of the first bit of user data, the first byte, or a trigger event at a process interface. Respectively, the stop event can be the last bit of user data, the last byte or a trigger event of a process interface.

NOTE 14: This definition is based on [19].

update time: the interval from a start event at the reference interface of a target until a following stop event at the same reference interface

NOTE 15: Depending on the type of reference interface, the start event can be the transfer of the last bit of user data, the last byte, or a trigger event at the process interface of a consumer.

NOTE 16: The stop event can be the last bit of user data, the last byte, or a trigger event of a process interface that can be referred to the following successful transmission of the same source

NOTE 17: This definition is based on [19].

up state: state of being able to perform as required

NOTE 18: This definition is based on entry IEV 192-02-01 in [49].

up time: time interval for which the item is in an up state

NOTE 19: This definition is based on entry IEV 192-02-02 in [49].

user equipment: An equipment that allows a user access to network services via 3GPP and/or non-3GPP accesses.

NOTE 20: This definition was taken from section 3.1 in [3].

vertical domain: a particular industry or group of enterprises in which similar products or services are developed, produced, and provided