3GPP TSG-SA WG1 Meeting #82	S1-181086

# Dubrovnik, Croatia, 7-11 May 2018												(revision of S1-18xxxx)

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.889 | CR | 0060 | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | gap analysis for real time data transmission in 6.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_FRMCS2 |  |  |  |  |  |  | Date: |  | 2018-04-26 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | As currenlty the potential requirements in 6.14 use case are mostly covered by TS 22.282 or 22.280, so to update the gap analysis results based on following analysis. - [R-6.14.5-001], The FRMCS System shall enable an authorized FRMCS-user to request another FRMCS User to establish real time data transmission. It is covered by the requirement in TS 22.282 as following: [R-6.1.3.2-001] The MCData Service shall provide a mean to share in real-time operational status information between members of a selected group. - [R-6.14.5-002], The FRMCS System shall provide a list of identities for the requesting authorized FRMCS User to select from. The list may contain functional identities, FRMCS User identities, or FRMCS Equipment Identities It is covered by the requirement in TS 22.282 as following: [R-5.2.2-009] The MCData Service SDS shall provide a selectable read receipt indication. When requested, the receiving entity shall provide receipt indication for delivered and read messages as appropriate. The read indication will implicitly indicate delivered as well. - [R-6.14.5-003],The real time data request shall either be based on the functional identity, FRMCS User Identity, or FRMCS Equipment Identity. It is covered by functional alias requirements in TS 22.280 that consideraing functional identity, user identity, and equipment identity, as following: [R-5.9a-002] If the MCX Service system supports functional aliases, the MCX User shall be reachable by its functional alias(es). - [R-7.4.5-004] The FRMCS User shall be able to accept a real time data transmission request. It is covered by the requirement in TS 22.280 as following: [R-5.21.1.2-003] The MCX Service shall provide a mechanism for an authorized MCX User (e.g. officer in the field with wearable camera) to accept or deny access to an MCX Service Group Communication from their MCX UE. - [R-7.4.5-005] The FRMCS User shall be able to reject a data in real time transmission request. It is covered by the requirements in TS 22.282 as following: [R-6.3.1.2-005] The MCData service shall provide a mechanism for an MCData User to reject an MCData Private Communication. - [R-7.4.5-007] The requesting FRMCS User shall be notified if the request is rejected or ignored. It is covered by the requirement in TS 22.280 as following: [R-6.19.1.1-003] The MCX Service shall notify the requesting MCX Participant and may notify one or more authorized users when a communication is queued, when a communication is rejected, when communications has started after being de-queued. There is no [R-7.4.5-006] requirement, so to adapt the index. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To change “not covered” to “covered by 22.282” or “covered by 22.280”in “SA1 spec covering” column, and update the comment column for each requirement. - Delete the row for [R-7.4.5-006], change index of the requirement [R-7.4.5-007] to [R-7.4.5-006] |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The currently gap analysis results are wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.14.5. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***************************** Start of Change  ***************************************

## 6.14	Use case: Data transmission in real time

### 6.14.1	Description

This use case allows the authorized FRMCS Users to exchange data in real time from each other, for example, health and status of train are transferred between intelligent on-train systems and train maintenance.

### 6.14.2	Pre-conditions

The user A is authorized FRMCS User (e.g., administrator who has authorization to the data).

User B is an equipment for monitoring the train.

User C is a normal FRMCS User (e.g., user without authorization to the data)

### 6.14.3	Service flows

User A requests User B to transmit the data (e.g., health, status of train, etc.) in real time.

User B receives the request from User A, and accepts the request.

User B starts to transmit the requested data in real time to User A.

User C requests User B to transmit the data (e.g., health, status of train, etc.) in real time.

User B receives the request from C, and rejects the request (e.g., without authorization to the data).

User C receives a notification that the request was rejected by User B.

### 6.14.4	Post-conditions

The data of User B is sent to User A in real time.

The data of User B is not sent to User C in real time.

### 6.14.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.14.5-001] | The FRMCS System shall enable an authorized FRMCS-user to request another FRMCS User to establish real time data transmission. | A | covered TS 22.282, | [R-6.1.3.2-001] The MCData Service shall provide a mean to share in real-time operational status information between members of a selected group |
| [R-6.14.5-002] | The FRMCS System shall provide a list of identities for the requesting authorized FRMCS User to select from. The list may contain functional identities, FRMCS User identities, or FRMCS Equipment Identities | A | covered TS 22.282, | [R-5.2.2-009] The MCData Service SDS shall provide a selectable read receipt indication. When requested, the receiving entity shall provide receipt indication for delivered and read messages as appropriate. The read indication will implicitly indicate delivered as well. |
| [R-6.14.5-003] | The real time data request shall either be based on the functional identity, FRMCS User Identity, or FRMCS Equipment Identity. | A | covered TS 22.280 | [R-5.9a-002] If the MCX Service system supports functional aliases, the MCX User shall be reachable by its functional alias(es). Functional Alias is specified by merging functional identity, user identity, and equipment identity. |
| [R-6.14.5-004] | The FRMCS User shall be able to accept a real time data transmission request. | A | covered TS 22.282, | [R-5.3.2-003] The MCData file distribution capability shall provide an option for each recipient to choose to receive the file or not (e.g. by storing the file and sending a link (URL) to all relevant members). By using this capability a user may choose when to receive the file. |
| [R-6.14.5-005] | The FRMCS User shall be able to reject a data in real time transmission request. | A | covered TS 22.282 | [R-6.3.1.2-005] The MCData service shall provide a mechanism for an MCData User to reject an MCData Private Communication. |
| [R-6.14.5-006] | The requesting FRMCS User shall be notified if the request is rejected or ignored. | A | covered TS 22.280, | [R-6.19.1.1-006] An MCX Service User shall be notified when his request to transmit is removed from the request queue. |

***************************** End of  Change  ***************************************