3GPP TSG-SA WG1 Meeting #82 	S1-181376

Dubrovnik, Croatia, 7 - 11 May 2018	(revision of S1-181071)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.278 | CR | 0245 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of streaming services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, China Telecom, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLESTR |  |  |  |  |  |  | Date: |  | 2018-04-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Mobile network operators are promoting to use LTE based system to support streaming services. But there are a lot of challenges the LTE system has to overcome. For example, -- when multiple access connections are available, which one is better to transmit streaming? -- radio resources are not fully utilized in certain areas and certain time while streaming services cost radio resources much more than other services. -- New technology e.g. local data center will help streaming transmission more efficiently. In the SI of eLESTR, potential service requirements for optimization for streaming transmission in LTE system have been identified and consolidated. It is proposed to add the requirements into TS22.278 to support more efficient streaming services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed that in the TS22.278 section 6, add a new subsection named: 6.7 support of streaming services and add the consolidated requirements identified in TR22.883. It is also proposed that in the TS22.278 section9, add a new subsection named: 9.6 Security and authorization requirements for streaming service and add the consolidated security requirements identified in TR22.883 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LTE system can’t support streaming services efficiently. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2,6.x, 9.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**************************************** begin of 1st Change **************************************

- -

# 3	Definitions and abbreviations

# 3.2	Abbreviations

…

SHE   service hosting environment

**************************************** end of 1st Change****************************************

**************************************** begin of 2nd  Change *************************************

6	Basic capabilities

…

# 6.X support of streaming services

A 3GPP system shall support a mechanism to indicate to UEs that operator streaming services are available.

The 3GPP system shall be able to schedule deliver a download-and-play service to UE at the off-peak hours.

The 3GPP system shall be able to support local connection to local streaming servers according to operator’s policy.

The 3GPP system shall be able to configure local streaming service area according to operator’s policy.

The 3GPP system shall be able to identify a user is in the specific local streaming service area.

The 3GPP system shall be able to identify a user is not in the specific local streaming service area and stop the local streaming service.

The 3GPP system shall be able to deliver streaming content via local service delivery immediately or postponed according to user’s preference.

According to operator’s policy, the 3GPP system may be able to automatically change streaming service route, e.g.  from local route to more general route, vice versa.

According to operator’s policy, the 3GPP system shall be able to configure which context information is related with streaming route.

The 3GPP system shall be able to collect context information related to update streaming route according to operator’s policy.

According to agreement between application server and Operator, the 3GPP system shall support the operator or the 3rd party to configure which user profile information belonging to the application server should be shared with the 3GPP system.

According to operator’s policy, the 3GPP system shall support the SHE which is deployed in different local area (no matter belongs to operator or the 3rd party) to configure which radio environment information can be shared to the application layer which is hosted in the SHE.

According to operator’s policy, the 3GPP system shall support the SHE which is deployed in different local area (no matter belongs to operator or the 3rd party) to get radio environment information based on the configuration.

**************************************** end of 2nd  Change****************************************

**************************************** begin of 3rd  Change **************************************

# 9.X Security and authorization requirements for streaming service

Security measures shall be provided to protect access to streaming service via different RATs or operators, e.g. the MNVOs with RAN/CN sharing.

The 3GPP system shall be able to authorize the streaming delivery based on subscription and on policy defined by a third party.

**************************************** end of 3rd  Change****************************************