3GPP TSG-SA WG1 Meeting #82	                                                      S1-181421

# Dubrovnik, CROATIA, 7-11 May, 2018												(revision of S1-181084)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.282 | CR | 0024 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS_22.282 CR_setup time requirement for monitoring and control of critical infrastructure communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MONASTERY 2 |  |  |  |  |  |  | Date: |  | 2018-05-08 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the requirement [R-6.13.2-002] in 6.13 of TR 22.889, [R-6.13.2-002] The FRMCS System shall setup the monitoring of critical infrastructure communication with an operator defined QoS and priority and setup time as defined in clause 12.10. QoS and priority of mission critical are already supported by 3GPP QoS. But there is a gap related to setup time that not covered by existing spec. so to add new requirement for setup time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new requirement for setup time, 1) less than 1 second for immediate setup of a data communication. 2) less than 3 second for normal setup of a data communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New requirement for setup time is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*************** Start modification  ****************

5.4	Data streaming capability

5.4.1	General

Data streaming is a fundamental capability of the MCData Service. Data streaming can be used to provide a standalone data streaming capability or can be invoked by a controlling application to support the purpose of the application.

5.4.2	Requirements

[R-5.4.2-001] The MCData Service shall provide a data streaming capability.

[R-5.4.2-002] The MCData data streaming capability shall provide an option that allows each recipient to choose to receive the data stream or not (e.g. by sending a link (URL) to all relevant members).

[R-5.4.2-003] The MCData data streaming capability shall allow a user to reject to receive the datastream.

[R-5.4.2-004] The MCData data streaming capability shall provide a sending user selectable indication for automatic reception by the UE.

[R-5.4.2-005] The MCData data streaming capability shall provide start and stop records to the sender for each recipient successfully receiving the data stream.

[R-5.4.2-006] The MCData data streaming capability shall make use of available system delivery efficiencies for streaming of common information to users within the same relevant area and able to receive at the same time.

[R-5.4.2-007] The MCData data streaming capability shall allow a user to cancel streaming of data they have initiated including data remaining buffered in the system waiting to be streamed.

[R-5.4.2-008] The MCData data streaming capability shall allow an authorised user to terminate streaming of data being sent and cancel streaming of data remaining buffered in the system waiting to be streamed.

[R-5.4.2-009] The MCData data streaming capability shall provide a means to guarantee the setup time of:

- Less than 1 second for immediate setup of a data communication.

- Less than 3 second for normal setup of a data communication.

NOTE 1:	Setup time of a communication is essential because of their safety related character.

NOTE 2:	Communication setup encompasses the value of the elapsed time between the communication establishment request and the indication of successful communication establishment.

*************** End modification  ****************