# Requirements

US1-1 As a user, I want to receive (voice or video) calls on my Mobile Device that are directed to any added fixed or mobile phone number.

R1-1-1 It shall be possible to configure the service that any incoming (voice or video) calls either directed to the MSISDN of the SIM in the Mobile Device, i.e., the native MSISDN / phone number / identity, or to any added phone number (fixed or mobile) can be received on the Mobile Device.

R1-1-2 For the added phone numbers, it shall be possible to configure different audible and / or visible Incoming Call Notifications. For the native phone number, the caller identity ringtones are applied, but these are not applied for the added phone numbers.

R1-1-2-1 The user shall be able to label native or any added phone number  to be presented in call logs or during ringing phase of incoming calls.

R1-1-2-2 Labels shall include text (up to 40 characters).

US1-2 As a user, I want to intuitively choose any available identity to be used for outgoing (voice or video) calls.

R1-2-1 When making a (voice or video) call, the user shall be able to choose from the available phone numbers the calling party phone number that shall be presented to the called party and in call logs of both calling and called party’s devices.

R1-2-1-1 The user shall be able to determine a default phone number for outgoing calls if no selection was made.

R1-2-1-2 The user should be able to make that default selection in advance either by each contact or by contact groups.

R1-2-1-3 If no default phone number was selected by the user, the default is set to the MSISDN of the SIM in the Mobile Device.

R1-2-1-4 When making or returning a call from the call log, the outgoing phone number used to place the call shall be the same phone number on which the call was placed or received.

R1-2-2 Emergency calls shall always use the native MSISDN of the Mobile Device.

R1-2-3 All voice and video services and features that are provisioned for a given identity shall be available to the user.

US1-3 As a user, I want to be able to add and remove phone numbers that are associated with a Mobile Device securely, quickly and simply.

R1-3-1 The Mobile Device shall offer UI procedures that allow to add other phone numbers to the Mobile Device.

R1-3-1-1 If a phone number, that belongs to another device, was added to a Mobile Device, it shall not automatically determine the reciproke addition.

R1-3-1-2 A user-friendly process shall be installed that ensures only those phone numbers can be added that the user is eligible to add.

R1-3-2 The Mobile Device shall offer UI procedures that allow to remove phone numbers from a Mobile Device (except the MSISDN of the SIM inside the Mobile Device – the native MSISDN).

R1-3-3 The Mobile Device shall offer UI procedures that allow enabling and disabling of the added phone numbers to the Mobile Device.

R1-3-4 When a phone number is added and enabled or disabled and removed, all Operator Voice and Video calls and Operator Messaging services shall be directed or not to the Mobile Device accordingly.

R1-3-5 The user shall be informed about successful addition, removal, enablement or disablement of an additional phone number on / from the Mobile Device.

R1-3-6 Any phone number can be added on one or more devices.

R1-3-7 It shall be possible to enable or disable any of the added phone numbers on a temporary basis.

R1-3-8 If the SIM is moved from the Mobile Device to another device, any existing configuration (i.e., but not limited to, added phone numbers, status of ‘enabled’ or ‘disabled’) for the Multi Identity service is maintained and carried over to the new Mobile Device, if the new device supports the service.

NOTE: If the device that the SIM has changed to is not Multi Identity capable, the implementing operator can define the behaviour.

US 1-4 As a user, I want to be able to see all call details in my call log.

R1-4-1Call log(s) shall present called/used identities by their phone number or Label.

US1-5 As a user, I want to use Operator Messaging services with the native identity of the Mobile Device I currently use and with added identities (MSISDN or fixed phone numbers).

R1-5-1 When using outgoing Operator Messaging services (e.g. sending a message or a file), the user shall be able to choose the sending party phone number from the available identities. This shall be presented to the receiving party and in the message logs of both sender and receiver devices.

R1-5-1-1 The user shall be able to determine a default phone number for outgoing Operator Messaging services if no selection was made.

R1-5-1-2 The user should be able to make that default selection in advance either by each contact or by contact groups.

R1-5-1-3 If no default phone number was selected by the user, the default is set to the MSISDN of the SIM in the MobileDevice.

R1-5-1-4 When answering a message from the messaging thread, the outgoing identity used shall be the same phone number as addressed.

R1-5-2 Critical / emergency messages (if available) shall always use the native MSISDN of the MobileDevice.

R1-5-3 All Operator Messaging services and features that are provisioned for a given identity shall be available to the user.

US1-6 As an operator I want to configure limitations of the service.

R1-6-1 The operator shall be able to limit the number of identities that can be added on the Mobile Device.

R1-6-1-1 If the limit of identities on a Mobile Device has been reached, the UI shall ensure that user cannot add any further identities and is informed in a friendly way.

R1-6-2 The operator of the Mobile Device identity shall be able to limit the identities that can be added by operator policy.

R1-6-3-1 The UI of the Mobile Device shall help the user to understand which phone numbers can be added. The user shall always be informed about the reason why it may not be possible to add a user-selected phone number.

R1-6-4 The operator shall be able to remove identities from a Mobile Device (e.g., but not limited to, for temporary special purpose numbers).

US 1-7 As a user, I want to control supplementary services per identity on my Mobile Device.

R1-7-1 The user shall be able to control supplementary services from the Mobile Device per available identity.

NOTE: All previously defined services for Operator Voice and Video apply to all identities on the device.