3GPP TSG-SA WG1 Meeting #83 	S1-182697

West Palm Beach, Florida, USA, 20 - 24 August 2018	(revision of S1-182649)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.278 | CR | 0255 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Requirement for Operator controlled Caching service delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom,CATT ,ZTE, CATR |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSTR |  |  |  |  |  |  | Date: |  | 2018-08-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the SI of eLESTR, potential service requirements for optimization for Operator controlled Caching service delivery have been identified and consolidated. It is proposed to add the requirements into TS22.278 to support operator controlled caching service delivery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add the requirement for operator controlled caching service delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LTE system can not support operator controlled caching service delivery efficiently. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

**************************************** begin of 1st Change **************************************

6.7	Support of streaming services

A 3GPP system shall support a mechanism to indicate to UEs that operator streaming services are available The operator provides information on how to use the caching service to the user who has subscribed to this service..

The 3GPP system shall support a mechanism to indicate to UEs that operator controlled caching service is supported. Based on operator policy, the 3GPP system shall support a mechanism that the subscribed UE is configured with the list of authorized content providers and data volume.

The 3GPP system shall be able to schedule delivery of a download-and-play service to UE to the off-peak hours.

The 3GPP system shall be able to support local connection to local streaming servers according to operator’s policy.

The 3GPP system shall be able to configure local streaming service area according to operator’s policy.

The 3GPP system shall be able to identify a user is in the specific local streaming service area.

The 3GPP system shall be able to identify a user is not in the specific local streaming service area and stop the local streaming service.

The 3GPP system shall be able to deliver streaming content via local service delivery immediately or postponed according to user’s preference.

According to the mobility of the users who do not always stay in one place, the 3GPP system should be able to adaptively change video server and the route to continue supply experience for streaming service, e.g. from a local streaming server to a non-local streaming server, vice versa.

According to operator’s policy, the 3GPP system shall be able to configure which context information is related with streaming route.

The 3GPP system shall be able to collect context information related to update streaming route according to operator’s policy.

According to agreement between application server and Operator, the 3GPP system shall support the operator or the 3rd party to configure which user profile information belonging to the application server should be shared with the 3GPP system.

According to operator’s policy, the 3GPP system shall support the SHE which is deployed in different local area (no matter belongs to operator or the 3rd party) to configure which radio environment information can be shared to the application layer which is hosted in the SHE.

According to operator’s policy, the 3GPP system shall support the SHE which is deployed in different local area (no matter belongs to operator or the 3rd party) to get radio environment information based on the configuration.

**************************************** end of 1st Change ****************************************