3GPP TSG-SA WG1 Meeting #83	S1-182713

West Palm Beach, FL, USA,  20-24 August 2018	(revision of S1-182141)

(revision of S1-182141)

Title:	FS_BMNS considerations

Agenda Item:	7.6

Source:	Nokia, Nokia Shanghai Bell, NTT DoCoMo

Contact:	Betsy Covell  betsy.covell@nokia.com

Abstract: This document provides considerations for security for TR 22.830. As no additional charging requirements were identified, it is proposed to delete the subclause on considerations for charging

Discussion:

The following potential requirements relate to security aspects for private slices.

| Potential Requirement | Clean up steps taken | Consolidated potential requirement |
| --- | --- | --- |
| [PR 5.1.2-1] The 3GPP system shall provide suitable APIs to allow use of a trusted 3rd party provided encryption between any UE served by a private slice and a core network entity in that private slice. | PR 5.1.2-1 and PR 5.6.2-1 are very similar, it is proposed to combine them | The 5G system shall provide suitable APIs to allow use of a trusted 3rd party provided encryption between aauthorized UE served by a private slice and a core network entity in that private slice. |
| [PR 5.6.2-1] The 3GPP system shall provide suitable APIs to allow use of additional encryption mechanisms, provided by a trusted vertical 3rd party between any UE served by a private slice and a core network entity in that private slice. |  |  |
| [PR 5.2.2-6] The 3GPP system shall support a mechanism for a 3rd party to authenticate a UE for access to a private network slice. | PR 5.2.-6 and PR 5.6.2-3 are very similar, proposed to combine them This requirement was not agreed to be carried forward. |  |
| [PR 5.6.2-3] The 3GPP system shall support a mechanism for a 3rd party to authenticate a UE for access to a private slice which is allocated to this 3rd party. |  |  |
| [PR 5.7.2-5] Subject to an agreement between a PLMN and a 3rd party, a 5G system shall be able to allow the PLMN to authenticate and authorize UEs for the usage of the 3rd party's own 3GPP network and to operate it in combination with their private network slice(s) in the PLMN. | Split into 2 requirements. The first to be included in clause 6 as a security consideration, the second to be included in clause 7. Subject to an agreement between an MNO and a 3rd party, a 5G system shall support a mechanism for the PLMN to authenticate and authorize UEs for access to both the 3rd party's own 5G network and private slice(s) of the PLMN associated with the 3rd party. And The 5G system shall provide a mechanism to enable an MNO to operate a 3rd party network and private slice(s) of the PLMN associate d with the 3rd party in a combined manner. | Subject to an agreement between an MNO and a 3rd party, a 5G system shall support a mechanism for the PLMN to authenticate and authorize UEs for access to both the 3rd party's own 5G network and private slice(s) of the PLMN associated with the 3rd party. |
| [PR 5.8.2-4] The 5G system shall be able to expose the information of the enabled security mechanisms of a UE to an authorised user for logging and auditing purposes. | This requirement is also included in cyberCAV, there is no need to carry it forward twice |  |
| [PR 5.9.2-3] The 3GPP system shall provide suitable APIs for additional 3rd party authorisation and authentication for controlling the access to network slices. | Small clean up of wording proposed | The 5G system shall provide suitable APIs for additional 3rd party provided authorisation and authentication for controlling the access to private slices. |

Proposed Text:

# 6	Considerations

## 6.1	Considerations on security

The 5G system shall provide suitable APIs to allow use of a trusted 3rd  party provided encryption between an authorized UE served by a private slice and a network entity within that private slice.

Subject to an agreement between an MNO and a 3rd party, a 5G system shall support a mechanism for the PLMN to authenticate and authorize UEs for access to both the 3rd  party's own non-public network and private slice(s) of the PLMN associated with the 3rd party.

The 5G system shall provide suitable APIs for additional authorisation and authentication provided by a trusted 3rd party for controlling the access to private slices.

## 6.2	Considerations on performance

Network slicing as an enabling capability is foundational for a variety of roles dependent on business models. The notion of a collection of functions and resources that constitute a given network slice is a building-block for virtualization.

Since service categories have diverse quality of service profiles and configurability options for a network slice, it would be useful to adapt the realization of a network slice to meet the established key performance targets. Such configuration choices would enable performance improvements, such as latency reduction, minimization of faults, and overall robustness.

From an end-to-end perspective, impact of any performance degradation in a virtualized core network and/or access network will need to be minimized to meet the agreed KPIs (Key Performance Indicators) [2].

Network slice performance studies are important for an assessment of the system-level KPIs (e.g., availability, reliability, latency, mobility, retainability, user experience, coverage, and capacity). Consistency in network slice performance is vital for supporting the service demands associated with different market scenarios and service-level agreements, based on business models and stakeholder requirements.

The performance aspects of network slicing are to be studied in terms of optimization and configurable arrangements.