3GPP TSG-SA WG1 Meeting #84 	S1-183316

Spokane, WA, USA, 12 - 16 November 2018	(revision of S1-183109)

Title:	Performance KPIs for TS 22.104

Agenda Item:	7.7

Source:	Nokia, Nokia Shanghai Bell, Siemens AG, Huawei, Volkswagen AG

Contact:	betsy.covell@nokia.com

Abstract: This contribution proposes adding the performance KPIs from 22.804 to 22.104.  The tables proposed below include content from the consolidated potential requirements in clause 8 of 22.804 that have been enhanced with additional details from the Annexes to provide additional context for the KPIs.

The changes include:

- An additional reference, related to the definition of ‘influence quantity’

- Definitions of terms used in the KPI tables

- KPI tables for periodic deterministic, aperiodic deterministic, non-deterministic, and mixed traffic communications

Proposed Text Changes:

Start of Change 1

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: " Service requirements for the 5G system".

[3]	IEC 61784-3: "Industrial communication networks – profiles – part 3: functional fieldbuses – general rules and profile definitions".

[4]	BZKI, "Aspects of dependability assessment in ZDKI", June 2017.

[5] 	BZKI, "Requirement Profiles in ZDKI", 2017.

[6]	IEC 61158: "Industrial communication networks – fieldbus specification".

[7]	IEC 61907, "Communication network dependability engineering".

[x]	DKE-IEV, German online edition of the International Electrotechnical Vocabulary (IEV), DKE - Deutsche Kommission Elektrotechnik Elektronik Informationstechnik in DIN und VDE, (German Commission Electrical Technology Electronics Information Technology), https://www2.dke.de/de/Online-Service/DKE-IEV/Seiten/IEV-Woerterbuch.aspx

Start of Change 2

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

characteristic parameter: numerical value that can be used for characterising the dynamic behaviour of communication functionality from an application point of view.

clock synchronisation service: the service to align otherwise independent UE clocks.

clock synchronicity: the maximum allowed time offset within the fully synchronised system between UE clocks.

NOTE 1: Clock synchronicity (or synchronicity) is used as KPI of clock synchronisation services.

error: discrepancy between a computed, observed or measured value or condition and the true, specified or theoretically correct value or condition.

NOTE 2: taken from [3].

communication service availability: percentage value of the amount of time the end-to-end communication service is delivered according to an agreed QoS, divided by the amount of time the system is expected to deliver the end-to-end service according to the specification in a specific area.

NOTE 3: The end point in "end-to-end" is assumed to be the communication service interface.

NOTE 4: The communication service is considered unavailable if it does not meet the pertinent QoS requirements. If availability is one of these requirements, the following rule applies: the system is considered unavailable in case an expected message is not received within a specified time, which, at minimum, is the sum of end-to-end latency, jitter, and survival time.

NOTE 5: This definition was taken from clause 3.1 in [2].

communication service reliability: ability of the communication service to perform as required for a given time interval, under given conditions.

NOTE 6: Given conditions would include aspects that affect reliability, such as: mode of operation, stress levels, and environmental conditions.

NOTE 7: Reliability may be quantified using appropriate measures such as meantime to failure, or the probability of no failure within a specified period of time.

NOTE 8: This definition is based on [7].

end-to-end latency: the time that takes to transfer a given piece of information from a source to a destination, measured at the communication interface, from the moment it is transmitted by the source to the moment it is successfully received at the destination.

NOTE 9: This definition was taken from clause 3.1 in [2].

influence quantity: quantity not essential for the performance of an item but affecting its performance.

NOTE 14: This definition is taken from IEV 151-16-31 in [x].

survival time: the time that an application consuming a communication service may continue without an anticipated message.

UE speed: measure of the mobility of a UE with respect to the communication partner (usually a stationary base station).

user experienced data rate: the minimum data rate required to achieve a sufficient quality experience, with the exception of scenario for broadcast like services where the given value is the maximum that is needed.

NOTE 25: This definition was taken from clause 3.1 in [2].

vertical domain: an industry or group of enterprises in which similar products or services are developed, produced, and provided.

Start of Change 3

## 5.2	Periodic deterministic communication

Periodic deterministic communication is periodic with stringent requirements on timeliness and availability of the communication service. A transmission occurs every transfer interval. A description of periodic deterministic communication can be found in clauses 4.3 and 4.4. Additional information on the underlying use cases of the sets of requirements in Table 5.2-1 can be found in Annex A. Further information on characteristic parameters and influence quantities used in Table 5.2-1 can be found in Annex C.

Table 5.2-1: Periodic deterministic communication service performance requirements

| Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability: target value in % (note 1) | Communication service reliability: mean time between failures | End-to-end latency: maximum (note 2) | Service bitrate: user experienced data rate | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area (note 3) | Remarks |
| 99,999 | below 1 year but >> 1 month | < transfer interval value | ≥ 200 kbit/s | ≤ 200 | 100 ms | ~ 500 ms | ≤ 42 m/s | < 25 | 50 km x 200 m | Railbound mass transit - Control of automated train (A.3.2); (note 4) |
| 99,999 to 99,99999 | ~ 10 years | < transfer interval value | – | 50 | 500 μs | 500 μs | ≤ 20 m/s | ≤ 20 | 50 m x 10 m x 10 m | Motion control (A.2.2.1) |
| 99,9999 to 99,999999 | ~ 10 years | < transfer interval value | – | 40 | 1 ms | 1 ms | ≤ 20 m/s | ≤ 50 | 50 m x 10 m x 10 m | Motion control (A.2.2.1) |
| 99,9999 to 99,999999 | ~ 10 years | < transfer interval value | – | 20 | 2 ms | 2 ms | ≤ 20 m/s | ≤ 100 | 50 m x 10 m x 10 m | Motion control (A.2.2.1) |
| 99,9999 | – | < 5 ms | 1 kb/s (steady state) 1,5 Mb/s (fault case) | < 1500 | < 60 s (steady state) ≥ 1 ms (fault case) | TBC | stationary | 20 | 30 km x 20 km | Electrical Distribution - Distributed automated switching for isolation and service restoration (A.4.4); (note 5) |
| 99,9999 to 99,999999 | ~ 10 years | < transfer interval value |  | 1 k | ≤ 10 ms | 10 ms | - | 5 to 10 | 100 m x 30 m x 10 m | Control-to-control in motion control (A.2.2.2) |
| > 99,9999 | ~ 10 years | < transfer interval value | – | 40 to 250 | 1 ms to 50 ms (note 6) (note 7) | transfer interval value | ≤ 14 m/s | ≤ 100 | ≤ 1 km2 | Mobile robots (A.2.2.3) |
| 99,9999 to 99,999999 | ~ 1 month | < transfer interval value | – | 40 to 250 | 4 ms to 8 ms (note 7) | transfer interval value | < 2 m/s | TBC | 50 m x 10 m x 4 m | Mobile control panels – remote control of e.g. assembly robots, milling machines (A.2.4.1) |
| 99,9999 to 99,999999 | ~ 1 year | < transfer interval | – | 40 to 250 | < 12 ms (note 7) | 12 ms | < 2 m/s | TBC | typically 40 m x 60 m; maximum 200 m x 300 m | Mobile control panels -remote control of e.g. mobile cranes, mobile pumps, fixed portal cranes(A.2.4.1) |
| 99,9999 to 99,999999 | ≥ 1 year | < transfer interval value | – | 20 | ≥ 10 ms (note 8) | 0 | typically stationary | typically 10 to 20 | typically ≤ 100 m x 100 m x 50 m | Process automation – closed loop control (A.2.3.1) |
| 99,999 | TBC | ~ 50 ms | – | ~ 100 | ~ 50 ms | TBC | stationary | ≤ 100 000 | several km2 up to 100.000 km2 | Primary frequency control (A.4.2); (note 9) |
| 99,999 | TBC | ~ 100 ms | – | ~ 100 | ~ 200 ms | TBC | stationary | ≤ 100.000 | several km2 up to 100.000 km2 | Distributed Voltage Control (A.4.3) (note 9) |
| > 99,9999 | ~ 1 year | < transfer interval value | – | 15 k to 250 k | 10 ms to 100 ms (note 7) | transfer interval value | ≤ 14 m/s | ≤ 100 | ≤ 1 km2 | Mobile robots – video-operated remote control (A.2.2.3) |
| > 99,9999 | ~ 1 year | < transfer interval value | – | 40 to 250 | 40 ms to 500 ms (note 7) | transfer interval value | ≤ 14 m/s | ≤ 100 | ≤ 1 km2 | Mobile robots (A.2.2.3) |
| 99,99 | ≥ 1 week | < transfer interval value | – | 20 to 255 | 100 ms to 60s (note 7) | ≥ 3 x transfer interval value | typically stationary | ≤ 10 000 to 100 000 | ≤ 10 km x 10 km x 50 m | Process monitoring (A.2.3.2), Plant asset management (A.2.3.3) |
| NOTE 1: One or more retransmissions of network layer packets may take place in order to satisfy the communication service availability requirement. NOTE 2: Unless otherwise specified, all communication includes 1 wireless link (UE to network node or network node to UE) rather than two wireless links (UE to UE). NOTE 3: Length x width (x height). Note 4: 2 UEs per train unit. Note 5: Communication includes two wireless links (UE to UE). NOTE 6: This covers different transfer intervals for different similar use cases with target values of 1 ms, 1 ms to 10 ms, and 10 ms to 50 ms. NOTE 7: The transfer interval deviates around its target value by < ± 25 %. NOTE 8: The transfer interval deviates around its target value by < ± 5 %. NOTE 9: Communication may include two wireless links (UE to UE) |  |  |  |  |  |  |  |  |  |  |

## 5.3	Aperiodic deterministic communication

Aperiodic deterministic communication is without a pre-set sending time, but still with stringent requirements on timeliness and availability of the communication service. A description of aperiodic deterministic communication can be found in clauses 4.3 and 4.4. Additional information on the underlying use cases of the sets of requirements in Table 5.3-1 can be found in Annex A. Further information on characteristic parameters and influence quantities used in Table 5.3-1 can be found in Annex C.

Table 5.3-1: Aperiodic deterministic communication service performance requirements

| Characteristic parameter (KPI) |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability | Communication service reliability: mean time between failures | Max Allowed End-to-end latency (note 1) | Service bit rate: user-experienced data rate | Message size [byte] | Survival time | UE speed | # of UEs | Service Area | Remarks |
| > 99,9999 | ~ 1 week | 10 ms | > 10 Mbit/s |  |  | ≤ 14 m/s | ≤ 100 | ≤ 1 km2 | Mobile robots – video streaming (A.2.2.3) |
| 99,9999% to 99,999999% | ~ 1 month | < 30 ms | > 5 Mbit/s |  |  | < 2 m/s | TBC | TBC | Mobile control panels - parallel data transmission (A.2.4.1) |
| 99,9999 | – | < 50 ms | 0,59 kb/s 28 kb/s | < 100 | – | stationary | 10~100 /km2 | TBC | Smart grid millisecond level precise load control (A.4.5) |
| > 99,99% | ~ 1 week | < 500 ms | ≥ 2 Mbit/s |  | ~ 500 ms | ≤ 42 m/s | < 25 | 50 km x 200 m | Railbound mass transit - CCTV communication service for surveillance cameras (A.3.2); (note 2) |
| > 99,99% | ~ 1 day | < 200 ms | ≥ 200 kbit/s |  | ~ 2 s | ≤ 42 m/s | <25 | 50 km x 200 m | Railbound mass transit - Emergency voice call (A.3.2); (note 2) |
| > 99,9% | ~ 1 month | < 10 ms |  |  |  | < 2 m/s | ≥ 3 | 20 m x 20 m x 4 m | Augmented reality; bi-directional transmission to image processing server (A.2.4.2) |
| NOTE 1: Unless otherwise specified, all communication includes 1 wireless link (UE to network node or network node to UE) rather than two wireless links (UE to UE). NOTE 2: 2 UEs per train unit. |  |  |  |  |  |  |  |  |  |

## 5.4	Non-deterministic communication

Non-deterministic communication subsumes all other traffic types than periodic/aperiodic deterministic communication. This includes periodic/aperiodic non-real-time traffic. A description of non-deterministic communication can be found in clauses 4.3 and 4. Additional information on the underlying use cases of the sets of requirements in Table 5.4-1 can be found in Annex A. Further information on characteristic parameters and influence quantities used in Table 5.4-1 can be found in Annex C.

Table 5.4-1: Non-deterministic communication service performance requirements

| Characteristic parameter (KPI) |  | Influence quantity |  |  | Remark |
| --- | --- | --- | --- | --- | --- |
| Communication service reliability: mean time between failures | Service bit rate: user-experienced data rate | UE speed | # of UEs | Service area (note) |  |
| ~ 1 month | ≥ 1 Mbit/s | ~ 0 m/s ≤ 20 m/s | ≤ 100 | 50 m x 10 m x 10 m | Motion control - software updates (A.2.2.1) |
|  | > 10 Mbit/s | ≤ 14 m/s | ≤ 100 | ≤ 1 km2 | Mobile robots; real-time video stream |
|  | ≥ 1 Gbit/s | ~ 0 m/s | ≥ 1 | train station | CCTV offload in train stations; |
| NOTE: Length x width x height |  |  |  |  |  |

## 5.5	Mixed traffic

Mixed traffic cannot be assigned to one of the other communication patterns exclusively. Additional information on the underlying use cases of the sets of requirements in Table 5.5-1 can be found in Annex A. Further information on characteristic parameters and influence quantities used in Table 5.5-1 can be found in Annex C.

Table 5.5-1: Mixed traffic communication service performance requirements

| Characteristic parameter (KPI) |  |  |  | Influence quantity |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability | Communication service reliability: mean time between failures | Max Allowed End-to-end latency (note 1) | Service bit rate: aggregate user-experienced data rate | Survival time | UE speed | # of UEs | Service Area |  |
| > 99,9999% | ~ 1 year | < 100 ms | 1 Gbit/s | ~ 500 ms | – (note 2) | 2 | 3 m x 1 m | Train coupling (A.3.3) |
| 99,9999999% | ~ 10 years | 16 ms |  |  | stationary | < 1000 | several km² | Wind power park control traffic (A.5.2); |
| NOTE 1: Unless otherwise specified, all communication includes 1 wireless link (UE to network node or network node to UE) rather than two wireless links (UE to UE). NOTE 2: UE speed is irrelevant since this communication takes place between two train segments, |  |  |  |  |  |  |  |  |