3GPP TSG SA WG1 Meeting #84	S1- 183161

Spokane, WA, USA, 12 - 16 November 2018	(revision of S1-183134)

Title:	cyberCAV – 22.104 –network performance: Communication service availability Agenda Item:

Source:	Huawei, Siemens

Contact:	Amanda Xiang, amanda.xiang@huawei.com,

Daniel Medina, daniel.medina@huawei.com,

Bahr, Michael bahr@siemens.com

Abstract: This contribution provides network performance requirement text as well as detail explanation annex regarding the relationship between communication service availability and network performance KPI, such as reliability.

# Rationale

TR 22.804 defines communication service availability and its performance requirements, which are defined and measured at the logical interface between application and communication system. Communication service availability requirements need to be translated into 3GPP network reliability performance requirements such as packet error ratio (PER).

# Proposal

------------------------- START OF PROPOSED CHANGES ----------------------------

### ------------------------- Start of Change 1 ----------------------------

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x1] 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x2] IEC 61907, "Communication network dependability engineering", 2009.

[x3] 3GPP TS 22.261: "Service requirements for the 5G system".

[x4] 3GPP TS 23.501:  "System Architecture for the 5G System".

[x5] Elsayed A. Elsayed (2012). "Reliability engineering" – 2nd edition.

------------------------- End of Change 1 ----------------------------

------------------------- Start of Change 2----------------------------

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [x1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [x1].

packet error ratio (PER): the ratio of PDUs (e.g., IP packets containing sensor updates) that have been received by a source PDU layer (e.g., UE) via the ingress communication service interface in a certain time period T but that are not successfully delivered within a specified deadline by the target PDU layer (e.g., UPF) to the egress communication service interface in the same time period T.

PER=Number of PDUs not successfully delivered by target PDU layer to egress interface during time period TNumber of PDUs received by source PDU layer via ingress interface during time period T

NOTE 1: In a 5G system, the packet error ratio may relate to the packet forwarding treatment that a QoS Flow receives edge-to-edge between the UE and the UPF over a PDU Session.

NOTE 2: The packet error ratio (PER) is an estimate of the packet error probability.

NOTE 3: This definition is based on [x4].

packet error probability: the probability that a PDU (e.g., IP packet containing a sensor update) that has been received by a source PDU layer (e.g., UE) via the ingress communication service interface is not successfully delivered within a specified deadline by the target PDU layer (e.g., UPF) to the egress communication service interface.

NOTE 4: The packet error probability p is the expected value of the packet error ratio (PER), i.e.,

p=EPER=limT→∞Number of PDUs not successfully delivered by target PDU layer to egress interface during time period TNumber of PDUs received by source PDU layer via ingress interface during time period T

communication service availability: percentage value of the amount of time the end-to-end communication service is delivered according to an agreed QoS, divided by the amount of time the system is expected to deliver the end-to-end service according to the specification in a specific area.

NOTE 5: The end point in "end-to-end" is assumed to be the communication service interface.

NOTE 6: The communication service is considered unavailable if it does not meet the pertinent QoS requirements. If availability is one of these requirements, the following rule applies: the system is considered unavailable in case an expected message is not received within a specified time, which, at minimum, is the sum of end-to-end latency, jitter, and survival time.

NOTE 7: This definition was taken from clause 3.1 in [x3].

### ------------------------- End of Change 2 ----------------------------

### ------------------------- Start of Change 3 ----------------------------

# 5	Network performance requirements

### 5.X  Network performance consideration for Communication Service Availability

Communication Service Availability is considered as important service performance requirement for cyber physical application, especially for the applications with deterministic traffic. Although the communication service availability can’t be directly measured by the network, communication service availability requirement is the combination of latency, survival time and reliability requirement for 5G system, because the system is considered unavailable to the cyber physical application when an expected message is not received (e.g. transfer time is bigger than the maximum transfer latency) by the application after application’s survival time expires.

A example of the relationship between network reliability, survive time configuration and communication service availability of logical link is illustrated in the following table x and annex X.

Table 2 example of relationship between packet error probability and communication service availability when survival time is one transfer cycle.

| Communication service availability | Packet error probability p | Reliability ( as defined in TS 22.261) 1-p |
| --- | --- | --- |
| 99.9999% | 10-3 | 99.9% |
| 99.999999% | 10-4 | 99.99% |
| 99.99999999% | 10-5 | 99.999% |
| 99.9999999999% | 10-6 | 99.9999% |
| 99.999999999999% | 10-7 | 99.99999% |

### ------------------------- End of Change 3 ----------------------------

### ------------------------- Start of Change  4 ----------------------------

# Annex X. Communication service availability, packet error ratio and survival time

# (Informative)

## X.1	Relationship between communication service availability, packet error ratio and survival time under assumption of independent packet errors

Note: The formula and KPIs listed in this section assume an ideal 5G system which is always up and running (i.e., no power outages, no system failures, etc.).

Figure 1 shows a sequence of transmissions over a logical link for a cyclic, distributed automation application (e.g., the link between a sensor and a motion controller). Periodic communication over the logical link is represented by a sequence of ticks (vertical lines). Each tick represents the point in time at which the target application (e.g., motion controller) expects to receive data (e.g., a sensor update) in each application cycle. A black tick denotes successful reception, whereas a white tick denotes a packet error or packet loss. For simplicity, packet errors and packet losses are not distinguished and simply referred to as packet errors. Red squares represent the periods of time during which the target application is not receiving data over the logical link due to packet errors (i.e., the packet was not correctly delivered within the deadline). Blue squares represent the periods of time during which the communication service is considered unavailable by the target application.

Figure 1. Consecutive packet errors on a logical link. Starting from the left side, the communication service is available until an error burst longer than the survival time (ST) occurs after a duration TTF1 (Time To Failure 1). The communication service remains unavailable for a duration TTR1 (Time To Recovery 1), until the first successful packet is received. And so on for TTF2, TTR2, etc.

A failure of the communication service occurs whenever m=TsurvivalTcycle+1 consecutive packet errors occur on the logical link, where Tcycle is the period of the application cycle; x is the largest integer that is smaller than or equal to x. A packet refers to a Protocol Data Unit (PDU) at the reference interface between application and communication system, e.g., an IP packet or an Ethernet frame. Here, an application message is assumed to fit into a PDU (i.e., messages are not fragmented). The communication service is “up” (i.e., fully restored) as soon as the first packet is correctly delivered after a failure. A survival time Tsurvival=2 Tcycle is assumed in Figure 1 (i.e., m=3). Thus, one isolated packet error and even up to 2 consecutive packet errors can be “survived” without problems by the target application. However, whenever 3 or more consecutive packet errors occur, the target application declares the communication service unavailable until the next packet is correctly received. In other words, the communication service failed.

Assuming packet errors occur independently with probability p, the communication service availability Alink of a logical link is given by (see Clause X.2 for proof)

Alink=1-pm=1-pTsurvivalTcycle+1,

where m is the number of consecutive packet errors that need to occur for the target application (e.g., motion controller) to declare the communication service unavailable.

Table 1 lists the values of p that lead to a communication service unavailability Ulink=1-Alink=10-6 for different values of m. The longer the survival time Tsurvival (i.e., larger m), the more relaxed becomes the packet error probability requirement.

Table 1. Required packet error probability (p) for Ulink=10-6 (Alink=99.9999%)

| Tsurvival | m | p |
| --- | --- | --- |
| 0 | 1 | 10-6 |
| Tcycle | 2 | 10-3 |
| 2 Tcycle | 3 | 10-2 |

Table 2 shows the relationship between packet error probability and communication service availability for m=2.

Table 2. Communication service availability for m=2

| Packet error probability p | Packet delivery probability (3GPP reliability as defined in TS 22.261) 1-p | Communication service unavailability Ulink=pm=pTsurvivalTcycle+1 (m=2) | Communication service availability Alink=1-pm=1-pTsurvivalTcycle+1 (m=2) |
| --- | --- | --- | --- |
| 10-3 | 99.9% | 10-6 | 99.9999% |
| 10-4 | 99.99% | 10-8 | 99.999999% |
| 10-5 | 99.999% | 10-10 | 99.99999999% |
| 10-6 | 99.9999% | 10-12 | 99.9999999999% |
| 10-7 | 99.99999% | 10-14 | 99.999999999999% |

## X.2	Derivation of relationship between communication service availability, packet error ratio and survival time

Note: The formula and KPIs listed in this section assume an ideal 5G system which is always up and running (i.e., no power outage, no system failures, etc.).

Note: The mean time between link failures (MTBFlink) in this section quantifies the communication service reliability (see subclause 3.1 and Annex C.2.2) under the assumption of independent packet errors.

The availability Alink of a logical link (i.e., communication service availability) can be computed as follows (see, e.g., [x5])

Alink=MTBFlinkMTBFlink+MTTRlink

where MTBFlink is the mean time between link failures and MTTRlink is the mean time to link recovery (expressed here, for simplicity, as the number of cycles instead of regular time units).

A link failure (failure of the communication service) occurs whenever m=TsurvivalTcycle+1 consecutive packet errors occur on the logical link, where Tcycle is the period of the application cycle. A link recovery (recovery of the communication service) occurs when the first packet is correctly delivered after a failure.

Let N denote the “time between link failures”. The mean time between link failures MTBFlink is related to the link reliability function ψn=P(N>n) (i.e., probability that there is no link failure in n cycles) as follows

MTBFlink=n≥0ψn

Assuming independent packet errors with error probability p, the link reliability function ψn obeys the following recursion

ψn=(1-p) ψn-1for m=1ψn=1-p(ψn-1+p ψn-2)for m=2ψn=1-p(ψn-1+p ψn-2+p2 ψn-3)for m=3⋮⋮

For arbitrary m,

ψn=(1-p)k=1mpk-1 ψn-k

Taking the sum for n≥m on both sides,

n≥mψn&=(1-p)k=1mpk-1 n≥mψn-kMTBFlink-n=0m-1ψn&=(1-p)k=1mpk-1 MTBFlink-n=0m-k-1ψn

Using the fact that n=0m-1ψn=m,

MTBFlink-m&=(1-p)k=1mpk-1 MTBFlink-(m-k)&=k=1mpk-1 MTBFlink-m+k-k=1mpk MTBFlink-m+k&=k=0m-1pk MTBFlink-m+k+1-k=1mpk MTBFlink-m+k&=MTBFlink-m+1+k=1m-1pk-pmMTBFlink

Thus,

MTBFlink&=1pmk=0m-1pk=1-pmpm(1-p)≈1pm

Similarly, let R denote the “time to link recovery”. In the absence of any active recovery measures, the mean time to link recovery MTTRlink is related to the probability φr=P(R>r) (i.e., probability that there is no link recovery in r cycles) as follows

MTTRlink&=r≥0φr&=r≥0pr&=11-p

where we have used the fact that φr=pr for r≥0, given the assumption of independent packet errors.

Combining MTBFlink and MTTRlink,

Alink=MTBFlinkMTBFlink+MTTRlink=1-pmpm(1-p)1-pmpm(1-p)+11-p=1-pm

Thus, the packet error ratio (p) requirement can be obtained from the communication service availability (Alink) requirement as follows

p≤m1-Alink

Note: The above inequality establishes an upper bound for the packet error ratio p, as indicated by the “≤” operator. In reality, the assumption of independent packet errors may not hold, in which case p will have to be significantly smaller than this upper bound.

### ------------------------- End of Change 3 ----------------------------

------------------------- END OF PROPOSED CHANGES ----------------------------