3GPP TSG-SA WG1 Meeting #85	S1-190156

Tallinn, Estonia, 18-22 February 2019	(revision of S1-19xxxx)

Title:	Use cases for multi-USIM devices

Agenda Item:	4 [MUSIM]

Source:				Qualcomm Incorporated

Contact:			Jack Nasielski   jackn @ qti.qualcomm.com

Abstract: Discuss and agree initial use cases for multi-USIM devices.

## 1	Introduction

This paper provides definitions, use cases and potential service requirements for devices that support multiple USIMs.

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Dual Rx: mode of operation of a UE in which the UE is capable of operation in dual receive (Rx) configuration in a given time instance.

Dual Tx: mode of operation of a UE in which the UE is capable of operation in dual transmit (Tx) configuration in a given time instance.

Single Rx: mode of operation of a UE in which the UE is only capable of operation (e.g., due to hardware or coexistence limitations) in a single Rx configuration in a given time instance.

Single Tx: mode of operation of a UE in which the UE is only capable of operation (e.g., due to hardware or coexistence limitations) in a single Tx configuration in a given time instance.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

MUSIM 	Multi-USIM

# x	Use Cases

## x.1	Emergency calls for multi-USIM devices

### x.1.1	Description

Isabel makes an emergency call with her MUSIM UE.

### x.1.2	Pre-conditions

Isabel has a smart phone with 2 active USIMs from different operators, labelled SIM1 and SIM2.  She has configured it to use SIM1 when she initiates a voice call.

### x.1.3	Service Flows

1. 	Isabel makes an emergency call using SIM1.

2.	Her UE tries all available RATs and voice domains to make the call.

3.	If the call fails on SIM1, her UE may use SIM2 and repeat emergency call procedures.

### x.1.4	Post-conditions

1. 	Emergency call is successful.

2.	If SIM1 was used to make call, SIM2 should not be used until emergency call procedures are complete.  Likewise, if SIM2 was used to make call, SIM1 should not be used until emergency call procedures are complete.

### x.1.5	Potential Service Requirements

Since emergency calls are mobile originated and require special handling, procedures for MUSIM UEs should be left to UE implementation.

When a MUSIM UE with 2 active USIMs makes an emergency call using one USIM, the other USIM should not be used until emergency call procedures are complete.

## x.1	Handling of service prioritization via UE configuration

### x.1.1	Description

MUSIM UEs exist today with various configurations to use multiple active USIMs.  Generally, the user specifies how each USIM will be used for voice, messaging, and cellular data services.   UE configuration may also allow USIMs to be prioritized for different services.  A particular USIM could be selected on a case by case basis, e.g., to select a USIM at each call origination, or configure a USIM for calls to specific contacts.  Configuration may also enable call forwarding from one USIM to the other in case the UE is engaged in a voice call when another call arrives.

### x.1.2	Pre-conditions

Isabel buys a new phone with MUSIM capabilities.  Her UE is capable of dual Rx and single Tx.  She has 2 active USIMs from different operators, labelled SIM1 and SIM2.  She is presented with menu options to configure her phone.

### x.1.3	Service Flows

1. 	Isabel chooses SIM1 as the default for voice and messaging services, and SIM2 as the default for cellular data.

2.	She can also make voice calls and send messages on SIM2 when the UE is not busy.  She has configured her device to forward incoming calls from SIM2 to SIM1 when she is busy on a voice call using SIM1.

### x.1.4	Post-conditions

1. 	Isabel enjoys services from 2 networks simultaneously.  Her device can also receive pages from both networks.

### x.1.5	Potential Service Requirements

Service priorities and configuration settings on a MUSIM UE should be set by the user.

A MUSIM UE should not allow one USIM to control or limit the capabilities of another USIM or allow one operator to restrict the use of another operator’s USIM.