3GPP TSG-SA1 Meeting #86	S1-191148

# Suzhou, China, 6th May 2019 - 10th May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.889 | CR | 0146 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update gap analysis of 12.7 builds stable positioning framework for FRMCS services and devices including trainborne and handheld devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Hansung University, ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_FRMCS2 |  |  |  |  |  | Date: |  |  | 2019-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The requirement [12.7-001] is covered by [R-5.11-002a] of TS 22.280 which is “The MCX Service shall be able to provide a mechanism for obtaining high accuracy Location information by integrating position information from multiple external sources (e.g. magnetometers, orientation sensors, GNSS)”. The requirement [12.7-002] is same as [R-12-14.2-001] which is “The consolidated positioning information i.e. the location information resulting from the combination of all positioning sources available to the FRMCS System shall support following absolute positioning accuracy based on the train speed at >= 90% of the occasions: 0 - 40km/h Longitudinal: ≤1 m lateral: 1 - 3 m Up to 120km/h Longitudinal: ≤34m lateral: 1 - 3 m Up to 160km/h Longitudinal: ≤45m lateral: 1 - 3 m Up to 320km/h Longitudinal: ≤89m lateral: 1 - 3 m Up to 500km/h Longitudinal: ≤200m lateral: 1 - 3 m”. It is covered by TS 22.261 sub-clause 6.27.2 and TS 22.261 sub-clause 7.3.2 – refer to positioning service level 3 (only one speed limit supported – up to 500 km/h – with horizontal accuracy of 1m)”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR Indicates the gap information on the requirement table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It makes working on the requirement which has been covered by other specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------ 1st changed section start ---------------------------------------

12.7.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [12.7-001] | The FRMCS System shall provide the alternative means than GNSS to obtain the position of the FRMCS Equipment. | A | TS 22.280 | The interfacing method and format of the location information from the LTE system is covered by 3GPP specification such as UE positioning in E-UTRAN[5]. Covered by [R-5.11-002a] of TS 22.280 which is “The MCX Service shall be able to provide a mechanism for obtaining high accuracy Location information by integrating position information from multiple external sources (e.g. magnetometers, orientation sensors, GNSS)”. |
| [12.7-002] | The positioning information shall provide an accuracy of [TBD] whilst the UE is travelling at a maximum of 500 km/h. | A/T | TS 22.261 | TS 22.261 sub-clause 6.27.2 TS 22.261 sub-clause 7.3.2 – refer to positioning service level 3 (only one speed limit supported – up to 500 km/h – with horizontal accuracy of 1m) See [R-12.14.2-001] of clause 12.14.2.5 |

------------------------------------------ 1st changed section end ---------------------------------------