3GPP TSG-SA WG1 Meeting #86 	S1-191181

Suzhou, China, 6 - 10 May 2019	(revision of S1-19xxxx)

Title:	FS_REFEC – KPIs section

Agenda Item:	8.6

Source:	NOVAMINT

Contact:	Thierry Bérisot, tberisot@novamint.com

Abstract: This document contains KPIS for IoT use cases for FS_REFEC.

---------- BEGIN OF CHANGE #1 ----------

## 6.4 Traffic Scenario: Container

### 6.4.1 Description

The description of the container use case is provided in 5.2.

### 6.4.2 Assumptions

For containers stored in a port or in a ship, it is assumed that most of the containers will not be able to communicate with the network directly due to the harsh environment and several hops will be needed to reach efficiently the containers able to communicate with the network.

In the figure 6.4.2-1, containers A, B, C, D, E, F, G and J cannot use WAN directly while containers K, I and  H are better positioned for WAN communication and can be used as ultimate relay to the network by the other containers. Containers C, D, F, G and J can be used as intermediate relay by A, B & E in multi hops

Figure 6.4.2-1: Communication between containers using multi Hop

In a typical case and as average, 2 to 3 hops are likely sufficient to provide a path to the network while in case of very dense area of containers stacked combined with very harsh environment, the use of 9 hops is foreseen as to be the upper maximum to be supported.

It is likely to have several relays able to communicate to the network and each of these ultimate relays could be used by several UEs which could be acting as well as intermediate relays. It is foreseen that to be efficient, a relay should have a capacity of relaying up to 4.000 UEs.

This will likely lead to the following topology.

Figure 6.4.2-2: Topology foreseen for container use case

The overall latency between the UE and the network using relay is not critical for this use case. A relay used for metering should support an operating range superior to 100 meter.

### 6.4.3 Potential New Functional Requirements

The requirements attached to this use case are provided in 5.2.

### 6.4.4 Potential Key Performance Requirements

The KPIs attached to this IoT use case are listed below:

| Scenario | Usage | Operating range of a relay UE | Number maximum messages per day | Message size | Overall latency between remote UE & last relay | Maximum Capacity of a relay | Average Number of Hops | Maximum number of hops |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Container | Indoor Outdoor Harsh Environment | > 100 m | 96 | 100 bytes | N/A | 4000 | 2-3 | 9 |

------------------------- END OF CHANGE #1 ----------------------------

---------- BEGIN OF CHANGE #2 ----------

## 6.5 Traffic Scenario: Wagon

### 6.5.1 Description

The description of the container use case is provided in 5.3.

### 6.5.2 Assumptions

TBD

### 6.5.3 Potential New Functional Requirements

The requirements attached to this use case are provided in 5.3.

### 6.5.4 Potential Key Performance Requirements

The KPIs attached to this IoT use case are listed below:

| Scenario | Usage | Operating range of a relay UE | Number maximum messages per day | Message size | Overall latency between remote UE & last relay | Maximum Capacity of a relay | Average Number of Hops | Maximum number of hops |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Wagon | Mostly Outdoor Indoor with Tunnel | > 100 m | 96 | 100 bytes | < 15s | 200 | 8-10 | 15 |

------------------------- END OF CHANGE #2 ----------------------------

---------- BEGIN OF CHANGE #3  ----------

Annex C: 
KPIs for IoT use cases

|  | Scenario | Usage | Operating range of a relay UE | Number maximum messages per day | Message size | Overall latency between remote UE & last relay | Maximum Capacity of a relay | Average Number of Hops | Maximum number of hops |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Container | Indoor Outdoor | > 100 m | 96 | 100 bytes | N/A | 4000 | 2-3 | 9 |
| 2 | Wagon | Mostly Outdoor Indoor with Tunnel | > 100 m | 96 | 100 bytes | < 15s | 200 | 8-10 | 15 |
| 3 | Smart Metering | Deep Indoor Indoor | > 100 m | 96 | 100 bytes | N/A | 10 | 2 | 5 |

Table C: KPIs for IoT Use cases

------------------------- END OF CHANGE #3 ----------------------------