3GPP TSG-SA WG1 Meeting #86 	S1-191371

Suzhou, China, 6 - 10 May 2019	(revision of S1-191120)

Title:	Multi-angle camera video production in dense user environments

Agenda Item:	8.2 FS_AVPROD: Feasibility Study on Audio-Visual Service Production

Source:	NICT

Contact:	Stanislav Filin, sfilin@nict.go.jp

Abstract: This contribution proposes an update to a “Single-source compressed Outside Broadcast Contribution” use case.

---------- Start Change ----------

## 5.6	Single- source compressed Outside Broadcast Contribution

### 5.6.1	Description

When working remotely from a broadcasting centre there is a requirement for producers, journalists and cameras crews to contribute audio, video and other data back to a broadcast centre for inclusion in a TV programme.

This is of a high quality using professional video cameras and other equipment. In some instances, this may use mobile phone technologies.

It may be a single live stream of content that is intended to go straight to air in a news programme. Or a longer OB such as a live sporting event.

In this use case compressed video may be used as it will go straight into a programme with little or no further signal processing. Speed is more important that quality in this use case.

There is also a use case for contribution and delivery of close to air or live content where uncompressed links are unavailable.

Ability to use 5G connectivity as a contribution feed of video, audio and ancillary data to a TV studio (e.g. news). We should be able to select content at standard definition (SD) (270 Mbit/s), High definition (HD 1920x1080 pixels (3 Gbit/s)) or UHD up to 24 Gbit/s. This should be possible up to a maximum frame rate of 120fps (25fps, 50 fps or 100fps for Europe). Various forms of compression may be used in order to provide a balance between quality, speed and available bandwidth. It should be noted that compression is usually on the video side and audio and data usually remain uncompressed.

To identify different bandwidth requirements, it may be helpful to look at different tiers of production and their compression and latency requirements.

Table 5.6.1-1: Compression tiers

| tier | Type of use | Bandwidth for UHD @50 FPS | Bandwidth for HD (1080i50) | Link latency | description |
| --- | --- | --- | --- | --- | --- |
| Mezzanine compression | Studio and high end production | 3 Gbit/s | 1 Gbit/s | <6 ms | Low complexity minimum compression for use in studio environments alongside existing studio resources or for contribution where uncompressed links are unavailable |
| Tier One | High profile sports or music events | 500 Mbit/s | 200 Mbit/s | <6 ms | compression used in live environment with no onward processing. Could be used for contribution |
| Tier 2 | Wide area sports events, general production | 100 Mbit/s | 80 Mb/s | <6 ms | Used when bandwidth is not available often mixed with wired sources, no onward processing |
| Tier 3 | news | >20 Mbit/s | 10 Mbit/s | 20 ms | Single camera contribution from a camera or smart phone |
| Reverse video/proxy | Monitoring and non-broadcast | 10 Mbit/s | <10 Mbit/s | <6 ms | Return video feed or lower quality feed for monitoring purposes |

The signal source will be derived from a single camera that gives out an uncompressed video feed this is then likely to be compressed before transmission via a 5G link. Different forms of compression may be required for different applications with low latency lightweight compression (e.g. VC2) used where quality needs to be maintained and increased compression used where (e.g. H264/5) used where bandwidth is extremely limited, and speed of contribution is required. For this use case we are concentrating on a single video source. Other use cases will explore using 5G for control of multiple cameras or sources without any on site facilities without compression.

There should be separate bi-directional audio links to enable the user at the remote site to communicate to the broadcast centre separate to the main video and audio feed.

On occasion it is necessary for the camera operation to be controlled remotely from the broadcast centre. This application may not require full quality uncompressed video for monitoring but does require extremely low round trip latency (<10 ms) in order to get accurate control of a remote camera. Functions are usually controlled via a standard protocol and functions include focus, zoom and orientation of the camera.

On occasion it may be necessary for the contribution to be mobile such as on a moving vehicle with speeds up to 120 MPH (200 km/h) such as on a train.

The limitations of current mobile or cellular bonded technologies means that the most common current user for this type of technology is within news where a single camera is used to contribute content back to a control room and from there into a production gallery for inclusion into a news programme.

As this user case concentrates on contribution quality is more important than latency. Whilst it is desirable for latency to approach <10ms higher figures up to around 2 seconds are acceptable.

5G connectivity could be provided by PN(s) or a combination of PN(s) and NPN(s)

If the area of an event is large, some parts of this area will be covered only by PN(s), while some parts also by NPN(s).

During the event mobile cameras may have to follow participants, which means that they could move from an area with only PN(s) coverage to the areas with both PN(s) coverage and NPN(s) coverage and back.

Cameras have physical capability to connect to both PN(s) and NPN(s) at the same time due to multiple RFs and subscriptions.

### 5.6.2	Pre-conditions

-	Uncompressed video signal.

-	Software or hardware-based compression tools to provide compressed video feed.

-	Video transport signal carried as an IP stream in SMPTE ST2110-20 (compliant to an ST2110-21 wide sender profile).

-	Audio transport signal carried as an IP stream in SMPTE ST2110-30 / AES67.

-	Devices compliant to the AMWA NMOS specifications IS-04 and IS-05 APIs.

-	AMWA NMOS IS-06 API between broadcast controller and a controller of the 3GPP infrastructure.

-	Suitable signal processing hardware or Facilities to provide a single video feed from multiple sources (OB truck).

-	5G transmission hardware (network interface).

-	One or several 5G PN(s) and one or several 5G NPN(s).

-	Video cameras are equipped with multi-RF UEs subscribed to 5G PN(s) and 5G NPN(s) and capable of connecting to more than one 5G PN or NPN at a time.

-	Signal path to 5G base station or cell.

-	Suitable connection to network at the receive point or broadcast centre.

-	Ability to uplink content from the broadcast centre to the field an added benefit.

-	Monitoring tools to identify signal path and quality of service.

### 5.6.3	Service Flows

-	Journalist turns on local UE.

-	Signal is routed to the network interface (journalist).

-	Establish link between network interface and 5G network (journalist).

-	Identify available bandwidth (cam engineer).

-	Configure signal processing to optimise against available bandwidth (engineer).

-	Send signal to base station (journalist).

-	Route signal to broadcast centre (automated).

-	Broadcast centre receives and monitors signal (engineer).

-	Broadcast centre sends back communications including clean feed and talk back (usually audio only) but may have compressed video (engineer).

-	Video signal is routed live to air via production gallery (director).

-	Mobile cameras are following some parts of the event transmitting uplink video streams using one or multiple connections to PN(s) and NPN(s) using multiple UEs and subscriptions.

-	While moving, UEs of these cameras do handover between cells of PN(s) and NPN(s) and adaptively redistribute uplink traffic among multiple uplink streams.

### 5.6.4	Post-conditions

Final production programme output to go to air either via 5G network or other directly connected hand-off.

Using multiple links to PN(s) and NPN(s) increases video quality and time duration during which video could be uploaded with required QoS in case of mobile cameras.

### 5.6.5	Existing features partly or fully covering the use case functionality

The 5G system shall support the provision of elemental flows of synchronous video, audio and data

feeds in a transparent way.

The 5G system shall support application layer protocols for discovery, registration, connection management, network control and security compatible with current broadcast best practice (NMOS).

The 5G system shall be able to transport IPv4 and IPv6 multicast traffic.

The 5G system shall be able to transport media RTP based streams of media.

The 5G system shall be able to transport IP-Packets with an MTU of 1500 bytes.

The 5G system shall support the deployment of a live content production network as an NPN providing coverage within the specific event area.

The 5G system shall support the standalone operation of a live content production network deployed as an NPN.

The 5G system shall support identifiers to uniquely identify an NPN.

### 5.6.6	Potential New Requirements needed to support the use case

Video link bandwidth shall support compressed video up to UHD 4K at 50 fps as per table below.

Table 5.6.6-1: Potential new requirements.

| Use Case | mobility | Uni or bi directional | Downlink data rate per UE | Uplink data rate per UE | link latency (note 2,3) | Packet size (bytes) | Total delay inc. application | Reliability (note 1) | # active of UE | Service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [PR 5.6.6-001] Mezzanine compression UHD video | stationary | bi | 20 Mbit/s | 3 Gbit/s | < 1sec | 1500 | <1.5 sec | QEF | <5 | 1000m2 |
| [PR 5.6.6-002] Mezzanine compression HD video | stationary | bi | 20 Mbit/s | 1 Gbit/s | < 1sec | 1500 | <1.5 sec | QEF | <5 | 1000m2 |
| [PR 5.6.6-003] Tier one events UHD | stationary | bi | 20 Mbit/s | 500 Mbit/s | <1 sec | 1500 | <1.5 sec | QEF | <5 | 1000m2 |
| [PR 5.6.6-004] Tier one events UHD | stationary | bi | 20 Mbit/s | 200 Mbit/s | <1sec | 1500 | <1.5 sec | QEF | <5 | 1000m2 |
| [PR 5.6.6-005] Tier two events UHD | slow (< 7km/h) | bi | 20 Mbit/s | 100 Mbit/s | <1 sec | 1500 | <1.5 sec | QEF | <5 | 1000m2 |
| [PR 5.6.6-006] Tier two events HD | slow (< 7km/h) | bi | 20 Mbit/s | 80 Mbit/s | <1 sec | 1500 | <1.5 sec | QEF | <5 | 1000m2 |
| [PR 5.6.6-007] Tier three events UHD (note 6) | slow to high (<7km/h to <200 km/h) | bi | 10 Mbit/s | <20 Mbit/s | <1 sec | 1500 | <1.5 sec | QEF | <10 | 1000m2 |
| [PR 5.6.6-008] Tier three events HD (note 6) | slow to high (<7 km/h to <200 km/h) | bi | 10 Mbit/s | 10 Mbit/s | < 1sec | 1500 | <1.5 sec | QEF | <10 | 1000m2 |
| Note 1: The 5G system shall be able to deliver Quasi Error Free (QEF) services, meaning less than one uncorrected error event per hour at the input of the video decoder. One or more retransmissions of network layer packets may take place in order to satisfy the reliability requirement. Note 2: This is the maximum end-to-end latency allowed for the 5G system to deliver the service in the case the end-to-end latency is completely allocated to the 5G system from the UE to the Interface to Data Network. Note 3: Latency is less important than QEF, longer latency can be accepted to avoid errors (e.g. existing satellite delays can be up to 5 seconds). Note 4: Use case is more likely to be deployed over a point to point dedicated link. Note 5: Bandwidth to include video, audio and data on a single link. Note 6: Could use either professional equipment or mobile phone equipped with dedicated newsgathering app (SMPTE requirements do not apply). |  |  |  |  |  |  |  |  |  |  |

Editor’s note: The desire is to extend the extend the existing functionality of modern broadcast networks, as described in [3], out to remote devices connected via a 5G network. Therefore, the following requirements are critical to the adoption of 5G technologies (also applicable to the uncompressed use case).

[PR 5.6.6-009]: The 5G system shall be able to distribute high-precision timing information as per the Precision Time Protocol (IEEE 1588-2008) in SMPTE ST2059 profile. To enable the synchronous timing of broadcast devices over the network and to provide data to enable the use of a UE media clock.

Editor’s note: confirmation required as to IEEE 802.1A suitability over IEEE 1588 with SMPTE 2059 profile.

[PR 5.6.6-010]: The 5G system shall have the ability to support open standard based broadcast workflows.

Editor’s note: VSF and SMPTE and exploring the requirements for SMPTE compatible video over wireless links.

Option 1:

[PR 5.6.6-011]: The 5G system shall support seamless service continuity for an uplink stream as described in Table 5.6.6-1 while maintaining multi-link connectivity with co-located PN(s) and NPN(s) and switching between them.

Option 2:

[PR 5.6.6-011]: The 5G system shall support seamless service continuity for an uplink stream as described in Table 5.6.6-1 while switching between co-located PN(s) and NPN(s).

---------- End Change ----------