3GPP TSG-SA WG1 Meeting #86 	S1-191564

Suzhou, China, 6 - 10 May 2019	(revision of S1-191332, S1-191180)

Title:	FS_REFEC – Metering use case

Agenda Item:	8.6

Source:	NOVAMINT

Contact:	Thierry Bérisot, tberisot@novamint.com

Abstract: This document contains the metering use case for FS_REFEC.

## 6.2 Traffic Scenario: Metering use case

### 6.3.1 Description

When deploying IoT metering solutions, utilities (gas, water, electricity…) are faced with the critical issue of Network ”black spots”. Some meters are very deep indoor and their coverage could not be assured by the network provider. These meters are likely not to be able to reach the eNB in a single hop. In some city, it could represent more than 5 % of the UEs deployed.

Furthermore, many meters that have coverage are at the edge of a cell and a subject to many repetitions. To be able to communicate, these meters need to increase drastically their battery consumption by emitting at full power. And therefore their battery life is strongly impacted and reduced – 2 or 3 years’ battery life instead of 15 years required for the service.

Enhanced relays will play a key role in achieving extensive coverage as well as better energy efficiency in smart metering scenarios.

### 6.3.2 Assumptions

For IoT there are different coverage enhancement (CE) levels depending on the different radio conditions associated to the number of repetitions possible (CE level 0 to CE level 2 for NB-IoT for example, 4 CE levels in eMTC).

It is assumed that up to 5 hops will be needed to provide indirect network connection to out of coverage meters and up to 3 hops for meters at the edge of the cell in EC level 2.

The figure 6.3.2-1 is illustrating the different scenarios possible for metering for different CE levels.

Figure 6.3.2-1: Smart Metering

In the figure, meters A & D are well positioned in CE level 0 while Meter B is very deep indoor and cannot communicate directly. Meter B could use Meter A as Relay. Meter C can communicate but battery is largely affected by more retransmission (CE Level 2). Meter C could use Meter A as well as Relay.

It is assumed that up to 5 hops will be needed to provide indirect network connection to out of coverage meters and up to 3 hops for meters at the edge of the cell in CE level 2.

The overall latency between the UE and the network using relay is not critical for this use case.

A relay should be able to relay 10 devices as an overall.

A relay used for metering should support an operating range superior to 100 m to be able to relay meters that are deep indoor.

### 6.3.3 Potential New Functional Requirements

The 5G system shall support that an IoT device can establish an indirect network connection to the eNB even when a direct network connection is available (Note: this could be restricted to some UEs).

### 6.3.4 Potential Key Performance Requirements

The KPIs attached to this IoT use case are listed below:

| Scenario | Usage | Operating range of a relay UE | Number maximum messages per day | Message size | Overall latency between remote UE & last relay | Maximum Capacity of a relay | Average Number of Hops | Maximum number of hops |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Smart Metering | Deep Indoor Indoor | [> 100 m] | [96] | [100] bytes | N/A | [10] | [2-3] | [5] |