3GPP TSG-SA WG1 Meeting #86 	S1-192008

Sophia Antipolis, France, 19 - 23 Aug 2019	(revision of S1-19xxxx)

Title:	Service continuity requirements for video production

Agenda Item:	8.2 AVPROD: Audio-Visual Service Production

Source:	NICT

Contact:	Stanislav Filin, sfilin@nict.go.jp

Abstract: This contribution proposes to add a table and two service continuity requirements from consolidated requirements in TR 22.827 to TS 22.263 Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA).

---------- Start Change ----------

It is proposed to add the following table and the following two requirements to a suitable parts of TS 22.263 Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA).

Table x.y: Performance requirements for low latency video.

| Profile | # of active UEs | UE Speed | Service Area | E2E latency | Packet error rate (Note 1) | Data rate UL | Data rate DL |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Uncompressed UHD video | 1 | 0 km/h | 1 km2 | 400 ms | 10-8% UL 10-5% DL | 12 Gbit/s | 20 Mbit/s |
| Uncompressed HD video | 1 | 0 km/h | 1 km2 | 400 ms | 10-7% UL 10-5% DL | 3 .2 Gbit/s | 20 Mbit/s |
| Mezzanine compression UHD video | 5 | 0 km/h | 1000 m2 | 1 s | 10-7 % UL 10-5 % DL | 3 Gbit/s | 20 Mbit/s |
| Mezzanine compression HD video | 5 | 0 km/h | 1000 m2 | 1 s | 10-7 % UL 10-5 % DL | 1 Gbit/s | 20 Mbit/s |
| Tier one events UHD | 5 | 0 km/h | 1000 m2 | 1 s | 10-7 % UL 10-5 % DL | 500 Mbit/s | 20 Mbit/s |
| Tier one events UHD | 5 | 0 km/h | 1000 m2 | 1 s | 10-6 % UL 10-5 % DL | 200 Mbit/s | 20 Mbit/s |
| Tier two events UHD | 5 | 7 km/h | 1000 m2 | 1 s | 10-6 % UL 10-5 % DL | 100 Mbit/s | 20 Mbit/s |
| Tier two events HD | 5 | 7 km/h | 1000 m2 | 1 s | 10-6 % UL 10-5 % DL | 80 Mbit/s | 20 Mbit/s |
| Tier three events UHD (Note 2) | 5 | 200 km/h | 1000 m2 | 1 s | 10-5 % UL 10-5 % DL | 20 Mbit/s | 10 Mbit/s |
| Tier three events HD (Note 2) | 5 | 200 km/h | 1000 m2 | 1 s | 10-5 % UL 10-5 % DL | 10 Mbit/s | 10 Mbit/s |
| Remote OB | 5 | 7 km/h | 1000 m2 | 6 ms | 10-6 % UL 10-5 % DL | 200 Mbit/s | 20 Mbit/s |
| NPN ground to air UHD up Link | 10 | 500 km/h | 700 km2 x 6000 m (Note 3) | 40 ms | 10-6 % UL 10-5 % DL | 100 Mbit/s | 20 Mbit/s |
| NPN ground to air HD up link | 10 | 500 km/h | 700 km2 x 6000 m (Note 3) | 40 ms | 10-6 % UL 10-5 % DL | 80 Mbit/s | 20 Mbit/s |
| NPN air to ground UHD down Link | 2 | 500 km/h | 700 km2 x 6000 m (Note 3) | 40 ms | 10-5 % UL 10-6 % DL | 20 Mbit/s | 100 Mbit/s |
| NPN air to ground HD down link | 2 | 500 km/h | 700 km2 x 6000 m (Note 3) | 40 ms | 10-5 % UL 10-6 % DL | 20 Mbit/s | 80 Mbit/s |
| NPN radio Camera UHD | 10 | 200 km/h | 1 km2 | 3 ms | 10-6 % UL 10-5 % DL | 100 Mbit/s | 20 Mbit/s |
| NPN radio camera HD | 10 | 200 km/h | 1 km2 | 3 ms | 10-6 % UL 10-5 % DL | 80 Mbit/s | 20 Mbit/s |
| NOTE 1: Packet error rate calculated considering 1500 B packets, and 1 error per hour as 𝐏𝐄𝐑=(𝟖 ∗𝒑𝒂𝒄𝒌𝒆𝒕 𝒔𝒊𝒛𝒆)⁄(𝒅𝒂𝒕𝒂 𝒓𝒂𝒕𝒆 𝟑𝟔𝟎𝟎). NOTE 2: Could use either professional equipment or mobile phone equipped with dedicated newsgathering app NOTE 3: 6000 m = height but in a cone formation (i.e. ground coverage with a circle of diameter 30 KM) |  |  |  |  |  |  |  |

The 5G system shall support seamless uplink and downlink service continuity as described in Table x.y while switching between co-located PLMN and NPN (e.g., due to mobility).

The 5G system shall support seamless service continuity as described in Table x.y: for an uplink stream while performing traffic steering, switching, and splitting among co-located PLMN(s) and NPN(s); for downlink while switching between co-located PLMN and NPN.

---------- End Change ----------