3GPP TSG-SA WG Meeting #87	S1-192024 (was xxx)

Sophia Antipolis, France, Aug. 19 - 23, 2019

Title:		Update of Section 5.8

Agenda Item:	FS_MINT

Source:	LG Electronics, KT Corp, LG Uplus

Contact:	sungduck.chun@lge.com

Abstract: This contribution proposes to update section 5.8

Discussion

Section 5.8 does not yet list potential requirements.

After disaster occurred, an impacted PLMN will eventually recover. However, at initial phase, the recovered PLMN may not be fully functional. E.g., after recovered, the PLMN may initially use small portion of its designed capacity and gradually increase its usage up to 100%.

If all subscribers of the PLMN start registration to the recovered PLMN simultaneously, the generated signaling spike may be beyond what the PLMN can handle. If the PLMN applies access control to avoid congestion, a lot of UEs who have not yet successfully finished registration to HPLMN may be stuck in limited service state. Thus, unconditional quick return to its HPLMN after the end of the disaster should be avoided.

Proposal

It is proposed to agree on the following text proposal

********************** FIRST CHANGE*****************

## 5.8	Return to Home Network after disaster period

### 5.8.1	Description

During disaster, UEs of impacted network are served by other neighboring network. When the impacted network recovers and goes into normal mode of operation, UEs that are temporarily served by other neighboring network need to forward back to its home network.

In this case, abrupt release of connection by neighboring network should be avoided. Otherwise, this may lead to bad service experience. For example, ongoing emergency call should not be released. In addition, releasing all UEs simultaneously will cause signaling surge and this may again cause network congestion and break-down. Also, the time gap between connection release by one network and connection establishment toward other network should be minimized, to enhance user experience.

Because these UEs are temporarily serviced by other networks, there is no need to support service continuity when the UEs returns to their home network.

### 5.8.2	Pre-conditions

Operation of HNetwork is interrupted by disaster. Users of HNetwork are served by NNetwork until HNetwork recovers from the impact of disaster.

### 5.8.3	Service Flows

Following is one example service flow:

1.	HNetwork finalizes fixing its network equipment and is ready to accept its own UEs.

2.	HNetwork informs NNetwork that it is ready to accept its UEs..

3.	For UEs in connected mode, to decide when to release connection of the UEs of HNetwork, NNetwork may consider the characteristic of ongoing communication. E.g., it may not immediately release a connection of the UE with active voice call. It is assumed that HNetwork and NNetwork do not support inter-PLMN handover..

4.	To make the switch from NNetwork to HNetwork faster, NNetwork may give some assistance information to the UEs. Based on this information, the UE may quickly find HNetwork.

5.	For UEs in Idle mode, NNetwork may use resource efficient means to inform the UEs that the HNetwork is recovered from disaster.

6.	From the moment when the UE leaves NNetwork to the moment when the UE successfully is registered in HNetwork, the UE cannot get any connectivity service. Thus, NNetwork may control when Idle mode UEs and Connected mode UEs leave NNetwork.

### 5.8.4	Post-conditions

All UEs temporarily served by NNetwork are gradually served by HNetwork.

### 5.8.5	Existing features partly or fully covering the use case functionality

TS 24.501 and TS 24.401 defines NAS procedure to support

Also, TS 36.331 and TS 38.331 describes RAN level information.

### 5.8.6	Potential New Requirements needed to support the use case

[R.5.8-001]	The 3GPP system shall be able to provide means for a network to provide in-advance notification of release of connection to a UE with active communication, when the PLMN intend to release the connection due to the end of Disaster condition.

[R.5.8-002]	The 3GPP system shall be able to support means to minimize time period from when the a UE leaves the other PLMN to when the UE successfully registers to its HPLMN, after Disaster condition becomes no longer applicable.

[R.5.8-003]	The 3GPP system shall be able to provide efficient means for a network to inform Disaster Inbound roamers that Disaster condition is no longer applicable.