3GPP TSG-SA WG1 Meeting #86 	S1-192152

Sophia Antipolis, France, 19 - 23 Aug 2019

Title:	pCR to TR 22.827 on New Use Case for Video Production with Remote Production Center

Agenda Item:	7.1 AVPROD: Audio-Visual Service Production

Source:	Tencent

Contact:	Lei Yixue, yixuelei@tencent.com

Abstract: This contribution proposes new use case for movie making with remote production center with a new potential requirement.

---------- Change#1 ----------

5.x  Video Production with Remote Prodction Centre

5.x.1 Description

Professional video production e.g. movie making is a large potential market for 5G as 5G can be used to transmit the video and audio content from local area towards the production centre.  This allows simplified and portable local equipment to be used and the computation tasks like rendering are not performed in local area.  However, to support such use case, 5G uplink need to support high data rate to transport the audio and video content which may be raw data or uncompressed.  In other words, this use case intends to utilize 5G network to move the complex computation tasks like rendering towards the production center e.g. in the cloud side.

5.x.2 Pre-conditions

5G network, either standalone NPN or NPN integrated with MNO are deployed in available in local area.

Suitable spectrum e.g. mmWave band is allocated/configured so that high speed uplink data rate can be supported.

AVProd devices in local area can be connted with the Video production center via 5G network with sufficient radio and wired resources.

5.x.3 Service Flows

- AVProd devices capture the audio video raw data and transmit to the production center with high speed uplink.  Since 8K and 4K video are becoming more and more popular for movie making and the FPS is also increasing e.g. from lower than 60 FPS towards 90 FPS and even high, it is estimated that 1 – 3 Gbps uplink data rate is needed when uncompressed data is needed to be transmitted to the production center.

- Video production are performed within the centre.

5.x.4 Post-conditions

After the video production, video is shown in the cinema or online.

5.x.5 Existing features partly or fully covering the use case functionality

5.x.6 Potential Requirements

[PR-5.x.6-001] 5G system should be able to support 1 to 3 Gbps in uplink direction to transmit uncompressed data to remote production center for professional video production.

| Table 5.14.6-1: Potential new requirements.Use Case | mobility | Uni or bi directional | Downlink data rate per UE | Uplink data rate per UE | Max Packet size | link latency | Total delay (inc. application) | reliability | # of active UE | Service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [PR 5.14.6-001] Movie Making with Remote Production Center | <200 km/h | Uni Note 1 |  | 1 - 3 Gbit/s |  |  |  |  | 5-10 | 1000m x 1000m |

Note 1: In uplink direction, the data rate is very high but in downlink direction, there may be corresponding packets e.g. ack for uplink traffic but would not bring high volum traffic load.

---------- Change#2 ---------

8	Consolidated potential requirements

8.1 Performance requirements

Table 8.1-3 Performance requirements for movie making with remote production center

| Use Case | mobility | Uni or bi directional | Downlink data rate per UE | Uplink data rate per UE | Max Packet size | link latency | Total delay (inc. application) | reliability | # of active UE | Service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [PR 5.x.6-001] Movie Making with Remote Production Center | <200 km/h | Uni Note 1 |  | 1 - 3 Gbit/s |  |  |  |  | 5-10 | 1000m x 1000m |

---------- End Change ----------