3GPP TSG-SA WG1 Meeting #86 	S1-192154

Sophia Antipolis, France, 19 - 23 Aug 2019

Title:	Clarification Multiple Working Clock Domains in gNB and pCR to TR 22.832

Agenda Item:	7.9  eCAV

Source:	Tencent

Contact:	Lei Yixue, yixuelei@tencent.com

Abstract: This contribution proposes to add an editor note regarding to the potential requirement [PR-5.5.6-001].

In Section 5.3 of TR 22.832, the use case for Multiple Working Clock Domains in gNB are provided and we believe this is a very important use case.  However, current [PR-5.3.6-002] says:

The 5G System shall be able to support up to 128 synchronization domains (with different synchronization domain identifiers / domain numbers).

In our view, all the description of this use case, the requirement is for the gNB as radio access node. Thus, if in this requirement we think it should be made clear otherwise there would be misunderstanding that one gNB can support less than 128 clock domains but via gNBs in different area 128 clock domains can be supported.  In such case, it seems the requirement is met but obviously this is not the intention of this use case and also the proposed requirement.  Therefore, we proposed to make this requirement more specific i.e. one gNB not core network or 5G system as a whole.  We don’t have strong opinion about the exact text thus propose a tentative rewording and also propose to add an editor note for further consideration.

The proposed changes are as follows:

---------- Change#1 ----------

5.3	Multiple Working Clock Domains in gNB

5.3.1	Description

Time-sensitive networking (TSN) will be used in industrial automation (e.g. for control loops in industrial communication networks on the factory floor) and in energy automation (e.g. for communication networks in substations). 5G networks with TSN capabilities will be integrated in such kind of industrial communication networks.

Working clock domains are TSN synchronization domains with a common time synchronization. A working clock domain includes all the devices in a local area that need to cooperate in the physical world, e.g. robots, robot arms, AGVs, conveyor belts, machines, etc. Working clock domains are structured according to the production process. A working clock domain has an arbitrary time scale (starting at 0 and counting) and a precision of ±1 µs. For a description of working clock domains see [2].

A gNB usually serves several UEs. A common deployment is a general 5G network infrastructure serving the factory floor without any mapping to the structure of the production processes (machine – cell – production line – factory). This leads to the situation that multiple working clock domains communicate through the same gNB (see Figure 5.3.1-1).

Figure 5.3.1-1: Multiple working clock domains communicating through the same gNB

In the working clock domains A..E, there might be a non-3GPP TSN network behind a UE. The sync master of the working clock domain is inside the working clock domain and may be in the non-3GPP TSN network behind on of the corresponding UEs (see working clock domains A and D in Figure 5.3.1-1) or on one of the UEs (see working clock domains B and C in Figure 5.3.1-1).

5.3.2	Pre-conditions

The 5G network with TSN / time synchronisation support is set up in an area of the factory. Multiple working clock domains are configured and initialized. Each working clock domain contains at least one UE served by the same gNB. The working clock domains are independent from each other, especially with respect to time scale and precision of time synchronization.

It can be assumed that network planning avoids spatial reuse of working clock domain identifiers at the same gNB.

5.3.3	Service Flows

TSN messages for time synchronization are sent within the working clock domains, including the path UE x1 – gNB 1 – UE x2 (with x being A,B,C, D, or E). gNB 1 needs to handle the TSN messages from different working clock domains (see Figure 5.3.3-1).

Figure 5.3.3-1: Multiple working clock domains to be handled at gNB

5.3.4	Post-conditions

Each working clock domain is time synchronized independently from the other working clock domains with the required precision.

5.3.5	Existing features partly or fully covering the use case functionality

The 5G system shall support a mechanism to process and transmit IEEE1588v2 / Precision Time Protocol messages to support 3rd-party applications which use this protocol. [2]

The 5G system shall support a mechanism to synchronize the user-specific time clock of UEs with a working clock. [2]

The 5G system shall support networks with up to 32 working clock domains. [2]

The 5G system shall support at least 2 simultaneous working clock domains on a UE. [2]

The working clock domains shall provide time synchronization with precision of ≤ 1 μs. [2]

NOTE 1:	The required precision of ≤ 1 μs is between the sync master and any device of the clock domain.

NOTE 2:	Different working clock domains are independent and can have different precision.

5.3.6	Potential New Requirements needed to support the use case

[PR-5.3.6-001]	The 5G System shall be able to support up to the maximum number of working clock domains for UEs connected through a 5G network.

NOTE 1:	The maximum number of working clock domains is 32 in TS 22.104 [2] v16.1.0. The domain number (synchronization domain identifier) is defined with one octet (unsigned integer) in IEEE 802.1AS [3]. This allows for 128 synchronization domains (1 global time domain and 127 working clock domains).

[PR-5.3.6-002]	The 5G System shall be able to support up to 128 synchronization domains (with different synchronization domain identifiers / domain numbers) at radio access level.

Editor note: It is necessary to make clear that this requirement need to be met by gNB and the exact wording can be further considered.

---------- End of Change ----------

Table 5.11.6-1: KPIs for ProSe communication for cyber-physical control applications

| Use case | Characteristic parameter |  |  |  |  |  | Influence quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5.11 – Cooperative carrying of work pieces | Communication service availability: target value in % | Communication service reliability: Mean Time Between Failure | End-to-end latency: maximum | Bit rate | Direction | Message Size [byte] | Transfer interval [ms] | Survival time [ms] | UE speed [km/h] | # of UEs connection | Service Area [m3] (note 3) |
| ProSe communication for cyber-physical control application | [] | [] | [2 ms] | [] | bi-directional; direct | [] |  | [] | UE-RAN: [20] UE-UE: 0 | 4-8 | 10 x 10 x 5; 50 x 5 x 5 |
| Motion control | 99,999 % to 99,99999 % | ~ 10 years | < transfer interval value | [] | bi-directional; direct | 50 | 0,5 | 500 μs | UE-RAN: [20] UE-UE: 0 | 4-8 | 10 x 10 x 5; 50 x 5 x 5 |
| Motion control | 99,9999 % to 99,999999 % | ~ 10 years | < transfer interval value | [] | bi-directional; direct | 40 | 1 | 1 ms | UE-RAN: [20] UE-UE: 0 | 4-8 | 10 x 10 x 5; 50 x 5 x 5 |
| Motion control | 99,9999 % to 99,999999 % | ~ 10 years | < transfer interval value | [] | bi-directional; direct | 20 | 2 | 2 ms | UE-RAN: [20] UE-UE: 0 | 4-8 | 10 x 10 x 5; 50 x 5 x 5 |
| Mobile robots | 99,9999 % | ~ 10 years | < transfer interval value | [] | bi-directional; direct | 40 to 250 | 1 ms to 50 ms (note 1) (note 2) | transfer interval value | UE-RAN: [20] UE-UE: 0 | 4-8 | 10 x 10 x 5; 50 x 5 x 5 |
| NOTE 1: This covers different transfer intervals for different similar use cases with target values of 1 ms, 1 ms to 10 ms, and 10 ms to 50 ms. NOTE 2: The transfer interval deviates around its target value by < ±25 %. NOTE 3: Service Area for direct communication. The group of UEs with direct communication might move throughout the whole factory site (up to several km²) |  |  |  |  |  |  |  |  |  |  |  |

Editor’s note: KPIs in Table 5.11.6-1 are for further study.

---------- End Change ----------