3GPP TR 22.831 V0.1.1 (2019-08)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Study on Support for Minimization of Service Interruption

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

LTE, 5G, PLMN, Roaming

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Overview	6

5	Use cases	7

5.1	Network reselection during disaster - General	7

5.1.1	Description	7

5.1.2	Pre-conditions	7

5.1.3	Service Flows	7

5.1.4	Post-conditions	7

5.1.5	Existing features partly or fully covering the use case functionality	8

5.1.6	Potential New Requirements needed to support the use case	8

5.2	A well-handled disaster begins	8

5.2.1	Description	8

5.2.2	Pre-conditions	8

5.2.3	Service flows	8

5.2.4	Post-conditions	9

5.2.5	Existing features partly or fully covering the use case functionality	9

5.2.6	Potential New Requirements needed to support the use case	9

5.3	Use case for HPLMN failure with part of CN still working	9

5.3.1	Description	9

5.3.2	Pre-conditions	9

5.3.3 	Service flow	9

5.3.4	Post-conditions	10

5.3.5	Existing features partly or fully covering the use case functionality	10

5.3.6	Potential New Requirements needed to support the use case	10

5.4	Network reselection during disaster - Non-disaster area	10

5.4.1	Description	10

5.4.2	Pre-conditions	11

5.4.3	Service Flows	11

5.4.4	Post-conditions	11

5.4.5	Existing features partly or fully covering the use case functionality	11

5.4.6	Potential New Requirements needed to support the use case	11

5.5	Network reselection during disaster with previous reject	12

5.5.1	Description	12

5.5.2	Pre-conditions	12

5.5.3	Service Flows	12

5.5.4	Post-conditions	12

5.5.5	Existing features partly or fully covering the use case functionality	12

5.5.6	Potential New Requirements needed to support the use case	13

5.6	Network protection for surge of incoming users during disaster	13

5.6.1	Description	13

5.6.2	Pre-conditions	13

5.6.3	Service Flows	13

5.6.4	Post-conditions	14

5.6.5	Existing features partly or fully covering the use case functionality	14

5.6.6	Potential New Requirements needed to support the use case	14

5.7	Use case on access control for “backup” PLMN	14

5.7.1	Description	14

5.7.2	Pre-conditions	14

5.7.3 	Service flow	14

5.7.4	Post-conditions	14

5.7.5	Existing features partly or fully covering the use case functionality	15

5.7.6	Potential New Requirements needed to support the use case	15

5.8	Return to Home Network after disaster period	15

5.8.1	Description	15

5.8.2	Pre-conditions	15

5.8.3	Service Flows	15

5.8.4	Post-conditions	15

5.8.5	Existing features partly or fully covering the use case functionality	15

5.8.6	Potential New Requirements needed to support the use case	16

5.9	A well-handled disaster ends	16

5.9.1	Description	16

5.9.2	Pre-conditions	16

5.9.3	Service flows	16

5.9.4	Post-conditions	16

5.9.5	Existing features partly or fully covering the use case functionality	16

5.9.6	Potential New Requirements needed to support the use case	16

5.10	Use case on “backup” PLMNs for the international roamer	16

5.10.1	Description	16

5.10.2	Pre-conditions	17

5.10.3 	Service flow	17

5.10.4	Post-conditions	17

5.10.5	Existing features partly or fully covering the use case functionality	17

5.10.6	Potential New Requirements needed to support the use case	17

6	Potential New Consolidated Requirements	18

7	Conclusions and Recommendations	18

Annex <A>: <Annex title>	19

A.1	Heading levels in an annex	19

Annex <X>: Change history	21

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document …

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Disaster Condition: This is the condition that a government decides whether it is applicable or not based on, e.g. a natural disaster or a man-made disaster. When this condition applies, users may have the opportunity to mitigate service interruptions and failures.

Disaster Roaming:  This is the special type of roaming service that applies during a Disaster Condition.

Disaster Inbound Roamers: Users that (a) cannot get service from the PLMN they would normally be served by, due to failure of service during a Disaster Condition, and (b) are able to register with other PLMNs.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

4	Overview

Editor’s Note: This clause provides a high-level overview of the feature. This section will be updated based on the study outcome.

5	Use cases

5.1	Network reselection during disaster - General

5.1.1	Description

When a UE is located within its home country, the UE camps on its home network. In normal situation where its home network is functioning properly, the neighboring network will reject any service request from the UE except the case when it is required by regulation e.g. for emergency call.

However, when the home network is disabled due to disasters and when other neighboring networks are not impacted, the only option for the UE to be online is to be served by the neighboring networks.

5.1.2	Pre-conditions

Yuri has a subscription with a network HomeNetwork in country K, and lives in OldCity within country K. In OldCity, the network HomeNetwork owns and operates a local routing center through which all traffic and signaling pass to/from the radio access networks of the HomeNetwork. The equipment of the local routing center is located within the basement of the building called OldBuilding.

In country K, there is another network NeighborNetwork. The NeighborNetwork also owns and operates a local routing enter in OldCity, but it is located in different district than that of OldBuilding.

The network HomeNetwork and the network NeighborNetwork establishes a special agreement so that the two networks provide services to users of each other only during critical disasters where the other network cannot provide services to its home customer.

In OldCity, using mobile payment service is a common practice. Thus, one cannot buy foods or ride a public transportation without a smartphone supporting mobile payment service. So, Yuri also uses mobile payment service to by foods or ride a public transportation with her smartphone.

5.1.3	Service Flows

Following is one example service flow:

1.	One day, when Yuri wakes up in the morning, she feels headache. So, she decides to go to hospital, and pick up her smartphone.

2.	She leaves home and arrives at the bus stop.

3.	It has been raining for several days in OldCity and this causes instability to OldBuilding. Suddenly, the OldBuilding collapses and the subsequent flood and the fire damaged wireline and routing equipment. In the end, connectivity between the radio access network and the core network of the HomeNetwork is lost. But, core network of HomeNetwork can communicate with other neighboring networks. The information related to disaster and the affected network and area can be delivered via authorized regulatory bodies, or via direct between the networks.

4.	In the area where Yuri is located, due to the disaster, there is no available cell of the HomeNetwork and only cells of NeighborNetwork are available. The smartphone of the Yuri camps on a cell of the NeighborNetwork. It starts registration procedure toward the NeighborNetwork.

5.	Based on the information that the disaster has disabled communication service by HomeNetwork in OldCity and the information of the UE (e.g. whether the Yuri is a valid subscriber of the HomeNetwork), the NeighborNetwork decides to provide connectivity service to the UE.

6.	Connectivity for Yuri’s smartphone is now provided by the NeighborNetwork.

7.	Bus arrives at the bus stop and Yuri get on the bus. Yuri touches her smartphone to the payment terminal of the bus. The data for the transaction is delivered by the NeighborNetwork.

5.1.4	Post-conditions

After successfully paying the fee for the bus ride, Yuri successfully arrives at the hospital.

Later, the relevant charging information for the connectivity service provided via the NeighborNetwork is reported to the HomeNetwork.

5.1.5	Existing features partly or fully covering the use case functionality

In TS 22.261, following requirements are specified:

-	The 5G system shall support a mechanism for the operator to authorize subscribers of other PLMNs to receive temporary service (e.g., mission critical services).

-	The 5G system shall be able to provide temporary service for authorized users without access to their home network (e.g., IOPS, mission critical services).

While these requirements in TS 22.261 assumes unavailability of the home network, current use case assumes core network of home network (e.g. at least HSS/UDM of home PLMN) is available and radio access network is not properly operating. Also, the target of this use case is not limited to users of mission critical services.

5.1.6	Potential New Requirements needed to support the use case

[R.5.1-001]	 Subject to regulatory requirements or operator’s policy, 3GPP system shall be able to support a UE of a given PLMN is able to obtain connectivity service (e.g. voice call, mobile data service) from another PLMN of same country when Disaster Condition is met

5.2	A well-handled disaster begins

5.2.1	Description

Within a country deploying the new Minimization of Service Interruption feature, a disaster strikes. Some radio infrastructure is rendered unworkable. Users of the PLMN that partially or fully cannot offer service are able to get service from participating operators offering Disaster Roaming services.

5.2.2	Pre-conditions

In country X, three operators have deployed Minimization of Service Interruption: X-Com, Y-Tel and Z-Cast. Each of these operators has established agreements with the government. The government has the right to declare a ‘Disaster.’ While MNOs often have restrictions to support national roaming, when the Disaster Condition has been established by the government, different roaming behaviours apply.

Fred lives in country X and has a subscription with X-Com. He is registered with the X-Com network and attends a soccer game at a stadium near the capital city. He attends the game with his friend Wang Fang a visitor from China. Her phone is registered (as an inbound roamer) on the Z-Cast network.

5.2.3	Service flows

There is a substantial earthquake. X-Com’s and Z-Cast’s radio equipment near the stadium is damaged and no longer operates. Fred and Wang Fang are without service on X-Com’s network and Z-Cast’s network, respectively.

The government of X declares a Disaster and informs operators X-Com, Y-Tel and Z-Cast that a Disaster Condition applies. (How this occurs is out of scope of this use case.)

Each of X-Com, Y-Tel and Z-Cast begin Disaster Roaming support. This allows UEs to identify the Disaster situation and begin Disaster Roaming support if required.

Fred’s phone determines that a Disaster Condition applies through communication it successfully receives from the Y-Tel network. The phone proceeds to select a PLMN under the ‘Disaster’ condition.

Fred’s phone normally cannot select the PLMN of Y-Tel due to the policy provisioned by the Home Operator X-Com. However, as the Disaster Condition applies, it is possible to select one of these PLMNs.

Fred’s phone selects Y-Tel’s PLMN and registers with the network. He is now a Disaster Inbound Roamer on that network.

NOTE: 	While an operator’s radio network may be unavailable in one location, it may be functional in other locations. Thus, the Disaster Condition criteria alone does not imply that a UE should become a Disaster Inbound Roamer. The UE also needs to have no service on their normally selected network.

Wang Fang’s phone detects the Disaster Condition. She is not prevented from registering with the Y-Tel network due to her HPLMN policy, however, the UE would not normally immediately initiate PLMN selection upon losing coverage. Due to the Disaster Condition, PLMN selection is triggered, and Y-Tel’s PLMN is selected. Wang Fang’s UE registers with the Y-Tel PLMN.

[Editor’s Note: Is Wang Fang’s UE really a Disaster Inbound Roamer, or is it just a normal international inbound roamer? If a normal Inbound Roamer?]

5.2.4	Post-conditions

Both Fred and Wang Fang’s UEs are registered with the Y-Tel network. As Disaster Inbound Roamers, their usage records will result in charging as established in the roaming agreement and national law.

5.2.5	Existing features partly or fully covering the use case functionality

UEs shall be able to receive information (e.g. regarding PLMN information) from any PLMN, even from networks PLMNs that a UE has not selected.

5.2.6	Potential New Requirements needed to support the use case

[R.5.2-001] PLMNs shall be able to inform UEs that a Disaster Condition applies.

5.3	Use case for HPLMN failure with part of CN still working

5.3.1	Description

One PLMN user can roam and access to another PLMN at any time once they have roaming service agreement.  Normally the PLMNs are from different countries without overlap coverage.   But for the PLMNs in same countries there is probably much overlap coverage between the two PLMNs.  Normally they don’t have roaming service agreement, so the user from one PLMN is not allowed to access to the other PLMNs.

Due to government regulation, one PLMN users are allowed to access the other PLMNs as backup PLMNs in the same country when the PLMN is broken down due to some urgent situations, e.g., fire disaster, earthquake, etc. even they don’t have roaming service agreement.

In this use case, it is assumed that the hPLMN is down but part of the Core Network can still work. So the hPLMN can request/notify the other PLMNs on demand to allow its users to access. Once the hPLMN is recovered, its users can return back ASAP.

5.3.2	Pre-conditions

Based on government regulation, there is a special agreement between the two PLMNs that when one PLMN is broken down due to disaster, the other PLMN can serve its UEs.

Necessary parameters for the urgent communication are pre-configured between the two PLMNs and UE.

The PLMN-B (same country) is not forbidden in hPLMN UE.

5.3.3 	Service flow

The hPLMN is broken down due to disaster (e.g., fire).

hPLMN notifies the PLMN-B the failure of hPLMN and requests to allow hPLMN users to access as fast as possible.

UE is trying to select the PLMN-B by performing PLMN selection once losing hPLMN connection.

PLMN-B accepts the UE’s access attempt based on the certain agreement between hPLMN and PLMN-B

UE successfully gets access to PLMN-B.

When the hPLMN is recovered

hPLMN notifies the PLMN-B to let the hHPLMN UEs to return back as fast as possible

Once PLMN-B receives the notification, it starts to release the hPLMN UEs on a proper way.

UE returns to hPLMN by performing PLMN selection.

5.3.4	Post-conditions

UE can normally access to mobile service through PLMN-B as fast as possible when the hPLMN is broken down

UE can return back to hPLMN as fast as possible when the hPLMN is recovered.

5.3.5	Existing features partly or fully covering the use case functionality

5.3.6	Potential New Requirements needed to support the use case

Editor’s Note: the potential requirements will provided in future

5.4	Network reselection during disaster - Non-disaster area

5.4.1	Description

This use case expands the use case in section 5.1 (Network reselection during disaster – General).

In normal situation where the home network is properly functioning, the neighboring network is not required to provide service to users which are not its own subscribers.

In this case, for the area where one network is not providing coverage due to reasons other than disaster, the neighboring network is not required to provide connectivity service for the other network’s users.

Even if the home network is experiencing some problem within some specific area due to the disaster, the neighboring network is not required to provide service to other network’s users if the user is located outside of the specific area.

5.4.2	Pre-conditions

Preconditions described in section 5.1.2 (Network reselection during disaster – General) applies.

In addition, following is further assumed:

-	The Country K is composed of three city, called OldCity, NearCity, FarCity. Due to some reasons, the network HomeNetwork has not yet installed equipment within FarCity.

5.4.3	Service Flows

Following is one example service flow:

1.	While Suji moves from OldCity to NearCity, disaster impacts some radio access network equipment of HomeNetwork installed in OldCity. The information regarding the impacted area and the impacted network is distributed, directly between the networks or indirectly by the authorities. Based on this information, the NeighborNetwork can verify whether a UE of HomeNetwork is located in area where it need to serve.

2.	As the train enters a tunnel in NearCity, the Suji’s smartphone temporarily goes out of coverage of HomeNetwork but is located within coverage of NeighborNetwork. It starts to registration procedure to NeighborNetwork.

3.	The NeighborNetwork checks whether the HomeNetwork in the NearCity is impacted due to disaster, and if so, additionally it checks information on the time period and the location of the impacted area. Because NearCity is not within impacted area, the NeighborNetwork refuses to provide any connectivity service to Suji’s smartphone.

4.	Within a country K, a lot of people move between NearCity and OldCity. Thus, the number of individual access attempt by the subscribers of HomeNetwork is huge and cause a lot of signaling exchange when each UE crosses boundaries of OldCity which is hit by disaster and NearCity which is not. Thus, NeighborNetwork broadcast information regarding when and where UEs of other network can access it.

5.4.4	Post-conditions

Suji’s smartphone cannot register to NeighborNetwork when it is in NearCity and FarCity,

Jame’s smartphone is connected online.

5.4.5	Existing features partly or fully covering the use case functionality

Currently, roaming supports between networks are based on the long-term agreement. Considering the use case described here, short-term dynamic update of whether roaming is allowed or not needs to be supported, based on time, location and cause.

5.4.6	Potential New Requirements needed to support the use case

[R.5.4-001]	 Subject to regulatory requirements or operator’s policy, 3GPP system shall be able to support a UE of a given PLMN is able to obtain connectivity service (e.g. voice call, mobile data service) from another PLMN only within the area where Disaster Condition is applicable.

[R.5.4-002]	 Subject to regulatory requirements or operator’s policy, 3GPP system shall be able to support for a PLMN to be aware of the area where it needs to support Disaster Inbound Roamers.

[R.5.4-003]	 3GPP system shall be able to provide a resource efficient means for a PLMN to indicate potential Disaster Inbound Roamers whether they can access it or not.

5.5	Network reselection during disaster with previous reject

5.5.1	Description

This use case expands the use case in section 5.1 (Network reselection during disaster – General).

Due to mobility, a UE can go through various environments. In a certain case, this environmental change causes a UE to select other network than its home network. This typically ends up with the UE marking the other network as forbidden PLMN.

Once a network is in the forbidden PLMN lists, the PLMN is not automatically selected by the UE unless a user manually selects the PLMN and instructs the UE to connect to the selected PLMN. This can lead to the UE being out of service, even when other neighboring network is willing to accept the UE during disaster.

In TS24.301, UE behavior for different ATTACH REJECT cause is specified. When a UE receives a Reject Cause such as #3 (Illegal UE) or #6 (Illegal ME), the UE shall consider the USIM invalid until switching off or changing of USIM and is not handled by using list of forbidden PLMNs.

5.5.2	Pre-conditions

Preconditions described in section 5.1.2 (Network reselection during disaster – General) applies.

Haru is a subscriber of HomeNetwork and lives in a high-rise apartment. There is a parking lots in the basement of the apartment.

5.5.3	Service Flows

Following is one example service flow:

1.	Haru leaves home and takes elevators to go to a parking lot.

2.	In the elevator, the radio signal from HomeNetwork is not available. The smartphone of the Haru starts searching for any available network but failed.

3.	The elevator arrives at the basement floor. Still, the smartphone cannot detect any signal of HomeNetwork, but it detects the signal of NeighborNetwork. The smartphone tries registration toward the NeighborNetwork. But, because there is no declared disaster in the area, the NeighborNetwork rejects the registration request. The NeighborNetwork is included in list of the forbidden PLMN by Haru’s UE.

4.	Haru starts driving and leaves the basement. Now, the smartphone detects HomeNetwork and is connected online again.

5.	Later on, disaster occurs in the OldCity, impacting the radio access network of HomeNetwork. Cells of HomeNetwork is not available any more. Thus, the UE now camps on the cell of NeighborNetwork, in limited mode. Later, in the OldCity, NeighborNetwork starts to serve the subscribers of the HomeNetwork. E.g., NeighborNetwork indicates that it can accept UEs from HomeNetwork.

6.	Previous restriction due to forbidden PLMN is lifted in the Haru’s UE. Now, the smartphone of the Haru accesses to NeighborNetwork for normal service.

5.5.4	Post-conditions

Haru’s smartphone is connected online via NeighborNetwork.

5.5.5	Existing features partly or fully covering the use case functionality

TS 22.011 is specifying UE behavior regarding network selection. If a PLMN is selected but the UE cannot register on it because registration is rejected with the cause "PLMN not allowed", the UE shall add the PLMN to the “Forbidden PLMN” list (clause 3.2.2.4.1). The UE shall not re-attempt to register on that network unless the same PLMN is selected again by the user. Thus, behavior defined in the current specification prevents the UE from accessing other PLMNs willing to provide connectivity service during disaster.

5.5.6	Potential New Requirements needed to support the use case

[R.5.5-001]	 The 3GPP system shall be able to provide means to enable a UE to try to access a neighbouring PLMN included in the list of forbidden PLMN, in case the UE cannot get connectivity service from its home PLMN due to disaster.

[R.5.5-002]	 The 3GPP system shall be able to provide means for a PLMN to indicate that it can service UEs of specific PLMN, overriding previous restriction such as forbidden PLMN.

5.6	Network protection for surge of incoming users during disaster

5.6.1	Description

When one network is impacted by disasters, almost all UEs of that network may try to access other available neighboring networks.

In case of international roaming, the number of incoming UEs of foreign country to one network is very small compared to the number of the network’s own users. However, in case of roaming caused when one network is out of order due to disaster, the number of incoming UEs from neighboring network is in the same order of the number of one network’s own UEs. If the load of accepting network is already high, additional UEs from neighboring network can cause domino effect, i.e., the accepting network may also soon goes out of order.

Thus, it should be taken into consideration how to effectively protect one network in case other neighboring network is impacted by disaster.

5.6.2	Pre-conditions

In country A, there are two networks, one is HNetwork and the other is NNetwork. Both networks provide connectivity service in TownBig and have same number of subscribers, which is 1 million. Considering several factors such as spectrum, characteristics of subscribers, the location of deployed equipment are quite different among the operators. So, it is unlikely that both networks cannot provide connectivity service at the same time.

The network HNetwork and the network NNetwork establishes a special agreement so that the two networks provide services to users of each other only during critical disasters where the other network cannot provide services to its home customer.

Also, it is assumed that only radio access network of one PLMN is interrupted while core network of the PLMN is not interrupted. So, in the service flow in section 5.x.3, the NNetwork can contact HNetwork to check whether the UE is valid or not.

Tony, with subscription to FNetwork of country B, travels to country A. Initially, Tony’s UE registers itself to HNetwork.

5.6.3	Service Flows

Following is one example service flow:

1.	Disaster such as fire hits the radio access networks of HNetwork in TownBig. All radio access network of HNetwork in TownBig are damaged and cannot function properly. As a result, all UEs with subscription to HNetwork located in TownBig start scan of other available networks and select NNetwork to get connectivity service.

2.	NNetwork is notified of service interruption in HNetwork and decides to provide service to users of HNetwork.

3.	UEs of HNetwork start accesses to NNetwork to register and get connectivity service.

4.	As load of NNetwork goes up, NNetwork decides to apply access control so that 50% of access request from UEs from NNetwork is reduced while imposing no access restriction to its own home UEs. At the same time, NNetwork does not limit access from Tony’s UE, because this UE is not from HNetwork and from foreign country.

5.	As load of NNetwork further goes up, NNetwork decides to prevent all access request from UEs from HNetwork except some prioritized services such as emergency calls. Still, NNetwork does not apply access limitation to its own users.

5.6.4	Post-conditions

NNetwork can control congestion by preventing access from UEs of neighboring network, while providing subscribed QoS to its home UEs.

5.6.5	Existing features partly or fully covering the use case functionality

TS 22.261 and TS 22.011 specifies requirements related to access control, e.g. unified access control, access class barring, etc. However, the specified access control mechanism does not provides tools for operator to apply different access barring among its home UEs and inbound roaming UEs.

In addition, the specified access control mechanism does not provides tools for operator to apply different access barring among inbound roaming UEs from different PLMNs. i.e, regardless of the PLMN to which a UE is subscribed, different incoming UEs use same access barring parameters.

In addition, TS22.011 specifies Extended Access Barring mechanism. EAB supports different barring based on whether a UE camps on (E)HPLMN or on VPLMN. But, this does not support barring for a UE of specific PLMN as described in this use case. Also, it is applicable only for UE which is delay tolerant.

5.6.6	Potential New Requirements needed to support the use case

[R.5.6-001]	 The 3GPP system shall be able to provide mechanism to minimize congestion caused by Disaster Inbound Roamer.

5.7	Use case on access control for “backup” PLMN

5.7.1	Description

Once hPLMN is broken down, more and more hPLMN users are trying to access the other “backup” PLMNs in short time, it may cause signalling storm or heavily impact the PLMN’s native users’ experience of the PLMNs.  In this case “backup” PLMNs enable to control the access from the hPLMN users.

5.7.2	Pre-conditions

Necessary parameters for the urgent communication are pre-configured between the two PLMNs and UE.

The “backup” PLMN-B (same country) is not forbidden in hPLMN UEs

5.7.3 	Service flow

hPLMN is broken down, and requests PLMN-B to activate “backup” mode to allow hPLMN users to access PLMN-B.

The UE of the hPLMN users accesses to PLMN-B by performing PLMN selection and indicate its special identity

More and more hPLMN UEs are trying to access to the “backup” PLMN-B in short time, which is getting overload or running out of resources

The “backup” PLMN-B makes access control or load balance on these hPLMN users so that the native PLMN-B’s UEs are not heavily impacted, e.g., barring the hPLMN UEs access attempts, or degrade the performance for the on-line hPLMN UE.

5.7.4	Post-conditions

Native UEs of the PLMN-B are not heavily impacted

The hPLMN UEs access to other allowed “backup” PLMNs or with limited performance

5.7.5	Existing features partly or fully covering the use case functionality

Access control and load balance, UE’s performance treatment are already specified in existing 3GPP specifications for the inbound roamers (e.g., TS 22.011, TS 22.261)

5.7.6	Potential New Requirements needed to support the use case

5.8	Return to Home Network after disaster period

5.8.1	Description

During disaster, UEs of impacted network are served by other neighboring network. When the impacted network recovers and goes into normal mode of operation, UEs that are temporarily served by other neighboring network need to forward back to its home network.

In this case, abrupt release of connection by neighboring network should be avoided. Otherwise, this may lead to bad service experience. For example, ongoing emergency call should not be released. In addition, releasing all UEs simultaneously will cause signaling surge and this may again cause network congestion and break-down. Also, the time gap between connection release by one network and connection establishment toward other network should be minimized, to enhance user experience.

Because these UEs are temporarily serviced by other networks, there is no need to support service continuity when the UEs returns to their home network.

5.8.2	Pre-conditions

Operation of HNetwork is interrupted by disaster. Users of HNetwork are served by NNetwork until HNetwork recovers from the impact of disaster.

5.8.3	Service Flows

Following is one example service flow:

1.	HNetwork finalizes fixing its network equipment and is ready to accept its own UEs.

2.	HNetwork informs NNetwork that it is ready to accept its UEs. HNetwork informs the rate with which NNetwork should releases UE whose home network is HNetwork.

3.	NNetwork releases connection of UEs whose home network is HNetwork. NNetwork controls the rate at which a number of UEs are released, so that surge of signaling at HNetwork is prevented. When deciding which UE to release first, NNetwork may consider the characteristic of ongoing communication. E.g., if service continuity cannot be provided, UE with emergency call should not be immediately released or redirected. Or, there is no need to provide service continuity for a UE, the NNetwork may release the UE at any time.

4.	To make the switch from NNetwork to HNetwork faster, NNetwork may give some assistance information to the UEs. Or, NNetwork may directly order the UEs to go to HNetwork.

5.	Based on actual amount of accesses by its home UE, HNetwork may inform updated parameter to NNetwork.

6.	In above flows, not only the connected mode UEs but also UEs in Idle mode are forwarded to HNetwork. E.g., the HNetwork may page the UE to move the UE to HNetwork or send SIBs to inform the UE that the HNetwork is recovered from disaster.

5.8.4	Post-conditions

All UEs temporarily served by NNetwork are gradually served by HNetwork.

5.8.5	Existing features partly or fully covering the use case functionality

TS 24.501 and TS 24.401 defines NAS procedure to support

Also, TS 36.331 and TS 38.331 describes RAN level information.

5.8.6	Potential New Requirements needed to support the use case

5.9	A well-handled disaster ends

5.9.1	Description

A Disaster Condition will eventually stop. The government of country X determines that the danger has passed, sufficient services have been restored, and the government decides to lift the Disaster Condition.  This initiates a return to normal service.

5.9.2	Pre-conditions

See use case in section 5.2, above.

The government of X declares and informs operators X-Com, Y-Tel and Z-Cast that Disaster Condition no longer applies. (How this occurs is out of scope of this use case.)

5.9.3	Service flows

Y-Tel informs all Disaster Inbound Roamers that the Disaster Condition no longer applies.

Fred’s and Wang Fang’s UEs detect the end of the Disaster Condition.

Fred’s and Wang Fang’s UEs deregister from Y-Tel and perform PLMN selection.

[Editor’s Note: Is Wang Fang’s UE really a Disaster Inbound Roamer, or is it just a normal international inbound roamer? If a normal Inbound Roamer, should it really deregister from Y-Tel?]

Fred’s UE registers with X-Com. Wang Fang’s UE registers with Z-Cast.

5.9.4	Post-conditions

Both Fred and Wang Fang receive service from their respective providers.

5.9.5	Existing features partly or fully covering the use case functionality

Normal PLMN procedures and requirements apply, except for the trigger to end Disaster Inbound Roaming.

5.9.6	Potential New Requirements needed to support the use case

[R.5.9-001] PLMNs shall be able to inform Disaster Inbound Roamers that a Disaster Condition has ended.

5.10	Use case on “Backup” PLMNs for the international roamer

5.10.1	Description

Here in this case, when the vPLMN-A is broken down due to a disaster, the subscriber of hPLMN1 tries to access to vPLMN-2 based on the “Backup” Agreement between hPLMN1 and vPLMN-2. In this use case, the hPLMN1 user can’t access vPLMN-2, e.g. there is no normal international roaming agreement or vPLMN-2 is very low in the UE’s preferred PLMN list, or the UE has been previously rejected from registering from vPLMN-2.

NOTE:	The “Backup” Agreement is a normal roaming agreement which may only be used when a Disaster Condition applies. It is expected that vPLMN-2 has roaming agreements with all of the PLMNs that vPLMN-A has roaming agreements at least as Backup Agreements.

5.10.2	Pre-conditions

hPLMN1 only has international roaming agreement with vPLMN-A

UE is attached to VPLMN-A

vPLMN-A has “backup” agreement with vPLMN-2

5.10.3 	Service flow

.

A UE roams to vPLMN-A, and accesses to services successfully, according to an international roaming agreement.

vPLMN-A cannot offer radio services when a disaster occurs.

All the users camped on vPLMN-A have no service due to the disaster.

The UE previously registered with vPLMN-A attempts to access vPLMN-2.

Normally the UE would not select vPLMN2 (for whatever reason.)

However, since the Disaster Condition applies, the Backup Agreement will be used.

vPLMN-2 authenticates the international roamers with hPLMN1 using the Backup Agreement.

vPLMN-2 accepts the UE as a Disaster Inbound Roamer.

5.10.4	Post-conditions

The international roamer accesses services during the Disaster Condition applies from vPLMN-2 successfully

5.10.5	Existing features partly or fully covering the use case functionality

5.10.6	Potential New Requirements needed to support the use case

[Editor’s NOTE: the potential requirements are provided in future.]

6	Potential New Consolidated Requirements

[Editor’s note: Add sub-clauses as needed. Text to be provided.]

7	Conclusions and Recommendations

[Editor’s note: Text to be provided.]

Annex <A>:
<Annex title>

Annexes are only to be used where appropriate:

Annexes are labelled A, B, C, etc. and are "informative" (3GPP TRs are informative documents by nature).

A.1	Heading levels in an annex

Heading levels within an annex are used as in the main document, but for Heading level selection, the "A.", "B.", etc. are ignored. e.g. A.1.2 is formatted using Heading 2 style.

Bibliography

The Bibliography is optional. If it exists, it shall follow the last annex in the document.

The following material, though not specifically referenced in the body of the present document (or not publicly available), gives supporting information.

Bibliography format

-	<Publication>: "<Title>".

OR

<Publication>: "<Title>".

Annex <X>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-05 | SA1#86 | S1-191066 |  |  |  | TR Skeleton | 0.0.0 |
| 2019-05 | SA1#86 | S1-191426, S1-191422, S1-191428, S1-191582, S1-191583, S1-191427, S1-191584, S1-191424, S1-191453, S1-191585 |  |  |  | Implement agreed documents in SA1#86 | 0.1.0 |
| 2019-08 | SA1#87 | S1-192492 |  |  |  | Alignment update | 0.1.1 |