3GPP TSG-SA WG Meeting #87	S1-192497 (was 025)

Sophia Antipolis, France, Aug. 19 - 23, 2019

Title:		Update on Section 5.2 and 5.9

Agenda Item:	8.8 FS_MINT

Source:	LG Electronics, LG Uplus, KT Corp, SAMSUNG, SK Telecom

Contact:	sungduck.chun@lge.com

Abstract: This contribution discusses on the aspect of international roaming users in case of disaster.

Discussion & Proposal

Section 5.2, 5.9 and 5.10 include aspects related to international roamers. Unlike national roaming,

It is proposed to agree on the text proposal below.

<<<<<<<<<<<< First change >>>>>>>>>>>>>>>>>

## 5.2	A well-handled disaster begins

### 5.2.1	Description

Within a country deploying the new Minimization of Service Interruption feature, a disaster strikes. Some radio infrastructure is rendered unworkable. Users of the PLMN that partially or fully cannot offer service are able to get service from participating operators offering Disaster Roaming services.

### 5.2.2	Pre-conditions

In country X, three operators have deployed Minimization of Service Interruption: X-Com, Y-Tel and Z-Cast. Each of these operators has established agreements with the government. The government has the right to declare a ‘Disaster.’ While MNOs often have restrictions to support national roaming, when the Disaster Condition has been established by the government, different roaming behaviours apply.

Fred lives in country X and has a subscription with X-Com. He is registered with the X-Com network and attends a soccer game at a stadium near the capital city. He attends the game with his friend Wang Fang a visitor from China. Her phone is registered (as an inbound roamer) on the Z-Cast network. Home network of Wang Fang’ phone has service agreement with both Y-Tel and Z-Cast. According to operator controlled PLMN selector lists, Z-Cast is most prioritized PLMN for Wang Fang’s phone.

### 5.2.3	Service flows

There is a substantial earthquake. X-Com’s and Z-Cast’s radio equipment near the stadium is damaged and no longer operates. Fred and Wang Fang are without service on X-Com’s network and Z-Cast’s network, respectively.

The government of X declares a Disaster and informs operators X-Com, Y-Tel and Z-Cast that a Disaster Condition applies. (How this occurs is out of scope of this use case.)

Each of X-Com, Y-Tel and Z-Cast begin Disaster Roaming support. This allows UEs to identify the Disaster situation and begin Disaster Roaming support if required.

Fred’s phone determines that a Disaster Condition applies through communication it successfully receives from the Y-Tel network. The phone proceeds to select a PLMN under the ‘Disaster’ condition.

Fred’s phone normally cannot select the PLMN of Y-Tel due to the policy provisioned by the Home Operator X-Com. However, as the Disaster Condition applies, it is possible to select one of these PLMNs.

Fred’s phone selects Y-Tel’s PLMN and registers with the network. He is now a Disaster Inbound Roamer on that network.

NOTE: 	While an operator’s radio network may be unavailable in one location, it may be functional in other locations. Thus, the Disaster Condition criteria alone does not imply that a UE should become a Disaster Inbound Roamer. The UE also needs to have no service on their normally selected network.

Wang Fang’s phone detects the coverage loss from Z-Cast. She is not prevented from registering with the Y-Tel network due to service agreement between Y-Tel and her HPLMN. PLMN selection is triggered, and Y-Tel’s PLMN is selected. Wang Fang’s UE registers with the Y-Tel PLMN.

### 5.2.4	Post-conditions

Both Fred and Wang Fang’s UEs are registered with the Y-Tel network. As Disaster Inbound Roamers, Fred’s usage records will result in charging as established in the roaming agreement and national law. On the other hand, Fang’s usage records will result in charging as per the normal roaming service agreement between Y-Tel and Fang’s home network.

### 5.2.5	Existing features partly or fully covering the use case functionality

UEs shall be able to receive information (e.g. regarding PLMN information) from any PLMN, even from networks PLMNs that a UE has not selected.

### 5.2.6	Potential New Requirements needed to support the use case

[R.5.2-001] 3GPP system shall be able to inform UEs impacted by Disaster that a Disaster Condition applies.

[R.5.2-002] 3GPP system shall be able to collect charging information for a Disaster Inbound UE with information that Disaster Condition applies for this UE.

<<<<<<<<<<<< Second change >>>>>>>>>>>>>>>>>

## 5.9	A well-handled disaster ends

### 5.9.1	Description

A Disaster Condition will eventually stop. The government of country X determines that the danger has passed, sufficient services have been restored, and the government decides to lift the Disaster Condition.  This initiates a return to normal service.

### 5.9.2	Pre-conditions

See use case in section 5.2, above.

The government of X declares and informs operators X-Com, Y-Tel and Z-Cast that Disaster Condition no longer applies. (How this occurs is out of scope of this use case.)

### 5.9.3	Service flows

Y-Tel informs all Disaster Inbound Roamers that the Disaster Condition no longer applies.

Fred’s and Wang Fang’s UEs detect the end of the Disaster Condition.

Fred’s UE deregister from Y-Tel and perform PLMN selection.

Fred’s UE registers with X-Com. Wang Fang’s UE remains with Y-Tel.

### 5.9.4	Post-conditions

Both Fred and Wang Fang receive service from their respective providers.

### 5.9.5	Existing features partly or fully covering the use case functionality

Normal PLMN procedures and requirements apply, except for the trigger to end Disaster Inbound Roaming.

### 5.9.6	Potential New Requirements needed to support the use case

[R.5.9-001] PLMNs shall be able to inform Disaster Inbound Roamers that a Disaster Condition has ended.

[R.5.9-002] The Disaster Inbound Roamers shall perform network reselection when a Disaster Condition has ended.