3GPP TSG-SA WG1 Meeting #87 	S1-192509

Sophia-Antipolis, FR, 19 - 23 Aug. 2019	(revision of S1-192480)

Title:	Main body of VIAPA technical specification

Agenda Item:	8.2

Source:	EBU, BBC, Sennheiser, b<>com, Orange

Contact:	ian.wagdin@bbc.co.uk

Abstract: This document includes the main body of the VIAPA technical specification, which contains:

- The scope of VIAPA is included with this contribution. The scope reflects the requirements from the AVPROD and CMed studies [1,2]

- List of references

- Definitions, and symbols related to audio/video production

- This overview has a general section, a section including professional audio/video productions,

  - medical imaging text will be included by CMed contribution.

[1]	3GPP TR 22.827: “Study on Audio-Visual Service Production”

[2]	3GPP TR 22.826: “Study on communication services for critical medical applications”

---------- Use Case template ----------

# 1	Scope

This document describes the service and performance requirements for the operation of professional video, audio and imaging via a 5G system, including a UE, NG-RAN and 5G Core network.

The aspects addressed in this document include:

-	Network service requirements specific for the operation of professional video, imaging and audio for PLMN and non-public networks (NPN)

- 	New key performance indicators (KPIs) for PLMN and NPN

-	KPIs for Multicast and Broadcast Services

-	Network Exposure Requirements

-	Clock synchronisation

-	Application Specific Requirements for video, imaging and audio

-	Mobile and airborne base stations for NPNs

-	Service continuity

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Internet Engineering Task Force (IETF), IETF RFC 4175

[3]	3GPP TS 22.104: Service requirements for cyber-physical control applications in vertical domains

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

AV Production: the process by which audio and video content are combined in order to produce media content. This could be for live events, media production, conferences or other professional applications.

Clock Synchronisation Service: The service to align otherwise independent UE clocks.

Clock Synchronicity: The maximum allowed time offset within a synchronisation domain between the master clock and any individual UE clockThe maximum allowed time offset within the fully synchronised system between UE clocks.

NOTE: This definition was taken from clause 3.1 in 3GPP TS 22.104 [3]

Compressed Video: a means of making video file or stream sizes smaller to meet various applications. Different applications have different compressions methodologies applied.

- Mezzanine compression: low latency and non-complex compression applied to a video signal in order to maintain the maximum amount of information whilst reducing the stream size to allow for the available bandwidth.

- Visually lossless compression: the maximum amount of compression that can be applied to a video signal before visible compression artefact appear.

- Highly compressed: use of compression to distribute content over very low bandwidth connections where the content is more important than the quality of the image.

Communication service availability: percentage value of the amount of time the end-to-end communication service is delivered according to an agreed QoS, divided by the amount of time the system is expected to deliver the end-to-end service according to the specification in a specific area.

NOTE 4: 	The end point in "end-to-end" is assumed to be the communication service interface.

NOTE 5: 	The communication service is considered unavailable if it does not meet the pertinent QoS requirements. If availability is one of these requirements, the following rule applies: the system is considered unavailable in case an expected message is not received within a specified time, which, at minimum, is the sum of maximum allowed end-to-end latency and survival time.

NOTE 6: 	This definition was taken from clause 3.1 in 3GPP TS 22.261 [4].

Communication service reliability: ability of the communication service to perform as required for a given time interval, under given conditions.

NOTE 7: 	Given conditions would include aspects that affect reliability, such as: mode of operation, stress levels, and environmental conditions.

NOTE 8: 	Reliability may be quantified using appropriate measures such as meantime to failure, or the probability of no failure within a specified period of time.

NOTE 9: 	This definition is based on 3GPP TS 22.104 [3].

Cue / Talkback: Audio messages sent from broadcast centre to location usually to instruct a presenter when to speak. This is not audible in the broadcast audio.

End-to-end Latency: the time that [it] takes to transfer a given piece of information from a source to a destination, measured at the communication interface, from the moment it is transmitted by the source to the moment it is successfully received at the destination.

NOTE: This definition was taken from clause 3.1 in 3GPP TS 22.261 [4]

Isochronous: The time characteristic of an event or signal that is recurring at known, periodic time intervals.

NOTE 1:	Isochronous data transmission is a form of synchronous data transmission where similar (logically or in size) data frames are sent linked to a periodic clock pulse.

NOTE 2:	Isochronous data transmission ensures that data between the source and the sink of the AV application flows continuously and at a steady rate.

Imaging System Latency: The time that takes to generate an image from a source, to apply a certain amount of processing, to transfer it to a destination and then to render the resulting image on a suitable display device, as measured from the moment a specific event happens to the moment that very same event is displayed on a screen.

In-Ear-Monitoring (IEM): A specialist type of earphone usually worn by a performer in which an audio signal is fed to a wireless receive device and attached earphone.

Media Clock: Media clocks are used to control the flow (timing and period) of audio / video data acquisition, processing and playback. Typically, media clocks are generated locally in every mobile or stationary device with a master clock generated by an externally sourced grand master clock (currently GPS but transitioning to 5G in future).

Mouth-to-ear Latency: End-to-end maximum latency between the analogue input at the audio source (e.g. wireless microphone) and the analog output at the audio sink (e.g. IEM). It includes audio application, application interfacing and the time delay introduced by the wireless transmission path.

Multi-Cam: The use of two or more cameras in an outside broadcast which can be cut between, important considerations are colour imagery, timing, framing and picture size and frequency.

Network Operator: The entity which offers 3GPP communication services.

Non-public network: a network that is intended for non-public use.

NOTE 10: This definition was taken from clause 3.1 in 3GPP TS 22.261 [4].

Outside Broadcast: A production where content is being acquired away from the broadcast centre and controlled from the location. Generates output for broadcast which may be sent back to the broadcast centre for inclusion into a programme or for onward distribution.

Radio Microphone: A microphone that uses a wireless connection to transmit either an analogue or digital audio channel on a dedicated radio frequency or multiplex to one or more dedicated receivers which then output an audio signal suitable for onward processing.

Survival time: the time that an application consuming a communication service may continue without an anticipated message.

NOTE 3: This definition was taken from clause 3.1 in 3GPP TS 22.261 [4].

Uncompressed Video: Uncompressed video is digital video that either has never been compressed or was generated by decompressing previously compressed digital video. RTP payload is described in [2].

Video, imaging and audio: The means of digital capture, transmission and storage of still and moving pictures and sound for professional use.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Tframe	Time interval between consecutive audio frames at application layer. Also used to denote the transfer interval in this document.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AV	Audio-Visual

IEM	In-Ear-Monitoring

# 4	Overview on Video, Imaging and Audio Professional Applications

## 4.1 Introduction

The present document introduces requirements related to professional video, imaging and audio services. Unlike other consumer multimedia applications envisioned for 3GPP systems, the applications in which this document focuses have more demanding performance targets and includes user devices that are managed in different workflows when compared to typical UEs.

This document focuses on services for the production of audio-visual data for any area that requires high quality images or sound. This may include AV production, medical or gaming applications.

To enable devices such as professional cameras, medical imaging equipment and microphones to use the 5G network either directly or via the addition of a dedicated intermediate technology certain key parameters are required.

This document focuses on services for the production of audio-visual data for any area that requires high quality images or sound. This may include AV production, medical or gaming applications.

## 4.2 Key parameters

### 4.2.1 System Latency

The overall system latency has an important impact on the applications  that this specification targets. In video production, overall system latency is referred to as imaging system latency and has an impact on the timing of synchronized cameras. For audio applications, overall system latency is referred to as mouth to ear latency and it is critical to maintain lip sync and avoid a performer to be put off by hearing their own echo. Finally, in medical applications the system latency impairs the achievable precision at a given gesture speed as it translates the time needed to traverse the whole imaging system into a geometrical error of the instruments position

.

Figure 4.2.1.-1 depicts the general functional blocks of an AV production or medical system.

Figure 4.2.1-1: Overall system Latency for video, imaging and audio applications.

The overall system latency comprises different latency elements as illustrated in Figure 4.2.1-1, where:

T1 = Time for image or audio frame generation

T2 = T4 = Time Delay through 5G Network, defined as the end-to-end latency in TS 22.261 [4]

T3 = Application processing time

T5 = Time for image display or audio playback

So that the overall system latency results from the sum of the of T = T1 + T2 + T3 + T4 + T5

### 4.2.2 Bandwidth

Video and imaging applications have extremely high bandwidth requirements and while compression may be used to mitigate this in certain user cases it often degrades the picture to the extent onward processing required by some applications is compromised.  For Video Production certain standards have been determined which indicate the maximum allowable compression for a given type of production.  In medical imaging, compression may introduce artifacts which can impact on diagnosis of critical illness and may also introduce additional delays which, in image assisted surgery, translate into misalignment between perceived instruments position on screen and their real position into patients’ body.

### 4.2.3 Reliability

Reliability is another key parameter for VIAPA. Late or lost packets can result in dropped audio/video frames or inconsistency of motion which can degrade a video or audio signal to below acceptable levels.

## 4.3 AV production

AV production includes television and radio studios, live news-gathering, sports events, music festivals, among others. Typically, numerous wireless devices such as microphones, in-ear monitoring systems or cameras are used in these scenarios. In the future, the wireless communication service for such devices could potentially be provided by a 5G system. AV production applications require a high degree of confidence, since they are related to the capturing and transmission of data at the beginning of a production chain. This differs drastically when compared to other multimedia services because the communication errors will be propagated to the entire audience that is consuming that content both live and on recorded medias. Furthermore, the transmitted data is often post-processed with filters which could actually amplify defects that would be otherwise not noticed by humans. Therefore, these applications call for uncompressed or slightly compressed data, and very low probability of errors. These devices will also be used alongside existing technologies which have a high level of performance and so any new technologies will need to match or improve upon the existing workflows to drive adoption of the technology.

The performance aspects that are covered in this document also target the latency that these services experience. Since these applications involve physical feedback on performances that are happening live, the latency requirements are very strict. One example if the transmission of professional microphones and in-ear monitors. These systems provide feedback for what the musicians are playing, and even small delays may affect their sensation of timbre, and ability to keep the tempo of the music.

This document also refers to how the network structure of the 5G system is configured in order to accommodate these applications. Many of these are nomadic scenarios that require simplified deployment often in different countries. For this reason, this the 5G system should enable non-public networks that can be deployed in an agile way.

## 4.4 Medical Applications

Medical applications is a generic term covering medical devices and applications involved in the delivery of care to patients.

Medical applications deployed into operating rooms consume communication services delivered by a 5G system over an NPN. In this document, we’ll deal with hybrid operating rooms which are rooms typically equipped with advanced imaging systems such as e.g. fixed C-arms (x-ray generator and intensifiers), CT scanners (Computer Tomography) and MR scanners (Magnetic Resonance). The whole idea is that advanced imaging enables minimally-invasive surgery that is intended to be less traumatic for the patient as it minimizes incisions and allows to perform surgery procedure through one or several small cuts. This is as an example useful for cardio-vascular surgery or neurosurgery to place deep brain stimulation electrodes.

In hybrid rooms, the different type of medical images that can be transmitted by 5G systems and processed by medical applications are e.g.:

- Ultra-high resolution video generated by endoscopes where it is expected that some scopes will produce up to 8K uncompressed (or compressed without quality loss) video, with the perspective to support also HDR (High Dynamic Range) for larger colour gamut management (up to 10 bits per channel) as well as HFR (High Frame Rate), i.e.; up to 120 fps. This will allow surgeons to distinguish small details like thin vessels and avoid any artefact that could potentially conduct surgeons to take wrong decisions.

- 2D Ultrasound images: A 2D ultrasound typically produces a data stream of uncompressed images of 512x512 pixels with 32 bits per pixel at 20 fps (up to 60 fps in the fastest cases), resulting in a data rate of 160 Mbit/s up to 500 Mbit/s.

- 3D Ultrasound volumes: Dedicated 3D probes tend to work at higher data rates, i.e. above 1 Gbit/s of raw data, and are expected to reach multi gigabit data rates in future (e.g. producing 3D Cartesian volumes of 256 x 256 x 256 voxels each encoded with 24 bits at 10 volumes per second or better).

- CT/MR scans: Images can range from a resolution of 1024x2024 to 3000x3000 pixels where higher resolutions are used for diagnosis purpose and lower ones are more suitable to fluoroscopy. In general, the frame rate is variable (5 to 30fps typically) where higher values are used to monitor moving organs in real time. Finally, color depths of 16bits is considered in general.