3GPP TSG-SA WG1 Meeting #88 	S1-193083

Reno, Nevada, USA, 18 - 22 November 2019	(revision of S1-19xxxx)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.153 | CR | 0043 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on MPS2 requirements and terminology |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS2 |  |  |  |  |  |  | Date: |  | 2019-11-18 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Several requirements appear confusing, some text is considered too specific or not needed, and RAT/system terminology can be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added clarifications to requirements, wording and terminology, applicable to same/similar text used in several sections/places. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some requirements, text and terminology are unclear or not accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 5.1, 5.4, 5.4a, 5.8.2, 5.15, 5.16, 9.1.2.1, 9.2.1, 9.2.2.1, 9.3.1, 9.3.2.1, 9.3.2.2, 9.3.3, 9.3.5 (double), 9.3.6. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# <First change>

# 4	General description

MPS is applicable in EPS and 5GS systems.

MPS provides priority treatment to increase the probability of an authorized Service User’s Voice, Video, and Data communication session being successful. Some form of priority treatment is applied to the MPS invocation and session establishment, and continues to be applied until the MPS session is released. The priority treatment may be applied before the invocation if a greater probability of success in receiving, recognizing, and processing the invocation is needed.

MPS allows qualified and authorized users to obtain priority access to the next available radio channel on a priority basis before other PLMN users, during situations when PLMN congestion is blocking session establishment attempts. In addition, MPS supports priority sessions on an "end-to-end" priority basis.

MPS is intended to be utilised for Voice, Video, and Data bearer services, including the Data Transport Service, in the Packet-switched (PS) domain and the IP Multimedia Subsystem (IMS). It also involves priority transitioning of MPS service (e.g., Voice) to the CS domain when the network does not support the requested service in the PS domain. MPS Voice, Video and Data sessions are based on providing priority treatment to the corresponding commercial services offered to the public.

MPS is also intended to be utilized for IoT communications through machine interactions that may not directly involve a human Service User.

MPS includes network functions that fall into the following broad categories:

Service Invocation: The process to recognize and identify a request for an MPS session. AN MPS Service Provider network recognizes an MPS invocation based on the presence of an MPS-unique identifier entered by the originating Service User in the service request received by the network from the UE, or based on the subscription profile of the originating UE, or as a regional/operator option the subscription profile of the terminating UE.

NOTE:	The option related to “subscription profile of the terminating UE” may not involve end-to-end priority because this option is based on providing priority only in the terminating network based on the terminating UE subscription profile.

Authorization: The process to verify that a Service User is authorized for MPS. This includes capabilities to verify authorization to receive priority treatment in the radio access network and to access the MPS application service (MPS Voice, Video, and Data).

End-to-End Priority Treatment: The process of providing priority treatment in all parts of the path, from one endpoint to the other endpoint(s). End-to-end priority treatment includes priority treatment by all MPS capable networks involved in the MPS session path, the origination network and the termination network as well as any transit networks in between.

Invocation-to-Release Priority Treatment: The process of providing priority treatment to all phases of a session, from invocation until release, including all steps in between.

The combination of End-to-End Priority Treatment and Invocation-to-Release Priority Treatment includes both pre- and post-authorization treatment and includes the following aspects:

1)	Priority processing of the Service User’s MPS invocation,

2)	Admission control and allocation of network resources (including bearer resources) in origination, termination, and transit networks, including handovers,

3)	Transport of signaling and media packets,

4)	Priority processing within EPS, 5GS and CN, and

5)	Processing of the Service Users release of an MPS service session.

Network Interconnection and Protocol Interworking: A Service User’s MPS invocation and session establishment will involve transport and processing, and the end-to-end signaling and media path may traverse multiple MPS Service Provider networks. These end-to-end cases include, but are not limited to:

1)	Voice, video, and data services over EPS or 5GS, including signaling for call/session establishment and media;

2)	EPS/5GS interworking with the CS domain, including a) calls originated in the CS domain and terminated in EPS/5GS, and b) calls from EPS/5GS to the CS domain;

3)	CS Fallback from EPS, for one or both ends of call, with maintenance of existing PS domain MPS services, either in EPS or in a legacy system, e.g., the GPRS Core; and

4)	Access to MPS data and video services not under IMS control.

<next change>

# 5	High level requirements

## 5.1	General

The system shall provide preferential treatment based on the subscription for MPS for:

-	An authorized Service User using a UE with an MPS subscription,

NOTE 1:	In this case of an UE with an MPS subscription, subscription related information stored in the UE is used, e.g., membership in a special access class as per TS 22.011 [6] is used for E-UTRAN access to the EPC and membership in Access Identity 1 as per TS 22.261 [7] is used for 5GS (NR and E-UTRA access to the 5GC).

-	An authorized Service User using a UE that does not have an MPS subscription, and

NOTE 2:	In this case, the Service User is not identified by the subscription related information stored in the UE (e.g., the Service User borrows a UE that does not have an MPS subscription).  In this case, radio interface priority may not be provided for the initial request for invocation of MPS.  It is provided only after MPS is established for the MPS Voice, Video or Data session.

-	An authorized IoT device with an MPS subscription.

NOTE 3:	In this case of an IoT device with an MPS subscription, subscription related information stored in the IoT device is used to allow invocation/revocation of MPS through machine interactions that may not directly involve a human MPS Service User.

Upon invocation of MPS, the system shall provide preferential treatment for access and core network resources associated with the session (i.e., signalling and media bearer related resources within a domain and across domains), including, when applicable, priority treatment towards an external (non-3GPP) network supporting MPS. A Service User is assigned a priority level by a regional/national authority i.e., agency authorised to issue priority levels. Upon MPS invocation the calling Service User's priority level is used to identify the priority to be used for the session being established.

Pre-emption of active sessions shall be subject to regional/national regulatory requirements.

Subject to regional/national regulatory policy, a PLMN should have the capability to retain public access as a fundamental function. Therefore, MPS traffic volumes should be limited (e.g. not to exceed a regional/national specified percentage of any concentrated network resource, such as eNodeB capacity), so as not to compromise this function.

<next change>

## 5.4	Priority session treatment in terminating network

When a terminating network receives an incoming MPS session establishment attempt, the MPS session shall receive priority treatment (priority access to signalling and media bearer resources for voice, video, and data) in the terminating PLMN, based on the originating Service User priority information. When requested, this includes MPS treatment towards an external (non-3GPP) network network supporting MPS.

As an operator option, the terminating network may invoke priority treatment for an incoming session from a non-MPS subscriber to an MPS subscriber (see clause 4).

When the terminating network supporting session establishment cannot assign the necessary resources to the MPS session, the MPS session request shall be:

-	Queued,

-	Processed for the next available resource in accordance with the Service User's priority level and session arrival time.

The network shall support the capability to inform the calling Service User about the status of the MPS session (e.g., tones or signalling messages can be used to indicate that the session request has been queued).

If the queued MPS session times out, then normal session processing applies.

## 5.4a	Priority Data Bearer Service

The Priority Data Bearer Service provides  MPS priority for data and video services not under IMS control.

When a Service User invokes Priority Data Bearer Service for transport of any data packets to and from that Service User, the network should give priority in admission/upgrade of the Priority Data Bearer(s) and in packet data scheduling in the event of congestion (for new sessions and upgrade to existing sessions), subject to regional/national regulatory policy. Specifically:

-	A Priority Data Bearer service session shall be given priority for admission/upgrade over non-Priority Data Bearer sessions during times of congestion;

-	Data packets belonging to a Priority Data Bearer service shall not be dropped before data packets belonging to a non-Priority Data Bearer service session, when the network is experiencing congestion, subject to the limitation imposed by public access. Priority Data Bearer session QoS, as required for the type of service invoked (e.g., packet delay), should be maintained throughout the activity of the data session.

MPS for Data Transport Service (DTS) is a generic priority packet transport service that applies independently of the specific data application being used, focused on providing priority only to those applications using the default bearer (EPS), or default QoS flow (5GS). It is a specific example of Priority Data Bearer Service.  MPS for DTS service specific requirements are described in clause 9.3.

<next change>

### 5.8.2	Roaming outside the home country

The system shall support MPS when a UE with an MPS subscription is roaming outside of the home country and the visited and home network support MPS, and roaming agreements are in place for MPS.

NOTE 1:	Roaming outside a home country is subject to national/regional rules and operator policy.

NOTE 2:	As specified in TS 22.261 [7] clause 6.22.2.2, for 5GS (E-UTRA and NR access to the 5GC)

the home PLMN can configure the UE to discover and select a network supporting MPS when roaming outside of the home country (e.g., the UE is configured to select a specific MPS capable visited PLMN when roaming in a country that may have multiple PLMNs where some are MPS capable and some are not MPS capable).

<next change>

## 5.15	Quality of Service (QoS)

In certain cases, the QoS characteristics (e.g., packet delay and packet loss) for an MPS session may be the same as the underlying communication service (e.g., voice, video or data) used to support the MPS session, with the MPS session being provided priority treatment subject to regional/national regulatory and operator policies. In other cases, there may be need to request specific QoS characteristics for the MPS session (e.g., specific data communications as described in clause 5.4a, 9.3).

The 3GPP network shall allow selection and/or configuration of QoS rule(s) appropriate for MPS Voice, Video and Data.

The 3GPP network shall allow enforcement of QoS rule(s) for MPS Voice, Video and Data.

## 5.16 Inter-network aspects

The system shall allow MPS priorities to be signalled across interconnected networks, including non-3GPP networks.

NOTE 1: 	Support of MPS services across networks is based on policy and agreements between the network operators.

NOTE 2: 	Intermediate 3GPP networks not supporting MPS (e.g., transit networks between an originating network and terminating network) should allow MPS priorities to be signalled transparently, based on operator policy and agreements for MPS.

The system shall support means to identify that an incoming voice, video, or data session from another network is an MPS session and to handle it with priority.

The system shall support measures to verify policy and admit an incoming MPS Voice, Video or Data session received from another network with priority.

# <next change>

### 9.1.2	Invocation and revocation

#### 9.1.2.1	Invocation

A Service User can invoke MPS to request priority for an MMTEL voice call (or conference call), e.g. when the normal MMTEL call cannot be established.

The system shall support means for a Service User using a UE with an MPS subscription to initiate MPS for an MMTEL voice call or an MPS MMTEL voice conference call.

NOTE 1:	MPS invocation using a UE with an MPS subscription is done using a predetermined method (e.g., by the Service User inclusion of an MPS-unique identifier or feature code as part of the request to establish an MPS for MMTEL voice call).

The system shall support means for a Service User using a UE with an MPS subscription to initiate MPS for an MMTEL voice call, or an MPS MMTEL voice conference call when roaming within the home country and outside the home country.

NOTE 2:	MPS invocation using a UE with an MPS subscription is done using a predetermined method (e.g., by the Service User inclusion of an MPS-unique identifier or feature code as part of the request to establish an MPS for MMTEL voice call) that is recognized by the visited PLMN.

The system shall support means for a Service User using a UE that does not have a subscription for MPS to initiate MPS for an MMTEL voice call or MMTEL voice conference call.

NOTE 3:	MPS invocation using a UE that does not have an MPS subscription is done using a predetermined method that is outside the scope of 3GPP (e.g., by the Service User use of a predetermine access number, such as a special telephone number).

The system shall support means for a Service User to:

-	initiate MPS for all participants of an MMTEL voice conference call, or

-	request upgrade of all participants of an ongoing MMTEL voice conference call to MPS.

The system shall provide MPS for a late participant joining an MPS for MMTEL voice conference after MPS was activated for all participants.

NOTE 4:	When MPS is invoked for all participants of a MMTEL voice conference, the user priority level is based on the host MPS Service User, except in cases where an individual participant used MPS to join the conference with a higher user priority level, the higher user priority level is kept.

NOTE 5:	The invocation of MPS for all participants may be done using a customized feature of a voice conference service (available only to the host) that indicates a request for priority to the system.

# <next change>

## 9.2	MPS for MMTEL video

### 9.2.1	General

The primary purpose of MPS for MMTEL video is to provide the Service User with priority video communication sessions in periods of severe network congestion during which normal commercial video service is degraded.

The system shall support:

-	MPS for MMTEL video and video conference calls for an authorized Service User using a UE with a subscription for MPS,

NOTE 1:	In the case of an UE with an MPS subscription, subscription related information stored in the UE is used, e.g., membership in a special access class as per TS 22.011 [6] is used for EUTRAN access to the EPC and membership in Access Identity 1 as per TS 22.261 [7] is used for 5G (NR and E-UTRAN access to the 5GC).

-	MPS for MMTEL video and video conference calls for an authorized Service User using a UE that does not have an MPS subscription, and

NOTE 2:	In this case, the Service User is not identified by the subscription information stored in the UE (e.g., the Service User borrows a UE that does not have an MPS subscription).  In this case, radio interface priority is not provided for the initial request for invocation of MPS.  It is provided only after MPS is established for the MPS for MMTEL video session.

-	MPS for all participants of an authorized MMTEL video conference call.

NOTE 3:	MPS for all participants of an MMTEL video conference call allows a Service User (i.e., the host) to establish an MMTEL video conference with MPS for all participants, or upgrade on ongoing MMTEL video conference to MPS for all participants regardless of their subscription to MPS.

### 9.2.2	Invocation and revocation

#### 9.2.2.1	Invocation

A Service User can invoke MPS to request priority for an MMTEL video call (or video conference), e.g. when the normal MMTEL video call cannot be established.

The system shall support means for a Service User using a UE with an MPS subscription to:

-	initiate MPS for an MMTEL video call or an MPS MMTEL video conference call, or

-	request upgrade of an established MMTEL video call, or an established MMTEL video conference call to MPS.

NOTE 1:	MPS invocation using a UE with an MPS subscription is done using a predetermined method (e.g., by the Service User inclusion of an MPS-unique identifier or feature code as part of the request to establish an MPS for MMTEL video call).

NOTE 2:	In the case where the MMTEL video call is already established but becomes unacceptable, the invocation by the Service User to upgrade the MMTEL video call to MPS may be done using a using a specialized MPS application on the UE (i.e., as an alternative to releasing and retrying the call which may fail).

The system shall support means for a Service User using a UE with an MPS subscription to:

-	initiate MPS for an MMTEL video call, or an MPS MMTEL video conference call when roaming within the home country and outside the home country, or

-	requests upgrade of an established MMTEL video call, or an established MMTEL video conference call to MPS when roaming within the home country and outside the home country.

NOTE 3:	MPS invocation using a UE with an MPS subscription is done using a predetermined method (e.g., by the Service User inclusion of an MPS-unique identifier or feature code as part of the request to establish an MPS for MMTEL video call) that is recognized by the visited PLMN.

NOTE 4:	In the case where the MMTel video call is already established but becomes unacceptable, the invocation by the Service User to upgrade the MMTel video call to MPS may be done using a using a specialized MPS application on the UE (i.e., as an alternative to releasing and retrying the call which may fail) that is recognized by the visited PLMN.

The system shall support means for a Service User using a UE that does not have a subscription for MPS to initiate MPS for an MMTEL video call or MMTEL video conference call.

NOTE 5:	MPS invocation using a UE that does not have an MPS subscription is done using a predetermined method (e.g., by the Service User use of a predetermine access number, such as a special telephone number) and is outside the scope of 3GPP.

The system shall support means for a Service User to:

-	initiate MPS for all participants of an MMTEL video conference call, or

-	request upgrade of all participants of an ongoing MMTEL video conference call to MPS.

The system shall provide MPS for a late participant joining an MPS for MMTEL video conference after MPS was activated for all participants.

NOTE 6:	When MPS is invoked for all participants of a MMTEL video conference, the user priority level is based on the host MPS Service User, except in cases where an individual participant used MPS to join the conference with a higher user priority level, the higher user priority level is kept.

NOTE 7:	The invocation of MPS for all participants may be done using a customized feature of a video conference service (available only to the host) that indicates a request for priority to the system.

# <next change>

## 9.3	MPS for DTS

### 9.3.1	General

MPS for Data Transport Service (DTS) is a generic priority packet transport service that applies independently of the specific data application being used.  It is a specific example of Priority Data Bearer Service described in clause 5.4a. MPS for DTS provides the Service User with priority for applications (e.g., content from a web site) using the default bearer (in the case of EPS), default QoS flow (in the case of 5GS) in periods of severe network congestion during which normal commercial data service is degraded.

The system shall support:

-	MPS for DTS for an authorized Service User using a UE with a subscription for MPS, and

NOTE 2:	In this case of an UE with an MPS subscription, subscription related information stored in the UE is used, e.g., membership in a special access class as per TS 22.011 [6] is used for EUTRAN access to the EPC and membership in Access Identity 1 as per TS 22.261 [7] is used for 5G (NR and E-UTRAN access to the 5GC).

-	MPS for DTS for an authorized Service User using a UE that does not have an MPS subscription.

NOTE 3:	In this case, the Service User is not identified by the subscription related information stored in the UE (e.g., the Service User borrows a UE that does not have an MPS subscription).  In this case, radio interface priority is not provided for the initial request for invocation of MPS.  It is provided only after MPS is established for the DTS session.

MPS for DTS may be used to by a Service User to obtain priority access (e.g., Virtual Private Network (VPN) access) to an external non-3GPP network (e.g., a private corporate network or private Government Agency network to download data and/or access enterprise applications such as email, text/chat/presence, voice and video).

The system shall support for an authorised Service User, an end-to-end MPS for DTS connection between the MPS subscribed UE and an associated external network supporting MPS on demand based on operator policy and when agreements are in place for MPS.

NOTE 4:	The end-to-end MPS for DTS connection between the UE and an associated external network is supported either using a web browser (e.g., use of a predetermined address (i.e., URL) in a web browser) or using a user-selectable option provided as part of the VPN client in the UE with an MPS subscription.

MPS for DTS provides an IoT device with subscription to MPS with priority for applications using a specific default bearer (in the case of EPS), default QoS flow (in the case of 5GS), and in the case of IoT connectivity with an external network, also provides priority for the terminating access network (connected to the external network).

The system shall support:

-	MPS for DTS for an authorized IoT device with a subscription for MPS, and

-	an end-to-end MPS for DTS connection between the MPS subscribed IoT device and an associated external network supporting MPS on demand based on operator policy when agreements are in place for MPS.

NOTE 5:	An MPS specialized application on the MPS subscribed IoT device is used to support MPS for DTS through machine interactions that may not directly involve a human Service User.

An authorized external network supporting MPS uses MPS for DTS to activate priority for remote MPS subscribed UEs and MPS subscribed IoT devices associated with the external network based on operator policy when agreements are in place for MPS.

The system shall support:

-	for an authorized external network supporting MPS, activation of MPS for DTS for remote MPS subscribed UEs associated with the external network on demand based on operator policy when agreements are in place for MPS, and

-	for an authorized external network supporting MPS, activation of MPS for DTS for remote MPS subscribed IoT devices associated with the external network on demand based on operator policy when agreements are in place for MPS.

NOTE 6:	MPS for DTS activation by an authorized external network supporting MPS involves prearrangement and configuration (i.e., subscription arrangement identifying the external network authorization for remote activation of DTS in IoT devices) within the MPS Service Provider PLMN.

### 9.3.2	Invocation and revocation

#### 9.3.2.1	Invocation

A Service User can invoke MPS for DTS to request priority, e.g. when the normal data session cannot be established.

The system shall support means for a Service User using a UE with an MPS subscription to initiate MPS for a DTS session.

NOTE 1:	MPS invocation using a UE with an MPS subscription is done using a predetermined method (e.g., by the Service User using a web browser to enter a predetermined address such as an URL).

The system shall support means for a Service User using a UE with an MPS subscription to initiate MPS for DTS session when roaming within the home country and outside the home country.

NOTE 2:	MPS for DTS invocation using a UE with an MPS subscription is done using a predetermined method (e.g., by the Service User using a web browser to enter a predetermined address such as an URL) recognized by the visited PLMN.

The system shall support means for a Service User using a UE that does not have a subscription for MPS to initiate MPS for DTS.

NOTE 3:	MPS for DTS invocation using a UE that does not have an MPS subscription is done using a predetermined method (e.g., by the Service User use of a predetermine URL address) and is outside the scope of 3GPP.

The system shall support means for a Service User using a UE with an MPS subscription to initiate an MPS for DTS session to an associated external network supporting MPS on demand based on operator policy and when agreements are in place for MPS.

NOTE 4:	Invocation of MPS for DTS is done either using a web browser (e.g., use of a predetermined address (i.e., URL) in a web browser) or using a user-selectable option provided as part of the VPN client in the UE with an MPS subscription.

The system shall support means for an IoT device with an MPS subscription to initiate an MPS for DTS session to an external network supporting MPS.

NOTE 5:	An MPS specialized application on the MPS subscribed IoT device is responsible for determining whether or not to invoke MPS for DTS in support of the data communication needs.  The MPS specialized application allows invocation/revocation of the DTS through machine interactions that may not directly involve a human Service User.

An authorized external network supporting MPS activates MPS for DTS to request priority for MPS subscribed UEs and MPS subscribed IoT devices associated with the external network when normal data service is unacceptable.

The 3GPP system shall support means for an authorized external network supporting MPS to request activation of MPS for DTS for the following cases:

-	a specific MPS subscribed UE or IoT device, and

-	a group of MPS subscribed UEs or IoT devices.

NOTE 6:	MPS for DTS invocation done by the external network supporting MPS involves prearrangement and configuration (i.e., subscription arrangement identifying the external network authorization for remote activation of DTS in IoT devices) within the MPS Service Provider PLMN.  For example, an MPS specialized application hosted in the external network (e.g., IoT application server) may be responsible for determining whether or not to invoke MPS for DTS.  The MPS specialized application allows invocation/revocation of the DTS through machine interactions that may not directly involve a human Service User.

The system shall support receiving and authenticating MPS for DTS invocation requests from authorized MPS Service Users via public non-3GPP networks.

NOTE 7:	The intent of the above requirement is to allow an authorized MPS Service User to send a request via the Internet to establish an MPS for DTS session for one or more active PDNs/DNs of a UE.

The system shall support discovery and selection of active data connections upon receipt of an MPS for DTS invocation request by an authorized MPS Service User.

#### 9.3.2.2	Revocation

The system shall support means to end an MPS for DTS session for the cases:

-	an MPS Service User using a UE with an MPS subscription,

-	an MPS Service User using a UE that does not have an MPS subscription, and

-	an IoT device with an MPS subscription.

The system shall support means for an authorized external network supporting MPS to explicitly request to release an MPS for DTS session or group of MPS for DTS sessions while maintaining basic connectivity to the MPS subscribed UE(s) or IoT device(s).

The system shall support means for an MPS Service User to explicitly release an MPS for DTS session from a UE with an MPS subscription when roaming, including outside the home country.

If MPS for DTS is not explicitly revoked by the Service User or an IoT device, the system in the RPLMN shall automatically revoke MPS:

-	upon UE detachment/deregistration (e.g., power down), or

-	when all the connections pertaining to the MPS for DTS session are released (without explicit release of the MPS for DTS session).

The system shall automatically revoke MPS when the maximum allowed duration for an MPS for DTS session is reached.

### 9.3.3	Authentication and authorization

The system shall support means to authenticate and authorize an MPS for DTS session request from a UE with an MPS subscription.

The system shall support means to authenticate and authorize a Service User request for an MPS for DTS session from a UE that does not have an MPS subscription.

NOTE 1:	The method for authenticating and authorizing the Service User using a UE that does not have an MPS subscription is predetermined.  As part of the authentication and authorization process, the Service User provides MPS credentials (e.g., PIN or security token) specifically assigned for the purpose of obtaining MPS from a UE that does not have an MPS subscription.

The system shall support means for a visited PLMN, including a visited PLMN outside of the home country, to verify with the home PLMN that a UE is authorized for MPS, and to authorize an MPS for DTS session.

NOTE 2:	The method for authenticating and authorizing UE for MPS when roaming is based on operator policy and roaming agreements between the visited PLMN and home PLMN for MPS as per clause 5.8.

The system shall support means to authenticate and authorize an MPS for DTS session request from an IoT device with an MPS subscription.

The system shall support means to authenticate and authorise an external network supporting MPS requesting activation of MPS for DTS for the following cases:

-	a specific MPS subscribed UE or IoT device, and

-	a group of MPS subscribed UEs or IoT devices.

# <next change>

### 9.3.5	Media

The system shall provide priority treatment to the affected media flows when an MPS for DTS session is activated.

The system shall provide priority treatment to the affected media flows when MPS for DTS is activated for the connection between an MPS subscribed UE or IoT device and an associated external network supporting MPS.

NOTE:	Both the MPS subscribed UE or IoT device and the external network must be served by the same PLMN.

### 9.3.5	Security

The system shall support security capabilities to verify an external network's authorization to request MPS for DTS activation.

### 9.3.6	QoS

The system shall be capable of supporting a set of default QoS characteristics for an active MPS for DTS session.

The system shall be capable of verifying authorization for a requested QoS modification to an active MPS for DTS session.

The system shall be capable of making authorized QoS modifications in support of MPS for DTS for all media traffic supported via the default bearer (in the case of EPS), or the default QoS flow (in the case of 5GS).

The system shall support means to allow a Service User to request QoS modifications for an active MPS for DTS session.

The system shall support means to provide an indication that a requested QoS modification of an active MPS for DTS session was successful.